| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E266 |
| --- | --- |

| Title: | CE6.a.4: Chroma intra prediction by reconstructed luma samples |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Vadim Seregin Woo-Jin Han Jungsun Kim Byeongmoon Jeon | Email: | jiale.chen@samsung.com v.seregin@samsung.com wjhan.han@samsung.com Jungsun.kim@lge.com bm.jeon@lge.com |
| Source: | [ Samsung Electronics Co., Ltd; LG Electronics ] |  |  |

_____________________________

# Abstract

This document reports the results of “Chroma intra prediction by reconstructed luma samples” method proposed in document JCTVC-D350 [1] within the context of CE6 [2]. In the proposed method, chroma samples are predicted from luma samples of same block by linear model relationship. Compared to the HM2.0, the average BD-rate gain is 0.8%, 7.8% and 5.9% for intra configuration, the average BD-rate gain is 0.4%, 8.6% and 5.6% for random access configuration, respectively for Y, Cb and Cr components. The BD-rate gain is 0.7%, 6.2%, 5.1% for all intra configurations when applying proposed method to 8x8 and 4x4 chroma block.

# Introduction

In current Test Model (HM) of High Efficiency Video Coding (HEVC) standard, 5 modes are used in chroma intra prediction: Vertical, Horizontal, DC, Diagonal with 45 degree (Vertical-8) and mode derived (named as DM mode in this document) from luma intra direction. When DM mode is selected, luma UDI mode is used for intra chroma prediction. Table 1 shows the mode numbers and corresponding codewords in HM2.0.

Table 1  Mode numbers and Codewords for intra chroma prediction modes

| Intra chroma prediction mode | Mode number | Codewords |
| --- | --- | --- |
| DM | 4 | 0 |
| Ver | 0 | 10 |
| Hor | 1 | 110 |
| DC | 2 | 1111 |
| Ver-8 | 3 | 1110 |

This contribution reports the results when the new chroma intra prediction method was used instead of Vertical-8 mode. The proposed method is originally proposed in JCTVC-B021[3] and optimized version is proposed in JCTVC-C206[4]. In the new method, chroma samples are predicted from reconstructed luma samples of same block by linear model relationship (named as LM mode in this document).

# LM prediction

When LM mode is used, the chroma values are predicted from reconstructed luma values of same block as follows.

(1),

where indicates the prediction of chroma samples in a block and indicates the reconstructed luma samples in the block. Parameters  and are derived from causal reconstructed samples around the current block.

The sampling ratio of chroma components is half of that of luma component and has 0.5 pixel phase difference in vertical direction in YUV420 sampling. Reconstructed luma is downsampled in vertical direction and subsample in horizontal direction to match size and phase of chroma singal, as follows.

(2),

The proposed method utilizes linear least square solution between causal reconstructed data of downsampled luma component and causal chroma component to derive model parameters  and .

(3),

(4),

whereand indicate reconstructed chroma samples and downsampled luma samples around the target block, indicates total samples number of neighboring data. As shown in fig.1, only left and above causal samples marked as gray circles are involved in the calculation to keep total samples number as power of 2. For a target N×N chroma block, when both left and above causal samples are available, total involved samples number is ; when only left or above causal samples are available, total involved samples number is .

Fig. 1 Locations of the samples used for the derivation of α and β

In the proposed method, the look-up table and multiplication is used to replace division when calculating parameter. All operation is implemented with integer implementation within 32 bits data accuracy. The detailed implementation of LM prediction method can found in JCTVC-C206.

# Experiment results

The proposed method is integrated in HM2.0 software and compared with it as anchor. The experiments were performed with the common test configuration described in JCTVC-D600 [5].

LM mode is used to replace Vertical-8 mode. The mode code-words were assigned according to mode statistics as in Table 2.

Table 2 New Mode Numbers and Codewords for intra chorma prediction mode

| Intra chroma prediction mode | Mode number | Codewords |
| --- | --- | --- |
| DM | 4 | 0 |
| LM | 0 | 10 |
| Ver | 1 | 110 |
| Hor | 2 | 1111 |
| DC | 3 | 1110 |

Table 3 shows average BD-rate saving of proposed method comparing to the anchor, HM2.0 software for all testing configurations. The average BD-rate saving of Y, Cb, Cr components are 0.8%, 7.8% and 5.9% in intra test configuration, and 0.4%, 8.6% and 5.6% in random access configuration with almost same encoding and decoding time to the anchor. The proposed method achieves maximum gain in case of intra high efficiency for NebutaFestival sequence with 3.4% in Y, 35.3% and 15.0% in Cb and Cr respectively. Tab.3 shows BD-rate saving for each test sequence class in intra test configurations. The result also shows that consistent bit saving is obtained by proposed method for sequences with different resolution.

Table 3 BD-rate saving of proposed LM mode

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -16.2 | -8.3 | -1.1 | -14.6 | -9.4 |
| Class B | -0.8 | -7.2 | -3.7 | -0.8 | -7.3 | -4.2 |
| Class C | -1.1 | -6.7 | -7.4 | -1.2 | -6.4 | -7.1 |
| Class D | -0.7 | -4.7 | -4.6 | -0.8 | -4.8 | -4.6 |
| Class E | -0.1 | -3.3 | -3.7 | -0.2 | -4.8 | -5.2 |
| All | -0.8 | -7.8 | -5.6 | -0.8 | -7.7 | -6.1 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -17.2 | -9.3 | -0.2 | -15.5 | -8.3 |
| Class B | -0.4 | -8.1 | -3.6 | -0.4 | -8.0 | -4.0 |
| Class C | -0.5 | -6.1 | -6.3 | -0.6 | -6.1 | -6.3 |
| Class D | -0.3 | -4.2 | -3.7 | -0.4 | -4.3 | -4.1 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | -8.8 | -5.6 | -0.4 | -8.4 | -5.6 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -2.8 | -1.7 | -0.1 | -3.5 | -2.0 |
| Class C | -0.2 | -2.7 | -2.7 | -0.3 | -2.8 | -3.0 |
| Class D | -0.3 | -1.5 | -1.3 | -0.1 | -2.3 | -1.6 |
| Class E | -0.1 | -2.3 | -2.9 | -0.1 | -4.3 | -5.5 |
| All | -0.2 | -2.3 | -2.1 | -0.2 | -3.2 | -2.8 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Following table shows the results when LM mode is only applied to 8x8 and 4x4 chroma block. The results show that most gain is achieved by small block.

Table 4 BD-rate saving of proposed LM mode

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -8.6 | -5.1 | -1.1 | -13.6 | -8.8 |
| Class B | -0.5 | -4.7 | -2.7 | -0.6 | -6.0 | -3.7 |
| Class C | -1.0 | -6.4 | -7.1 | -1.2 | -6.2 | -7.0 |
| Class D | -0.6 | -4.5 | -4.4 | -0.8 | -4.7 | -4.6 |
| Class E | -0.1 | -1.9 | -2.8 | -0.2 | -3.8 | -4.5 |
| All | -0.6 | -5.4 | -4.4 | -0.8 | -7.0 | -5.7 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Subjective Comparison

Figure 1 shows the subjective improvements of the proposed methods for Cr components. Each figure is the first frame of NebutaFestival QP37 of the anchor and the proposed respectively.

Figure 1. (a) A part of first Cr image of the anchor

Figure 2. (b) A part of first Cr image when the proposed method is applied

# Complexity analysis

In this section, complexity of three chroma intra prediction methods (LM mode, UDI mode, Plane mode) is analyzed by comparing the operation number.

Tab.4, Tab.5 and Tab.6 shows the operation number for each pixel of proposed LM mode, UDI mode and plane mode. UDI mode is in HEVC and plane mode is used in AVC as chroma intra prediction method.

Tab.4 operation number of proposed LM mode

| Prediction step | Proposed LM mode |  |  |
| --- | --- | --- | --- |
|  | Mul | Add | Shift |
| Calculation of and for one block | ~=3·N +6 | ~=6·N +4 | ~=4 |
| Down-sample for each pixel | 0 | ~=0.5 | ~=0.5 |
| Prediction for each pixel | 1 | 1 | 1 |
| Total operation number for each pixel prediction | 1+3/ N +6/ N2 | 1.5+6/ N +6/ N2 | 1.5+4/ N2 |

Tab.5 operation number of general UDI chroma mode

| Prediction step | Plane mode |  |  |
| --- | --- | --- | --- |
|  | Mul | Add | Shift |
| Initialision the Main and Left reference array for one block | ~ | ~ | ~ |
| Position calculation for each pixel | 0 | 1+2/N | 1/N |
| Interpolation for each pixel | 2 | 3 | 1 |
| Total operation number for each pixel prediction | 2 | 4+2/N | 1+1/N |

Tab.6 operation number of plane mode based on JM17.1

| Prediction step | Plane mode |  |  |
| --- | --- | --- | --- |
|  | Mul | Add | Shift |
| Calculation of A, B and C for one block | ~=N +4 | ~=2·N +4 | ~=3 |
| Prediction for each pixel | 1+2/ N | 1+4/ N | 1 |
| Total operation number for each pixel prediction | 1+3/ N +4/ N2 | 1+6/ N +4/ N2 | 1+3/ N2 |

# Conclusion

This contribution reports the results of the new chroma intra prediction method proposed in JCTVC-D350 where chroma samples are predicted from luma samples of same block by linear model relationship. Experimental results show that the proposed method achieves 0.8%, 7.8% and 5.9% BD-rate reduction respectively for Y, Cb, Cr components in intra test with 3.4%, 35.3% and 15.0% maximum gain for NebutaFestival sequences. The BD-rate gain is 0.7%, 6.2%, 5.1% for all intra configurations when applying proposed method to 8x8 and 4x4 chroma block. The encoding/decoding measurement and operation number complexity analysis prove that complexity of proposed LM mode is similar and even smaller to existing method in HEVC or AVC.

The proposed method can be easily extended to other YUV sampling format. For YUV 4:2:2 or YUV 4:4:4 sequences, the overall gains are expected to be more remarkable.

Since proposed method achieves solid coding gain without evident complexity increase, we recommend adopting it in HEVC test model.

# References

- J. Chen, V. Seregin, S. Lee, W. Han, J. Kim, B.M. Jeon, “CE6.a: Chroma intra prediction by reconstructed luma samples”, JCTVC-D350, Daegu, January, 2011.

- A. Tabatabai, “Core Experiment 6: Intra Prediction Improvement”, JCTVC-D606, Daegu, January, 2011

- J. Chen, V. Seregin, “Chroma intra prediction by reconstructed luma samples”, JCTVC-C206, Guangzhou, October, 2010.

- J. Kim, S.W. Park, et al, “New intra chroma prediction using inter-channel correlation”, JCTVC-B021, Geneva, July 2010

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Daegu, January, 2010.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: text for working draft

#### 7.4.7 Coding unit semantics

intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples of the current block. The value shall be in the range from 0 to 4 (0: LM mode, 1: vertical prediction, 2: horizontal prediction, 3: DC prediction, and 4: the same prediction mode with luma samples).

#### 8.3.2 Derivation process for chroma intra prediction mode

Table 8-4 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ]

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 34 ) |
| 0 | 34 | 34 | 34 | 34 | 34 |
| 1 | n/a | 0 | 0 | 0 | 0 |
| 2 | 1 | n/a | 1 | 1 | 1 |
| 3 | 2 | 2 | n/a | 2 | 2 |
| 4 | 0 | 1 | 2 | 3 | X |

#### 8.3.3.1 Intra chroma sample predictions

Table 8-5 – Specification of intraPredMode and associated names

| intraPredMode | Name of intraPredMode |
| --- | --- |
| 0 | Intra_Vertical |
| 1 | Intra_Horizontal |
| 2 | Intra_DC |
| Otherwise (3..33) | Intra_Angular |
| 34 | Intra_LM |

8.3.3.1.6 Specification of Intra_LM prediction mode

Inputs to this process are:

–	neighbouring chroma samples pC[ x, y ], with x, y = -1..nS-1,

–	neighbouring luma samples pY[ x, y ], with x, y = -2..2*nS-1,

–	luma samples of current block rY[ x, y ], with x, y = 0..2*nS-1,

–	a variable nSC specifying the prediction block size.

Output of this process is:

–	predicted chroma samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is 34.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived with following ordered step.

1.	Derivation process for linear model parameters a and b

a = clip3( -215, 215-1, a1s*lmDiv[ abs( a2s ) ]+ (1<<(k1-1))>> k1)

b = ( L - ( ( a * C ) >> k1 ) + 1<<(k2-1)) >> k2

where,

a1s = a1 >> max(0, log2(abs(a1))-14)

a2s = a2 >> max(0, log2(abs(a2))-5)

a1 = (LC<<k2) –L*C

a2 = (LL<< k2) –L*L

k1 = max(0, log2(abs(a2))-5) - max(0, log2(abs(a1))-14) + 2

With

k3 = max(0, BitDepthC+log2(nS)-14 )

pY'[x, y]=(pY[2x,2y]+pY[2x,2y+1])>>1

value of L, C, LL, LC, k2 is derived with following steps.

–	If all samples pC[ x, -1 ], with x=0..nS-1 and pC[ -1, y ], with y=0..nS-1 are marked as “available for intra prediction,”, L, C, LL and LC is derived as,

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3
C = y=0nS−1pC[−1, y]+x=0nS−1pC[x, −1]>>k3
LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3
LC = y=0nS−1pY'[−1, y]∗pC[−1, y]+y=0nS−1pY'[x, −1]∗pC[x,−1]>>k3

k2 = log2(nS>>k3)

–	Otherwise, If all samples pC[ x, -1 ], with x=0..nS-1 are marked as “not available for intra prediction” and all samples pC[ -1, y ], with y=0..nS-1 are marked as “available for intra prediction,” , L, C, LL and LC is derived as,

L = y=0nS−1pY'[−1, y]>>k3
C = y=0nS−1pC[−1, y]>>k3
LL = y=0nS−1pY'[−1, y]2>>k3
LC = y=0nS−1pY'[−1, y]∗pC[−1, y]>>k3

k2 = log2(nS>>k3)

–	Otherwise, If all samples pC[ x, -1 ], with x=0..nS-1 are marked as “available for intra prediction” and all samples pC[ -1, y ], with y=0..nS-1 are marked as “not available for intra prediction,” , L, C, LL and LC is derived as,

L = x=0nS−1pY'[x, −1]>>k3
C = x=0nS−1pC[x, −1]>>k3
LL = x=0nS−1pY'[x, −1]2>>k3
LC = y=0nS−1pY'[x, −1]∗pC[x,−1]>>k3

k2 = log2(2*nS>>k3)

2.	The value of the prediction samples predSamples[ x, y ] is derived by

predSamples[ x, y ] = Clip1C(((pY’[ x, y ]*a)>>13)+b), with x, y = 0..nS-1

9.1.2 Mapping process for chroma intra prediction mode

Inputs to this process are:

- bits for intra_chroma_pred_mode[ xB ][ yB ]

- intraPredMode specifying the corresponding luma intra prediction mode of the current block.

Output of this process is the value of intra_chroma_pred_mode[ xB ][ yB ]

Table 9-4 shows the coding tables of intra_chroma_pred_mode[ xB ][ yB ] depending on the value of intraPredMode.

According to different intra luma prediction modes, different coding tables shall be used in this process.

Table 9-4 Binarization for intra_chroma_pred_mode[ xB ][ yB ]

| - intra_chroma_pred_mode | - intra_luma_pred_mode |  |  |  |
| --- | --- | --- | --- | --- |
|  | - 0 | - 1 | - 2 | - Others (3..33) |
| - 0 (LM) | - 10 | - 10 | - 10 | - 10 |
| - 1(vertical) | - n/a | - 111 | - 111 | - 110 |
| - 2 (horizontal) | - 111 | - n/a | - 110 | - 1111 |
| - 3 (DC) | - 110 | - 110 | - n/a | - 1110 |
| - 4 (same as luma) | - 0 | - 0 | - 0 | - 0 |