| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E270 |
| --- | --- |

| Title: | CE12.1: Cross-verification of the deblocking filter from Ericsson (JCTVC-E276) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen | Email: | v.seregin@samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This document reports cross-check results of the deblocking filter of JCTVC-E276 from Ericsson.

# Test Results

Related software based on HM2.0 was provided by proponent. It was verified that implementation of the algorithm is consistent with the description of the original proposal [1]. Software was compiled on 64-bit Linux and executed on Linux cluster. Simulation tests were conducted under common test condition [2] and decoder was run on single PC with 32-bit Windows 7 to measure the decoding time. The following tables represent test results, more details can be found in the accompanying Excel sheets.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | 0.0 | 0.0 | -1.5 | 0.0 | 0.0 |
| Class B | -0.7 | 0.0 | 0.0 | -0.8 | 0.0 | 0.0 |
| Class C | -0.6 | 0.0 | 0.0 | -0.8 | 0.0 | 0.0 |
| Class D | -0.5 | 0.0 | 0.0 | -0.6 | 0.0 | 0.0 |
| Class E | -0.9 | 0.0 | 0.0 | -1.0 | 0.0 | 0.0 |
| All | -0.7 | 0.0 | 0.0 | -0.9 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | 0.0 | 0.0 | -1.4 | 0.8 | 1.1 |
| Class B | -0.9 | -0.2 | -0.4 | -1.0 | -0.1 | -0.1 |
| Class C | -0.8 | -0.2 | -0.2 | -0.8 | 0.3 | 0.0 |
| Class D | -0.6 | -0.3 | -0.3 | -0.6 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -0.2 | -0.2 | -0.9 | 0.2 | 0.2 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.4 | -1.3 | -1.4 | -1.3 | -0.9 | -0.4 |
| Class C | -1.0 | -0.5 | -0.6 | -0.8 | 0.3 | -0.2 |
| Class D | -0.9 | -0.7 | -0.8 | -0.6 | 0.2 | 0.2 |
| Class E | -3.3 | -3.8 | -3.2 | -2.6 | -4.7 | -3.3 |
| All | -1.5 | -1.4 | -1.4 | -1.2 | -1.0 | -0.7 |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

These simulation results are matched with BD-rate and decoder running time provided by proponent.

Additionally, as recommended in CE12 description [3], informal subjective test was performed for Kimono Qp 32 low-complexity low-delay, Kimono Qp 37 high-efficiency random-access, Vidyo3 Qp 32  low-complexity low-delay, Vidyo3 Qp 37 high-efficiency low-delay using LCD monitor.

For this purpose decoder was run with command –d 8 to make output 8-bit yuv, and it was observed with provided deblocking filter that there is no evident visual quality difference for motion pictures, but if comparing frame by frame it is possible to find regions with better and worse visual quality comparing to the anchor.

# Conclusion

In this document results of the JCTVC-E276 were confirmed and no evident visual quality difference was observed on sample test cases.

# References

[1] Andrey Norkin et al., “Development of HEVC deblocking filter” JCTVC-D377, Geneva, Mar. 2011

[2] Frank Bossen “Common test conditions and software reference configurations” JCTVC-D600, Daegu, Korea, Jan. 2011

[3] Andrey Norkin et al., “Description of CE12: Deblocking filtering” JCTVC-D612, Daegu, Korea, Jan. 2011