| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E272 |
| --- | --- |

| Title: | CE4: Cross-verification on slice-boundary filtering (JCTVC-E044) - Subset2 case1 and case2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this document, 2 cross-verification results are given in the context of CE4. The BD-rates are matched with those reported in JCTVC-E044. These methods were proposed in JCTVC-D128 at the previous Daegu meeting. The average loss of subset2-case 1 is 4.3% with 9% and 15% encoding and decoding time increase. The average loss of subset2-case 2 is 4.1% with 8% encoding and 18% decoding time increase.

# Introduction

In this document, 2 cross-verification results are given in the context of CE4. These methods were proposed in JCTVC-D128 [1] at the previous Daegu meeting. Summarized test conditions are given in Table 1. The software provided by MediaTek was investigated and run independently.

Table 1 Test conditions for case 1 and 2 of subset 2 of CE4

| Case | Software | Condition |
| --- | --- | --- |
| 1 | MediaTek based on HM2.0 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice-independent DF and ALF, slice boundary filter off |
| 2 | MediaTek based on HM2.0 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice-independent DF and ALF, slice boundary filter on |

# Results

The BD-rates from independent test are matched with those reported in JCTVC-E044 [2]. In summary, the average loss of subset2-case 1 is 4.3% with 9% and 15% encoding and decoding time increase. The average loss of subset2-case 2 is 4.1% with 8% encoding and 18% decoding time increase. According to these results, the average gain from slice-boundary filtering is about 0.2%. Detailed results are given in Table 2 and 3, respectively. Complete data can be found in spreadsheets enclosed with this document.

Table 2 Results of subtest 2 case 1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.9 | 5.2 | 6.0 | 5.2 | 5.9 | 6.1 |
| Class B | 5.6 | 5.7 | 6.4 | 6.0 | 6.7 | 6.9 |
| Class C | 5.7 | 5.9 | 6.3 | 6.0 | 6.8 | 6.9 |
| Class D | 3.9 | 4.5 | 4.8 | 4.1 | 4.4 | 4.8 |
| Class E | 8.6 | 10.7 | 9.1 | 8.3 | 12.7 | 11.6 |
| All | 5.6 | 6.2 | 6.4 | 5.8 | 7.0 | 7.0 |
| Enc Time[%] | 115% |  |  | 124% |  |  |
| Dec Time[%] | 145% |  |  | 108% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 6.5 | 10.7 | 10.0 | 5.7 | 6.5 | 7.3 |
| Class B | 5.2 | 6.9 | 6.4 | 5.2 | 5.8 | 5.3 |
| Class C | 4.4 | 5.6 | 5.8 | 4.4 | 4.7 | 4.7 |
| Class D | 2.7 | 3.3 | 3.6 | 2.6 | 2.5 | 2.8 |
| Class E |  |  |  |  |  |  |
| All | 4.7 | 6.7 | 6.4 | 4.5 | 4.9 | 5.0 |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 118% |  |  | 108% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.8 | 7.4 | 5.8 | 3.9 | 4.0 | 3.6 |
| Class C | 3.0 | 5.2 | 5.2 | 2.9 | 2.8 | 2.4 |
| Class D | 1.7 | 2.9 | 2.6 | 1.9 | 1.8 | 2.2 |
| Class E | 1.7 | 2.0 | 2.1 | 1.3 | 3.5 | 3.8 |
| All | 2.7 | 4.7 | 4.2 | 2.7 | 3.1 | 3.0 |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 106% |  |  | 104% |  |  |

Table 3 Results of subtest 2 case 2

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.8 | 4.0 | 4.8 | 4.9 | 4.8 | 5.1 |
| Class B | 5.2 | 4.3 | 4.8 | 5.5 | 5.4 | 5.4 |
| Class C | 5.5 | 4.7 | 4.8 | 5.8 | 5.6 | 5.5 |
| Class D | 3.8 | 3.5 | 3.6 | 4.0 | 3.4 | 3.6 |
| Class E | 7.8 | 8.3 | 7.0 | 7.6 | 10.4 | 9.6 |
| All | 5.3 | 4.7 | 4.9 | 5.5 | 5.7 | 5.6 |
| Enc Time[%] | 113% |  |  | 120% |  |  |
| Dec Time[%] | 144% |  |  | 110% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 6.4 | 9.8 | 9.3 | 5.4 | 5.8 | 6.7 |
| Class B | 5.0 | 6.2 | 5.5 | 4.8 | 5.1 | 4.7 |
| Class C | 4.3 | 5.0 | 5.2 | 4.2 | 4.2 | 4.1 |
| Class D | 2.6 | 2.8 | 3.0 | 2.5 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 4.6 | 6.0 | 5.7 | 4.3 | 4.4 | 4.5 |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 124% |  |  | 111% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.8 | 7.1 | 5.5 | 3.6 | 3.7 | 3.4 |
| Class C | 3.0 | 5.0 | 5.0 | 2.8 | 2.6 | 2.2 |
| Class D | 1.7 | 2.4 | 2.4 | 1.8 | 2.0 | 2.2 |
| Class E | 1.7 | 2.4 | 1.6 | 1.4 | 3.1 | 2.9 |
| All | 2.6 | 4.5 | 3.9 | 2.5 | 2.9 | 2.7 |
| Enc Time[%] | 105% |  |  | 100% |  |  |
| Dec Time[%] | 116% |  |  | 102% |  |  |

# Conclusions

In this document, 2 cross-verification results are given in the context of CE4. The BD-rates are matched with those reported in JCTVC-E044. The average loss of subset2-case 1 is 4.3% with 9% and 15% encoding and decoding time increase. The average loss of subset2-case 2 is 4.1% with 8% encoding and 18% decoding time increase.

# References

- JCT-VC, “Slice Boundary Processing and Picture Layer RBSP”, JCTVC-D128, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “CE4 Subset2: Slice Boundary Filter”, JCTVC-E044, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011