| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E274 |
| --- | --- |

| Title: | CE10: Cross-verification on Cisco/TI’s transform (JCTVC-E243) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this document, the cross-verification results on transform of Cisco/TI (JCTVC-E243) are given in the context of CE10. In the test, the BD-rates were matched with those reported in JCTVC-E243.

# Introduction

In this document, the cross-verification results on transform of Cisco/TI (JCTVC-E243 [1]) are given in the context of CE10. In JCTVC-D224 [2], new transform was proposed. The transform was based on the matrix multiplication and the low complexity implementation can be possible by partial factorization. For this cross-verification, the source code provided by Cisco/TI was investigated and run independently. The modification of scaling before and after transform was noticed in the provided source code. And additional clip was applied to control the bit-width of inverse transform.

# Results

The verification tests were done under test conditions defined in B.1.2 of JCTVC-D610 [3]. Two separate tests were designed with pre-defined low and high QPs. Summarized results are given in Table 1 and Table 2. The BD-rates were matched with those reported in JCTVC-E243. The average gain at low QPs is 1.4% and at high QPs is 0.1%. Complete data can be found in spreadsheets enclosed with this document.

Table 1 Test results at low QPs

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.4 | -1.5 | -1.5 | -2.4 | -1.5 | -1.5 |
| Class B | -1.5 | -2.0 | -2.1 | -3.4 | -1.8 | -1.7 |
| Class C | -0.3 | -0.5 | -0.5 | -2.0 | -1.0 | -0.9 |
| Class D | -0.4 | -0.5 | -0.5 | -1.9 | -1.2 | -1.1 |
| Class E | -2.2 | -2.4 | -2.4 | -2.4 | -1.7 | -1.9 |
| All | -1.1 | -1.3 | -1.4 | -2.5 | -1.4 | -1.4 |
| Enc Time[%] | 105% |  |  | 111% |  |  |
| Dec Time[%] | 92% |  |  | 89% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -1.0 | -0.8 | -1.6 | -0.9 | -0.9 |
| Class B | -1.1 | -1.1 | -1.3 | -2.0 | -0.8 | -0.8 |
| Class C | -0.6 | -0.3 | -0.4 | -1.4 | -0.7 | -0.9 |
| Class D | -0.5 | 0.0 | -0.1 | -1.4 | -0.8 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -0.8 | -0.6 | -0.7 | -1.6 | -0.8 | -0.8 |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 92% |  |  | 88% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.1 | -0.8 | -1.0 | -2.3 | -0.6 | -0.7 |
| Class C | -0.7 | -0.3 | -0.3 | -1.6 | -0.6 | -0.7 |
| Class D | -0.7 | 0.1 | 0.0 | -1.5 | -0.5 | -0.6 |
| Class E | -1.1 | -0.9 | -0.8 | -1.8 | -1.0 | -1.3 |
| All | -0.9 | -0.5 | -0.5 | -1.9 | -0.7 | -0.8 |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 95% |  |  | 92% |  |  |

Table 2 Test results at high QPs

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.1 | 0.4 | -0.2 | -1.4 | -1.1 |
| Class B | -0.1 | -0.2 | 0.5 | -0.1 | -1.0 | -1.0 |
| Class C | -0.1 | 0.1 | -0.1 | -0.1 | -1.4 | -0.9 |
| Class D | -0.1 | 0.3 | -0.2 | -0.1 | -1.7 | -0.7 |
| Class E | 0.0 | 0.5 | 0.0 | -0.2 | -0.5 | -0.7 |
| All | -0.1 | 0.1 | 0.1 | -0.1 | -1.2 | -0.9 |
| Enc Time[%] | 116% |  |  | 105% |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.3 | 0.2 | -0.1 | -0.1 | -0.3 |
| Class B | 0.0 | -1.3 | 1.3 | -0.1 | -0.7 | -0.9 |
| Class C | -0.4 | 0.5 | 0.5 | 0.0 | -0.6 | -1.1 |
| Class D | 0.0 | 0.7 | -3.0 | -0.2 | -1.8 | -2.3 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.2 | -0.2 | -0.1 | -0.9 | -1.2 |
| Enc Time[%] | 103% |  |  | 96% |  |  |
| Dec Time[%] | 101% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.7 | -1.8 | -0.1 | -2.2 | 0.4 |
| Class C | -0.1 | 1.0 | -1.1 | 0.0 | -2.8 | -3.0 |
| Class D | 0.1 | -1.8 | -0.2 | -0.4 | -2.7 | -1.3 |
| Class E | -0.5 | -2.2 | -1.9 | -0.2 | -1.6 | -0.6 |
| All | -0.2 | -0.8 | -1.2 | -0.2 | -2.4 | -1.1 |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 105% |  |  |

# Conclusions

In this document, the cross-verification results on transform of Cisco/TI were given in the context of CE10. The BD-rates were matched with those reported in JCTVC-E243. For low QPs, the average gain is 1.4%, reportedly. For high QPs, the average gain is 0.1%, reportely.

# References

- JCT-VC, “Transform design for HEVC with 16 bit intermediate data representation”, JCTVC-E243, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011

- JCT-VC, “Unified transform design for HEVC with 16 bit intermediate data representation”, JCTVC-D224, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “Description of Core Experiment 10 (CE10): Core Transform Design”, JCTVC-D610, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011