| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E275 |
| --- | --- |

| Title: | CE10: Cross-verification on Qualcomm’s transform (JCTVC-E370) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this document, the cross-verification results on transform of Qualcomm (JCTVC-E370) are given in the context of CE10. As a verification result, the BD-rates are matched with those reported in JCTVC-E370. The average luma gain is 0.1% with 2% encoding and 4% decoding time increase.

# Introduction

In this document, the cross-verification results on transform of Qualcomm (JCTVC-E370 [1]) are given in the context of CE10. In JCTVC-D256 [2], new transform was proposed. The transform was based on the LLM factorization and small size transforms can be re-used for larger size transform. For this cross-verification, the source code provided by Qualcomm was investigated and run independently. The modification of multiplication factors in butterfly structure was noticed in the provided source code.

# Results

The verification test was done under common test conditions (JCTVC-D600 [3]). Summarized results are given in Table 1. The BD-rates were matched with those reported in JCTVC-E370. The average gain is 0.1% with 2% encoding and 4% decoding time increase. Complete data can be found in spreadsheets enclosed with this document.

Table 1 Test results

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.33 | -0.54 | -0.74 | -0.77 | -0.21 | -0.10 |
| Class B | -0.16 | -0.02 | -0.15 | -0.22 | -0.12 | -0.20 |
| Class C | 0.04 | 0.04 | -0.05 | -0.04 | -0.05 | -0.27 |
| Class D | 0.05 | 0.11 | 0.06 | -0.01 | -0.03 | -0.24 |
| Class E | -0.17 | -0.03 | -0.39 | -0.40 | -0.10 | -0.14 |
| All | -0.11 | -0.09 | -0.24 | -0.28 | -0.10 | -0.19 |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.21 | -0.45 | -0.34 | -0.47 | 0.01 | 0.13 |
| Class B | -0.10 | 0.03 | -0.05 | -0.17 | 0.07 | 0.02 |
| Class C | -0.05 | 0.04 | 0.01 | -0.03 | 0.19 | 0.11 |
| Class D | 0.06 | -0.05 | 0.13 | 0.01 | 0.19 | 0.09 |
| Class E |  |  |  |  |  |  |
| All | -0.08 | -0.10 | -0.06 | -0.17 | 0.11 | 0.09 |
| Enc Time[%] | 105% |  |  | 103% |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.13 | 0.41 | 0.34 | -0.10 | 0.03 | 0.27 |
| Class C | -0.01 | 0.47 | 0.28 | -0.03 | 0.27 | 0.16 |
| Class D | -0.05 | 0.53 | 0.77 | 0.12 | 0.15 | 0.22 |
| Class E | -0.31 | -0.86 | -0.28 | -0.21 | 0.31 | 0.02 |
| All | -0.12 | 0.22 | 0.32 | -0.05 | 0.17 | 0.19 |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 97% |  |  |

# Conclusions

In this document, the cross-verification results on transform of Qualcomm are given in the context of CE10. The BD-rates were matched with those reported in JCTVC-E370. The average gain is 0.1% with 2% encoding and 4% decoding time increase.

# References

- JCT-VC, “CE10: Scaled orthogonal integer transforms supporting recursive factorization structure”, JCTVC-E370, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011

- JCT-VC, “Efficient 16 and 32-point transforms”, JCTVC-D256, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011