| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E276_r1 WG11 Number: m19803 |
| --- | --- |

| Title: | CE12.1: Ericsson deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin, Kenneth Andersson, Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a deblocking filter developed from the current HEVC deblocking filter. The proposed filter provides an average BD-rate reduction of 1% on the six common test configurations. The decoding time is reported to be 99.9% of HM2.0. It is reported that the subjective performance of the proposal is better than HM 2.0.

# Introduction

The HEVC deblocking filter shows good subjective and objective performance and is computationally simpler than the H.264 deblocking filter. Despite its merits, the HEVC deblocking filter has limited capacity of reducing ringing artifacts as it modifies the first pixel by delta and the second pixel by delta/2. Therefore, small deviations cannot be efficiently smoothed out. Moreover, the same offsets are used for modifying second pixels from both sides of the block border although the structure may be different. The HEVC deblocking filter always modifies at least two pixels from the block border if the deblocking is turned on by the block based filter decision and the clipping value of the delta is not equal to zero. The proposal addresses the stated problems without changing the filtering decisions of the current HEVC deblocking filter. The tc and β thresholds are not changed from HM2.0.

Simulation results show that the proposal decreases the average BD-rate by 1% for all six configurations [2]. The computational complexity of the decoder is 99.9% of the HM2.0.

# Algorithm description

The proposed algorithm consists of few modifications to the HEVC deblocking filter. These modifications are: a modified deblocking filter, the filter length decisions and chroma filtering decisions.

## Filter description

The proposal modifies input pixels according to the following formulas.

 = (9*(q0-p0)-3*(q1-p1))/16

p0’ = p0 + 

q0’ = q0 - 

p1 = (p0 - 2*p1+ p2 +2*)/4;

p1’ = p1 + p1

q1 = (q0 - 2*q1+ q2 -2*)/4;

q1’ = q1 + q1

The proposed filter attenuates small signal deviations stronger than the HEVC filter. When applied to a signal in the form of a slope (10 20 30 40 50 60), the filter does not modify the signal, since , p and q are all equal to zero. When the proposed filter is applied to an unfiltered input signal that has a form close to that of a step function (a blocking artefact), the filter will smooth it. These features are also present in the current HEVC deblocking filter and are kept in the proposal.

## Filtering decisions

The block based filter decision in HEVC is kept and if that allows filtering two additional conditions are checked.

### Check that the artifact is not a natural edge

If a line based decision abs() < (thr1) is fullfilled pixels p0, q0, p1, q1 are allowed to be modified. The delta is equal to zero if the signal on the sides of the block border forms a line and increases with the strength of a blocking artifact. This condition helps to differentiate blocking artifacts from natural edges. It also reduces computational complexity of the filter as p and q are calculated less often. The value of thr1 is set equal to 10*tc(QP) and therefore adapted to the QP.

### Choose between filtering one or two pixels on each side of the block boundary

A block based filtering decision decides between filtering two or one pixels. Pixels p1, q1 are modified if there is a lower spatial activity on a side of the block border when the artifact decision in section 2.2,1 is fulfilled. The block based decision is taken separately for p1, q1. The condition is as in the following. When calculating the value d to decide whether to filter the block boundary 
d = |p22 - 2*p12 + p02| + |q22 - 2*q12 + q02| + |p25 - 2*p15 + p05| + |q25 - 2*q15 + q05|, the values of d on the left and the right from the block boundary are saved as

dp = |p22 - 2*p12 + p02| + |p25 - 2*p15 + p05|
dq = |q22 - 2*q12 + q02| + |q25 - 2*q15 + q05|

These values are then reused to decide whether to filter one or two pixels on each side of the block boundary. Pixel p1 is modified if dp < thr2 and q1 is modified if dq < thr2. The value of thr2 is chosen equal to β(QP)/6 and therefore is QP-adaptive.

## Chroma filtering

The chroma filtering decision has been modified compared to the HM2.0. In HM2.0, the chroma boundary is filtered only if one of the blocks is INTRA. The proposed modification is to add filtering of the chroma block boundary if one of the blocks adjacent to the block boundary has coefficients for the respective chroma component. The tc value is tc(QP), same as for luma component.

# List of modifications

- Formulas for pixel modifications for “weak” deblocking filter are changed (separate deltas for pixels p1 and q1).

- Check for avoiding filtering of natural edges is added.

- Decision to filter p1 and q1 depends on spatial activity on the respective side of block boundary (values used in HM2.0 for block filtering decisions are reused).

- Filtering of chroma boundary if there are chroma coefficients on in one of the adjacent blocks is added.

# Compression performance

The compression performance of the proposed method is shown in the table below. The encoding time is not available as the encoding has been done on heterogeneous cluster. The decoding time on a single computer is shown in the next section. The proposed scheme has been compared against the HM2.0 anchor. The comparison results are given in Table 1. One can see that the proposal shows an average BD-rate [3] reduction of 1%.

Table 1 Comparison of proposed deblocking vs HM2.0 as anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | 0.0 | 0.0 | -1.5 | 0.0 | 0.0 |
| Class B | -0.7 | 0.0 | 0.0 | -0.8 | 0.0 | 0.0 |
| Class C | -0.6 | 0.0 | 0.0 | -0.8 | 0.0 | 0.0 |
| Class D | -0.5 | 0.0 | 0.0 | -0.6 | 0.0 | 0.0 |
| Class E | -0.9 | 0.0 | 0.0 | -1.0 | 0.0 | 0.0 |
| All | -0.7 | 0.0 | 0.0 | -0.9 | 0.0 | 0.0 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | 0.0 | 0.0 | -1.4 | 0.8 | 1.1 |
| Class B | -0.9 | -0.2 | -0.4 | -1.0 | -0.1 | -0.1 |
| Class C | -0.8 | -0.2 | -0.2 | -0.8 | 0.3 | 0.0 |
| Class D | -0.6 | -0.3 | -0.3 | -0.6 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -0.2 | -0.2 | -0.9 | 0.2 | 0.2 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.4 | -1.3 | -1.4 | -1.3 | -0.9 | -0.4 |
| Class C | -1.0 | -0.5 | -0.6 | -0.8 | 0.3 | -0.2 |
| Class D | -0.9 | -0.7 | -0.8 | -0.6 | 0.2 | 0.2 |
| Class E | -3.3 | -3.8 | -3.2 | -2.6 | -4.7 | -3.3 |
| All | -1.5 | -1.4 | -1.4 | -1.2 | -1.0 | -0.7 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

# Complexity considerations

In order to evaluate the complexity of the proposed deblocking, the decoding time on a single machine has been measured for both the proposal and the anchor. The time has been obtained without writing decoded pictures on the disk in order to minimize the influence of the I/O time.

Table 2 Decoding time of the proposal vs HM2.0 decoding time

| Configuration | Dec. time proposal [s] | Dec. time HM2.0 [s] | Dec. time proposal [%] |
| --- | --- | --- | --- |
| Random access | 3560.64 | 3577.89 | 99.5% |
| Low delay | 1693.05 | 1682.46 | 100.6% |
| All intra | 2466.53 | 2507.11 | 98.4% |
| Random access loco | 1220.62 | 1209.93 | 100.9% |
| Low delay loco | 7198.41 | 7209.22 | 99.9 % |
| All intra loco | 2777.47 | 2765.52 | 100.4% |
| Average |  |  | 99.9% |

Table 1 asserts that the proposal has similar complexity compared to HM2.0 anchor.

# Visual inspection

The proposal was compared with HM2.0 for some sequences and coding conditions by blind viewing of HM2.0 and the proposal by expert viewers. The video sequences were shown repeatedly in order to spot the differences. After deciding if they were similar or if one was better the identities were revealed. The outcome was that the proposal was found to be slightly better than the anchor.

# Software

Attached to the contribution is the software for Ericsson’s deblocking filter. The software package contains two files:

TComLoopFilter.h and TComLoopFilter.cpp.

The proposed software modifications are turned on by the following statement in TComLoopFilter.h.

#define ER_DEBLOCK  1

#if ER_DEBLOCK

#define ER_DBL_LUMA 1

#define ER_DBL_CHROMA 1

#else

#define ER_DBL_LUMA 0

#define ER_DBL_CHROMA 0

#endif

The proposal consists of two parts, one is a change to luma and another is a change to chroma deblocking.

Changes to luma are turned on by defining

#define ER_DBL_LUMA 1

Changes to chroma are turned on by defining

#define ER_DBL_CHROMA 1

The default configuration of the proposal has both luma and chroma components turned on.

The modifications for luma include 38 lines of code. The modifications for chroma include 43 lines of code.

# Conclusions

The contribution proposes few modifications to the HEVC deblocking filter. These modifications comprise the modification of the “weak filter”, adding a check for the natural edges and deciding between filtering one or two pixels from the block boundary dependent on the spatial activity. The change to chroma is to add filtering of the chroma component if there are coefficients for this chroma component in one of the adjacent blocks. The filter achieves a BD-rate reduction of 1% on average for the 6 test configurations with similar decoding time. Visual inspection by expert viewers indicates that the subjective performance of the proposal is better than HM2.0. We suggest adoption of this proposal to the next version of HM.

# References

- Andrey Norkin, Kenneth Andersson and Rickard Sjöberg, “Development of HEVC deblocking filter”, JCTVC-D377, Daegu, Jan. 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C600, JCT-VC Meeting, Daegu, Jan. 2010.

- Gisle Bjontegaard, “Calculation of average PSNR differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A. Proposed changes to WD for luma filtering

The proposed changes are marked with editing bars.

##### 8.6.1.4.1 Filtering process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	modified reconstruction of the picture.

Let s’ represent the luma sample array recPictureL of the current picture.

A variable β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Error: Reference source not found with luma quantization parameter ( qPL + 4 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified specified as Error: Reference source not found with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-1)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-2)

- The variables dp, dq and d are derived as follows:

dp = | p2,2 – 2*p1,2 + p0,2 |  + | p2,5 – 2*p1,5 + p0,5 | 		(8-3)

dq = | q2,2 – 2*q1,2 + q0,2 | + | q2,5 – 2*q1,5 + q0,5 |		(8-4)

d = dp+dq

- If bS is not equal to 0 and d is less than β, for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.6.1.4.3 is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’		(8-5)

s’[ xC + xB – i – 1, yC + yB + k ] = pi’		(8-6)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-7)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-8)

- The variables dp, dq and d are derived as follows:

dp = | p2,2 – 2*p1,2 + p0,2 |  + | p2,5 – 2*p1,5 + p0,5 | 		(8-9)

dq = | q2,2 – 2*q1,2 + q0,2 | + | q2,5 – 2*q1,5 + q0,5 |		(8-10)

d = dp+dq

3.	-

If bS is not equal to 0 and d is less than β, for each sample location ( xC + xB + k, yC + yB ), k = 0..7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.6.1.4.3 is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’		(8-12)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’

##### Filtering process for a luma sample

[Ed: (WJ) no filtering when bS is equal to 0]

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	a variable bS specifying the boundary filtering strength,

–	variables d, dp, dq, β and tC.

Output of this process is:

–	number of filtered samples nD,

–	filtered sample values, pi’ and qi’ with i = 0..nD – 1,

When the variable bS is not equal to 0, the following applies:

–	If d is less than ( β >> 2 ), | p3 – p0 | + | q0 – q3 | is less than ( β >> 3 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, the following strong filtering applies while nD is set equal to 3:

p0’ = Clip1Y( ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-13)

p1’ = Clip1Y( ( p2 + p1 + p0 + q0 + 2 ) >> 2 )		(8-14)

p2’ = Clip1Y( ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )		(8-15)

q0’ = Clip1Y( ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-16)

q1’ = Clip1Y( ( p0 + q0 + q1 + q2 + 2 ) >> 2 )		(8-17)

q2’ = Clip1Y( ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )		(8-18)

–	Otherwise, the following weak filtering applies while nD is set equal to 2:

 =  (9*(q0-p0)-3*(q1-p1)+8)>>4		(8-19)

If  abs( )  is smaller than tC *10 the following steps apply

 = Clip3( -tC, tC,  )		(8-20)

p0’ = Clip1Y( p0 +  )		(8-21)

q0’ = Clip1Y( q0 -  )		(8-22)

If  dp is smaller than β/6 the following steps apply

p = Clip3( - (tC>>1), tC>>1, (p2+p0+1)>>1)- p1+)>>1 )

p1’ = Clip1Y( p1 + p )		(8-23)

If  dq is smaller than β/6 the following steps apply

q = Clip3( - (tC>>1), tC>>1, (q2+q0+1)>>1)- q1+)>>1 )

q1’ = Clip1Y( q1 – q )		(8-24)

# Appendix B. Proposed changes to WD for chroma filtering

The proposed changes are marked with editing bars.

#### 8.6.1.3 Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process are three arrays of size (2)x(nS)x(nS), bS, bSCb, bSCr specifying the boundary filtering strength for luma, and two chroma components respectively.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the block edge is also a coding unit edge and the following condition is true, the variable bS[ 0 ][ xEk ][ yEj ] is set equal to 4.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 3.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

- The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

- Depending on the value of filterDir, the variables bSCb[ filterDir ][ xEk ][ yEj ] and bSCr[ filterDir ][ xEk ][ yEj ] are derived as follows.

- If the block edge is also a coding unit edge and the following condition is true, the variables bSCb[ 0 ][ xEk ][ yEj ] and bSCr[ filterDir ][ xEk ][ yEj ] are set equal to 4.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variables bSCb[ 0 ][ xEk ][ yEj ] and bSCr[ filterDir ][ xEk ][ yEj ] are set equal to 3.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise,

- If the following condition is true, the variable bSCb[ filterDir ][ xEk ][ yEj ] is set equal to 2,

-The sample p0 or q0 is in a transform unit which contains non-zero chroma component Cb transform coefficient level.

-Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

- If the following condition is true, the variable bSCr[ filterDir ][ xEk ][ yEj ] is set equal to 2.

-The sample p0 or q0 is in a transform unit which contains non-zero chroma component Cr transform coefficient level.

-Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

#### 8.6.1.4 Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- For xDk set equal to xB+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yB+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7 	(8-25)

    - Filtering process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs and the modified luma picture buffer as output.

- For yDm set equal to yB+( m << 3 ), m=0..nD - 1, the following applies:

- For xDk set equal to xB+( k << 3 ), k=0..nD – 1, the following ordered steps apply:

    - Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i][ yDm ] ) for i = 0..7 	(8-26)

    - Filtering process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, and the boundary filtering strength bSHor as inputs and the modified luma picture buffer as output.

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( Max( log2CUSize, 4 ) – 4 ).

- For xDk set equal to ( xB / 2 )+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to ( yB / 2)+( m << 2 ), m=0..nD*2 – 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = bSCb[ 0 ][ xDk*2 ][ yDm*2 ] 		(8-27)

- Filtering process for chroma block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- For yDm set equal to ( yB / 2 )+( m << 3 ), m=0..nD – 1, the following applies:

- For xDk set equal to ( xB / 2 )+( k << 2 ), k=0..nD*2 – 1, the following ordered steps apply:

- Boundary filtering strength bSHor is derived as follows:

bSHor = bSCr[ 1 ][ xDk*2 ][ yDm*2 ] 		(8-28)

- Filtering process for chroma block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause  is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor as inputs and the modified chroma picture buffer as output.

##### 8.6.1.4.2 Filtering process for chroma block edge

[Ed.: (WJ) cIdx cannot be 0 here]

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left chroma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	variables bS, bSCr and bSCb specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

Output of this process is:

–	modified reconstruction of the picture.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Error: Reference source not found with luma quantization parameter ( qPL + 4 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

qi = s’[ xC + xB +i, yC + yB + k ]		(8-29)

pi = s’[ xC + xB – i – 1, yC + yB + k ]		(8-30)

- If  cIdx is equal to 1 and bSCb is greater than 0, the following ordered steps apply:

- The filtering process for a sample specified in subclause  is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bSCb and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB , yC + yB + k ] = q0’		(8-31)

s’[ xC + xB – 1, yC + yB + k ] = p0’		(8-32)

- If  cIdx is equal to 2 and bSCr is greater than 0, the following ordered steps apply:

- The filtering process for a sample specified in subclause  is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bSCr and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB , yC + yB + k ] = q0’		(8-33)

s’[ xC + xB – 1, yC + yB + k ] = p0’

–	Otherwise (verticalEdgeFlag is equal to 0), for each sample location ( xC + xB +k, yC + yB ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

qi = s’[ xC + xB +k, yC + yB + i ]		(8-34)

pi = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-35)

- If cIdx is equal to 2 and bSCb is greater than 0,  the following ordered steps apply:

- The filtering process for a sample specified in subclause  is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bSCb and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-36)

s’[ xC + xB +k, yC + yB – 1 ] = p0’

- If cIdx is equal to 2 and bSCr is greater than 0,  the following ordered steps apply:

- The filtering process for a sample specified in subclause  is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bSCr and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-37)

s’[ xC + xB +k, yC + yB – 1 ] = p0’

##### 8.6.1.4.4 Filtering process for a chroma sample

[Ed: (WJ) no filtering when bS is equal or less than 2]

Inputs of this process are:

–	sample values, pi and qi with i = 0..1,

–	a variable bS specifying the boundary filtering strength.

–	a variable tC.

Output of this process is:

–	The filtered sample values, p0’ and q0’.

When the variable bS is greater than 1, the filtered sample values p0’ and q0’ are derived by

 = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )	(8-38)

p0’ = Clip1C( p0 +  )		(8-39)

q0’ = Clip1C( q0 -  )

# Appendix C. Contribution of luma and chroma changes to total performance (informative)

The following results are obtained with only modifications for luma. The purpose of this Appendix is to show the independent gains and computational complexity of both parts (luma and chroma) and are informative. The results can be obtained with the attached software by setting the following statements:

#define ER_DEBLOCK  1

#if ER_DEBLOCK

#define ER_DBL_LUMA 1

#define ER_DBL_CHROMA 0

#else

#define ER_DBL_LUMA 0

#define ER_DBL_CHROMA 0

#endif

The BD-rate results for luma modifications are reported in Table 3.

Table 3 Comparison of proposed deblocking vs HM2.0 as anchor for luma only changes

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | 0.0 | 0.0 | -1.5 | 0.0 | 0.0 |
| Class B | -0.7 | 0.0 | 0.0 | -0.8 | 0.0 | 0.0 |
| Class C | -0.6 | 0.0 | 0.0 | -0.8 | 0.0 | 0.0 |
| Class D | -0.5 | 0.0 | 0.0 | -0.6 | 0.0 | 0.0 |
| Class E | -0.9 | 0.0 | 0.0 | -1.0 | 0.0 | 0.0 |
| All | -0.7 | 0.0 | 0.0 | -0.9 | 0.0 | 0.0 |
| Enc Time[%] | 117% |  |  | 96% |  |  |
| Dec Time[%] | 102% |  |  | 97% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -0.1 | -0.1 | -1.4 | -0.1 | 0.0 |
| Class B | -0.9 | 0.0 | -0.1 | -1.0 | 0.0 | 0.0 |
| Class C | -0.8 | -0.1 | -0.1 | -0.8 | 0.0 | -0.1 |
| Class D | -0.6 | -0.1 | 0.0 | -0.6 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -0.1 | 0.0 | -0.9 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 89% |  |  | 97% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.4 | -0.2 | -0.4 | -1.3 | -0.1 | 0.0 |
| Class C | -1.0 | 0.0 | -0.3 | -0.9 | -0.2 | -0.1 |
| Class D | -0.8 | -0.3 | -0.4 | -0.6 | 0.0 | 0.0 |
| Class E | -3.2 | -0.8 | -0.8 | -2.5 | -0.2 | -0.4 |
| All | -1.5 | -0.3 | -0.5 | -1.2 | -0.1 | -0.1 |
| Enc Time[%] | 107% |  |  | 102% |  |  |
| Dec Time[%] | 84% |  |  | 97% |  |  |

Decoding time of the proposal with luma only modifications computed on a single PC is provided in Table 4. The results indicate that chroma gains come with an average complexity increase of 0.3%. One can also see that the objective improvement results of luma and chroma changes are additive.

Table 4 Decoding time of the proposal with only luma modifications vs HM2.0 decoding time.

| Configuration | Dec. time proposal [s] | Dec. time HM2.0 [s] | Dec. time proposal [%] |
| --- | --- | --- | --- |
| Random access | 3552.87 | 3586.48 | 99.1% |
| Low delay | 1684.89 | 1687.22 | 99.9% |
| All intra | 2483.50 | 2515.90 | 98.7% |
| Random access loco | 1214.88 | 1214.70 | 100.0% |
| Low delay loco | 7219.58 | 7231.99 | 99.8% |
| All intra loco | 2780.97 | 2774.11 | 100.2% |
| Average |  |  | 99.6% |

This condition can be computed instead on a higher level, in ”Filtering process for luma block edge subsection” as it is applied to all the lines in the current 8-pixel edge.

This condition can be computed instead on a higher level, in ”Filtering process for luma block edge” subsection as  it is applied to all the lines in the current 8-pixel edge.