| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E277 |
| --- | --- |

| Title: | CE10: Fast integer transform based on modified Loeffler’s factorization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee Jianle Chen Vadim Seregin Yoon Mi Hong Woo-Jin Han Pankaj Topiwala | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com jianle.chen@samsung.com v.seregin@samsung.com yoonmi.hong@samsung.com wjhan.han@samsung.com pankaj@fastvdo.com |
| Source: | Samsung Electronics Co., Ltd. and FastVDO |  |  |

_____________________________

# Abstract

In this proposal, a variant of fast integer transform proposed in JCTVC-D365 is presented. This transform is based on modified Loeffler’s factorization without lifting scheme. The main features of the proposed transform include, a) it can be implemented with both factorized butterfly form or matrix multiplication form; b) 4x4, 8x8 and 16x16 transforms are embedded in 32x32 transform. c) input signal and intermediate transpose buffer for inverse transform are represented within 16 bit; d) six scalar values are used to replace de-quantization scaling matrix for all transform size. Since the proposed transform supports two different forms of implementation with the same performance, the optimal implementation can be chosen depending on which platform will be used. With fast factorization, the average gain over the latest HM-2.0 is 0.1% with 1% encoding and 3% decoding time increase. With matrix multiplication, encoding and decoding time is increased 12% and 18%, respectively.

# Introduction

In this proposal, a variant of fast integer transform proposed in JCTVC-D365 [1] is presented. This transform is based on modified Loeffler’s factorization without lifting scheme.

# Fast integer transform based on modified Loeffler’s factorization

In this transform design, the last 3 stages of odd parts transform are re-organized from the second variants of JCTVC-D365 in order to save number of operations further. Fig. 1 depicts signal flow graph reported in JCTVC-D365. Fig.2 depicts modified signal flow graph which the last 3 stages are re-designed. Calculation of last stages become simpler since cos(π/4) and sin(π /4) have same values.

Figure 1 Signal flow graph of fast integer transform proposed in JCTVC-D365

Figure 2 Modified signal flow graph with simplified last stages

On top of this modification, pre-scaling is incorporated in the transform core in order to give the ability to convert the factorized form into one matrix without loss. Since the intermediate sinusoidal values are approximated as dyadic rational (JCTVC-D365), with proper pre-scaling, round effects inside transform can be completely removed and factorized form can be converted into one matrix. To get the un-scaled output, post-scaling is applied after transform.

Moreover, the pre-scaling can be incorporated into intermediate dyadic rational in order to remove pre-scaling and right-shift for intermediate value calculations. Instead of pre-scaling, intermediate dyadic rational are scaled up to integer value. Since the scaled factorized form is converted into matrix form, matrix multiplication doesn’t require pre-scaling.

Fig. 3 shows simple example of removing rounding error within a butterfly with pre and post-scaling. This can be transformed into Fig. 4, which has same effects by removing the pre-scaling and right-shift in a butterfly. If multiple stages are used for transform design, the post-scaling can be moved to last stage. This modification can remove redundant right shift operations.

Figure 3 Removing rounding error in butterfly with pre- and post-scaling

Figure 4 Removing rounding error in butterfly with integer multiplication and post-scaling

For the whole part of 4x4, 8x8 and 16x16 transforms, lossless matrix conversion is supported. And for 32x32, 6 stages are converted into matrix. To minimize the matrix size, the last stage is not considered into matrix conversion since it only requires simple calculations for 4 output values for 32-point transform. The red box of Fig. 3 represents the part which is converted into matrix form.

Figure 5 Split of signal flow graph to support matrix conversion for 32x32 transform

# De-quantization and intermediate data bit depth restriction

De-quantization process of the proposed solution can be specified with following equation.

dij = ( cij * LevelScale[ qP%6 ] ) << ( qP/6), with i, j = 0..nS-1

where LevelScale[6] = {21, 23, 28, 30, 23, 38}, only 6 va	lues for all transform size block; cij represents transform coefficients after parsing and dij is transformed coefficients after dequantization; nS is size of transform block.

Following pre-scaling and saturation operation is used to restrict the dynamic range of inverse transform input signal with 16 bits representation.

d’ij = Clip3(-215, 215-1, ( dij +offset) >> PreScaleBitblk), with i, j = 0..nS-1

where PreScaleBitblk is equal to 0, 1, 2, 3 respectively for transform block 4x4, 8x8, 16x16 and 32x32.

- Number of operations

Number of operations for inverse transform is given in this section. Table 1, 2, 3 and 4 summarize the number of operations for 32, 16, 8 and 4-point transform including addition, multiplication and shift. According to Table 1 and 2, proposed transform with fast factorization has less number of total operations, and especially less number of multiplications. However, the number of operations for 4-point and 8-point transform in HM is less than the proposed transform. And factorization implementation has significantly less number of arithmetic operations than matrix multiplication implementation, especially for large size transform.

Table 1 Number of operations for 2D 32x32 transform

| 32x32 | addition | multiplication | shift | total |
| --- | --- | --- | --- | --- |
| HM-2.0 | 13440 | 7424 | 4224 | 25088 |
| Matrix | 66816 | 65792 | 2304 | 134912 |
| Factorization | 14976 | 6656 | 2944 | 24576 |

Table 2 Number of operations for 2D 16x16 transform

| 16x16 | addition | multiplication | shift | total |
| --- | --- | --- | --- | --- |
| HM-2.0 | 2624 | 1408 | 832 | 4864 |
| matrix | 8448 | 8192 | 512 | 17152 |
| factorization | 2560 | 1344 | 768 | 4672 |

Table 3 Number of operations for 2D 8x8 transform

| 8x8 | addition | multiplication | shift | total |
| --- | --- | --- | --- | --- |
| HM-2.0 | 576 | 0 | 160 | 736 |
| matrix | 1088 | 1024 | 128 | 2240 |
| factorization | 480 | 256 | 224 | 960 |

Table 4 Number of operations for 2D 4x4 transform

| 4x4 | addition | multiplication | shift | total |
| --- | --- | --- | --- | --- |
| HM-2.0 | 80 | 0 | 16 | 96 |
| matrix | 144 | 128 | 32 | 304 |
| factorization | 80 | 48 | 64 | 192 |

# Bit-width analysis

To check the bit-width of inverse transform, the derivation of bit-width at 4 intermediate stages were done using software developed for B.2.32 of JCTVC-D610 [2]. As you can see, bit-width of all intermediate stage are restricted under 16-bit as depicted in Table 5.

Table 5 Bit-width of inverse transform

|  | IQ | ITInput | ITStage1 | ITOutput |
| --- | --- | --- | --- | --- |
| 4x4 | 12 | 16 | 16 | 11 |
| 8x8 | 13 | 16 | 16 | 12 |
| 16x16 | 14 | 16 | 16 | 12 |
| 32x32 | 15 | 16 | 16 | 12 |

# Results

With fast factorization, the average gain over the latest HM-2.0 under common test condition (JCTVC-D600 [3]) is 0.1% with 1% encoding and 3% decoding time increase.

Table 6 Test results with common test condition (fast factorization)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.07 | -0.47 | -0.31 | -0.83 | -0.40 | -0.31 |
| Class B | -0.10 | -0.12 | -0.14 | -0.30 | -0.37 | -0.43 |
| Class C | 0.07 | -0.23 | -0.30 | -0.10 | -0.45 | -0.66 |
| Class D | 0.05 | -0.14 | -0.22 | -0.07 | -0.47 | -0.66 |
| Class E | -0.08 | -0.29 | -0.39 | -0.48 | -0.43 | -0.59 |
| All | -0.03 | -0.24 | -0.26 | -0.35 | -0.42 | -0.52 |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 109% |  |  | 102% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.3 | -0.5 | 0.3 | 0.4 |
| Class B | 0.0 | 0.0 | 0.2 | -0.1 | 0.2 | 0.3 |
| Class C | 0.1 | 0.2 | 0.1 | 0.0 | 0.2 | 0.2 |
| Class D | 0.1 | 0.1 | 0.3 | 0.0 | 0.5 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.1 | -0.1 | 0.3 | 0.3 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.6 | 0.3 | -0.1 | 0.3 | 0.5 |
| Class C | 0.0 | 0.5 | 0.2 | -0.1 | 0.4 | 0.2 |
| Class D | 0.0 | 0.0 | -0.1 | 0.1 | 0.4 | 0.5 |
| Class E | -0.3 | -0.8 | 0.6 | -0.1 | 0.3 | 0.3 |
| All | -0.1 | 0.1 | 0.2 | -0.1 | 0.4 | 0.4 |
| Enc Time[%] | 102% |  |  | 96% |  |  |
| Dec Time[%] | 106% |  |  | 95% |  |  |

With matrix multiplication, the average gain over the latest HM-2.0 under common test condition (JCTVC-D600 [3]) is same with fast factorization with 12% encoding and 18% decoding time increase.

Table 7 Test results with common test condition (matrix multiplication)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.5 | -0.3 | -0.8 | -0.4 | -0.3 |
| Class B | -0.1 | -0.1 | -0.1 | -0.3 | -0.4 | -0.4 |
| Class C | 0.1 | -0.2 | -0.3 | -0.1 | -0.4 | -0.7 |
| Class D | 0.1 | -0.1 | -0.2 | -0.1 | -0.5 | -0.7 |
| Class E | -0.1 | -0.3 | -0.4 | -0.5 | -0.4 | -0.6 |
| All | 0.0 | -0.2 | -0.3 | -0.3 | -0.4 | -0.5 |
| Enc Time[%] | 124% |  |  | 126% |  |  |
| Dec Time[%] | 121% |  |  | 126% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.3 | -0.5 | 0.3 | 0.4 |
| Class B | 0.0 | 0.0 | 0.2 | -0.1 | 0.2 | 0.3 |
| Class C | 0.1 | 0.2 | 0.1 | 0.0 | 0.2 | 0.2 |
| Class D | 0.1 | 0.1 | 0.3 | 0.0 | 0.5 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.1 | -0.1 | 0.3 | 0.3 |
| Enc Time[%] | 107% |  |  | 105% |  |  |
| Dec Time[%] | 110% |  |  | 111% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.6 | 0.3 | -0.1 | 0.3 | 0.5 |
| Class C | 0.0 | 0.5 | 0.2 | -0.1 | 0.4 | 0.2 |
| Class D | 0.0 | 0.0 | -0.1 | 0.1 | 0.4 | 0.5 |
| Class E | -0.3 | -0.8 | 0.6 | -0.1 | 0.3 | 0.3 |
| All | -0.1 | 0.1 | 0.2 | -0.1 | 0.4 | 0.4 |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 117% |  |  | 120% |  |  |

The results for low QPs and high QPs are provided to check if there is abnormal behavior at the extreme conditions. The tests were done under test conditions defined in B.1.2 of JCTVC-D610 [2]. The average gain at low QPs is 0.8% with 4% encoding time increase and 6% decoding time decrease. The average gain at high QPs is 0.0% with 3% encoding and 3% decoding time increase. Encoding and decoding time are inaccurate since clustered systems usually utilize different CPUs for each test. (The encoding and decoding time will be updated) The results are given in Table 8 and 9.

Table 8 Results at low QPs (Butterfly form)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -1.0 | -1.0 | -0.8 | -0.6 | -0.6 |
| Class B | -0.3 | -0.9 | -1.0 | -1.6 | -0.7 | -0.8 |
| Class C | 0.2 | -0.2 | -0.2 | 0.7 | 1.4 | 1.3 |
| Class D | 0.2 | -0.2 | -0.2 | 0.5 | 1.1 | 1.1 |
| Class E | -1.6 | -1.7 | -1.7 | -1.4 | -1.6 | -1.8 |
| All | -0.3 | -0.7 | -0.8 | -0.5 | 0.0 | 0.0 |
| Enc Time[%] | 106% |  |  | 109% |  |  |
| Dec Time[%] | 97% |  |  | 92% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.5 | -0.3 | -1.2 | -0.4 | -0.4 |
| Class B | -0.7 | -0.3 | -0.6 | -1.6 | -0.3 | -0.4 |
| Class C | -0.4 | 0.0 | 0.0 | -0.8 | -0.1 | -0.3 |
| Class D | -0.3 | 0.3 | 0.2 | -0.8 | -0.2 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.1 | -0.2 | -1.1 | -0.2 | -0.4 |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | -0.3 | -0.5 | -2.1 | -0.3 | -0.3 |
| Class C | -0.5 | -0.1 | -0.1 | -1.2 | -0.2 | -0.3 |
| Class D | -0.6 | 0.2 | 0.0 | -1.3 | -0.4 | -0.4 |
| Class E | -1.0 | -0.7 | -0.5 | -1.7 | -0.7 | -0.8 |
| All | -0.7 | -0.2 | -0.3 | -1.6 | -0.4 | -0.4 |
| Enc Time[%] | 98% |  |  | 102% |  |  |
| Dec Time[%] | 94% |  |  | 92% |  |  |

Table 9 Results at high QPs (Butterfly form)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.3 | -0.1 | 0.4 | 0.3 |
| Class B | 0.0 | 0.2 | 0.4 | 0.0 | 0.4 | 0.4 |
| Class C | 0.0 | 0.0 | -0.1 | 0.1 | -0.7 | -0.2 |
| Class D | 0.0 | 0.7 | 0.0 | 0.0 | -0.8 | -1.0 |
| Class E | 0.3 | 0.4 | 0.1 | 0.2 | -0.2 | 0.3 |
| All | 0.1 | 0.3 | 0.1 | 0.1 | -0.2 | -0.1 |
| Enc Time[%] | 115% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 108% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.2 | 0.8 | -0.2 | 0.3 | -0.5 |
| Class B | 0.2 | -0.3 | 1.1 | -0.1 | -1.0 | -0.3 |
| Class C | -0.2 | 0.5 | 0.6 | 0.0 | -1.2 | -0.1 |
| Class D | -0.2 | -1.1 | -1.3 | 0.0 | -2.7 | -0.5 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.2 | 0.3 | 0.0 | -1.3 | -0.4 |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.8 | -0.1 | 0.2 | -0.3 | 1.1 |
| Class C | 0.2 | 0.8 | -0.2 | 0.1 | -1.4 | -2.2 |
| Class D | 0.5 | -1.2 | 0.0 | 0.0 | -2.6 | -1.6 |
| Class E | 0.0 | -2.0 | 0.3 | -0.3 | -3.2 | -1.3 |
| All | 0.1 | -0.7 | 0.0 | 0.0 | -1.7 | -0.9 |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |

# Conclusions

In this proposal, a variant of fast integer transform proposed in JCTVC-D365 [1] was presented. This transform was based on modified Loeffler’s factorization without lifting scheme. To reduce the number of operations further, the last 3 stages of odd parts of transform were re-designed.

In order to give the ability to convert the factorized form into one matrix without loss, pre-scaling was incorporated in the transform core. Because this proposal supports two different forms of implementation with the same performance, the optimal implementation can be chosen depending on which platform will be used.

In addition, 4x4, 8x8 and 16x16 transforms are obtained from 32x32 transform to be used as embedded form. The input signal and intermediate transpose buffer for inverse transform are represented within 16 bit and the de-quantization matrix is removed by scalar values. With fast factorization, the average gain over the latest HM-2.0 is 0.1% with 1% encoding and 3% decoding time increase. With matrix multiplication, encoding and decoding time is increased 12% and 18%, respectively.

We recommend adopting the proposed transform solution, which supports both of known full factorization implementation and matrix multiplication implementation, into the next version of HM.

# References

- JCT-VC, “Fast Integer Transforms for the HM, and Complexity Analysis”, JCTVC-D365, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “Description of Core Experiment 10 (CE10): Core Transform Design”, JCTVC-D610, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd.  and FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Transformation process for 4 samples :

for ( i = 0; i < 4; i++ ) {

sum = 0;

for ( j = 0; j < 4; j++ ) {

sum += IT4[i][j]*Src[j];

}

Des[i] = sum >> 10;

}

where IT4 is

Transformation process for 8 samples :

for ( i = 0; i < 8; i++ ) {

sum = 0;

for ( j = 0; j < 8; j++ ) {

sum += IT8[i][j]*Src[j];

}

Des[i] = sum >> 10;

}

where IT8 is

Transformation process for 16 samples :

for ( i = 0; i < 16; i++ ) {

sum = 0;

for ( j = 0; j < 16; j++ ) {

sum += IT16[i][j]*Src[j];

}

Des[i] = sum >> 10;

}

where IT16 is

Transformation process for 32 samples :

prevX19 = src[19];

prevX13 = src[13];

prevX27 = src[27];

prevX5  = src[5];

src[13] = (-22*(prevX19 + prevX13)) >> 5;

src[19] = (-22*(prevX19 - prevX13)) >> 5;

src[27] = (-22*(prevX27 + prevX5)) >> 5;

src[5]  = (-22*(prevX27 - prevX5)) >> 5;

for ( i = 0; i < 32; i++ ) {

sum = 0;

for ( j = 0; j < 32; j++ ) {

sum += IT32[i][j]*Src[j];

}

Des[i] = sum >> 10;

}

where IT32 is