| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E278 |
| --- | --- |

| Title: | CE6.b1 Report on Short Distance Intra Prediction Method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoran Cao, Tsinghua Xiulian Peng, USTC Changcai Lai, HiSilicon Yunfei Wang, Tsinghua Yongbing Lin, HiSilicon Jizheng Xu, Microsoft Lingzhi Liu, HiSilicon Jianhua Zheng, HiSilicon Yun He, Tsinghua Haoping Yu, Huawei Feng Wu, Microsoft | Email: | laichangcai@huawei.com jzxu@microsoft.com haopingyu@huawei.com |
| Source: | Tsinghua University, University of Science and Technology of China, HiSilicon, Microsoft, Huawei |  |  |

_____________________________

# Abstract

This document reports the experimental results of the short distance intra prediction (SDIP) scheme for core Experiment 6 on intra prediction improvement. SDIP was presented in JCTVC-C101 and JCTVC-C270. By dividing the NxN block into lines or non-square blocks, SDIP can reduce the energy of the prediction residuals by reducing the distance of predicted pixel and its reference pixels. When integrated into the HM2.0 software, it was reportedly shows 2.5% and 4.0% bit rate saving on average, under all intra high efficiency and low complexity conditions, respectively, with about 24% (intra) and 47% (intra loco) encoding time increase and no obviously decoding time increase. Up to 6.4% bit rate saving is achieved on sequences with rich textures.

# Introduction

The traditional block-based intra coding in AVC/KTA/HM uses one NxN square block as the reconstruction unit and prediction unit. The pixels inside a square block are all predicted from the boundaries of neighboring reconstructed blocks, producing poor predictions for pixels on the right-bottom part than the others in some regions of sequences. To better exploit spatial correlations, the short distance intra prediction coding scheme is proposed by partitioning one NxN square block into several lines or non-square blocks with rectangle shape. In the block, pixels are predicted and reconstructed line by line or rectangle by rectangle. Therefore, the prediction distance can be obviously shortened.

# Algorithm description

In SDIP, one NxN square block which is smaller than 32x32 is divided into several lines or non-square blocks with rectangle shape. In the block, pixels are predicted and reconstructed line by line or rectangle by rectangle.

## Block Partitions

In SDIP modes, one CU that smaller than 64x64 can be partitioned as lines or non-square blocks with rectangle shape as Fig.1 shows. One 32x32 CU can be partitioned as four 8x32 or 32x8 Pus, and the 16x16 CU can not only be divided into four 8x8 PU as did in HM, but also be divided into four 4x16/16x4 PU, and a 4x16/16x4 PU can be further split into four 1x16/16x1 partitions. As similar, one 8x8 CU can also be divided into four 2x8/8x2 PU, and every 4x4 PU can be further divided into four 1x4/4x1 partitions. Hence, two types of PU are supported in SDIP, the first is rectangular PU named as hNx2N/2NxhN, where h means half (1/2), the second is line based PU named as 1xN/Nx1. For 32x32 CU, only rectangular SDIP PU is used. For 16x16 and 8x8 CU, both the rectangular and line based PU are supported because there are more textures in these kind of CUs.

Fig. 1 SDIP block partitions based on HM

## SDIP intra prediction and coding

Fig.2 UDI prediction for 2x8

The prediction, transform and entropy modules in HM2.0 can be reused for SDIP partitions. In the prediction for SDIP partitions, UDI of HM is used as Fig.2 and the UDI modes can be from 9 to 34 and the number of modes is configurable as HM. One UDI mode flag is identified for every 4x16/16x4 or 2x8/8x2 partition. If a 4x16/16x4 is divided into four 1x16/16x1, the 1x16/16x1 partitions used the same modes that identified in 4x16/16x4 and similar for 1x4/4x1 partitions in 4x4 PU. In the prediction of SDIP blocks and square blocks, we used the bidirectional intra prediction for mode 6 (BUDI) as described in JCTVC-Exxx [1].

In the SDIP modes, all four luma prediction modes in the current CU and the four modes 0-3 are tested in mode decision, among which the four luma modes are mapped to low indices and modes 0-3 are mapped to high ones. Only the first five modes by the new indices are searched in mode decision. The new index is then transmitted in the bitstream instead of the original modes. This approach is also applied to blocks not selecting SDIP mode, where SIZE_NxN partitions might benefit from it.

The same transform matrices in HM2.0 were reused in SDIP modes but the transform size is partition size related and the quantization scale matrix is modified respectively. The nxm blocks are transformed by the following steps as described in [2].

Cnxm = Tm x Bnxm x TnT              	(1)

where Bnxm denotes a block with n pixels m rows. Tn and Tm are the transform matrices of size nxn and mxm, respectively. Cnxm denotes the transformed nxm block.

At the entropy coding stage, different scanning orders and contexts are designed for different partitions. Generally, for rectangular partitions, the coefficients are first scanned from high frequency to low frequency into a 1D buffer as did in MDCS of HM2.0, and then reorganized into a 4x4 or 8x8 block and coded as traditional. For 1x16 partitions, since the transform is one dimensional, there is no need to scan from high to low. The coefficients are directly reorganized into 4x4 and coded. For 4x4 using 1x4 transforms, the coefficients of a 4x4 block are coded as a whole. They are scanned from four DC coefficients to other AC coefficients. The significant map is coded using the frequency as a context, i.e. there are four context models for 4x4. The coefficient levels are coded similar to 4x4 in HM. In SDIP modes, RDOQ is used in both CABAC and LCEC.

# Test conditions and Results

The proposal has been integrated into HM2.0. The results were tested in Intra and Intra LoCo cases under the recommended test conditions of CE6. The following platform and compiler have been used and results were shown in Table1.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2003, 64 bits | Intel Xeon X7560 @2.27GHz | 256G RAM | VS2008 |

Table1. Results of normal configuration

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -0.5 | -0.8 | -2.1 | -2.0 | -2.6 |
| Class B | -1.9 | -1.0 | -1.4 | -3.7 | -3.7 | -4.8 |
| Class C | -3.5 | -2.4 | -2.6 | -4.8 | -5.0 | -5.7 |
| Class D | -3.6 | -2.1 | -2.2 | -5.1 | -4.8 | -5.2 |
| Class E | -2.9 | -2.2 | -2.1 | -4.9 | -7.5 | -6.0 |
| All | -2.5 | -1.6 | -1.8 | -4.0 | -4.4 | -4.8 |
| Enc Time[%] | 124% |  |  | 147% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |

The test results show that 2.5% and 4.0% bit rate saving on average, under all intra high efficiency and low complexity conditions, respectively. Up to 6.4% bit rate saving is achieved on sequences with rich textures.

Additional results: 1x4_OFF configuration

The purpose of this configuration is to test the performance of SDIP while turning off the 1x4/4x1 mode for 4x4 PU. The following platform and compiler have been used and Table3 show the results of this configuration.

Table 2. Results of 1x4_OFF configuration

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -0.6 | -0.9 | -2.0 | -2.0 | -2.6 |
| Class B | -1.9 | -1.1 | -1.5 | -3.6 | -3.8 | -4.9 |
| Class C | -3.2 | -2.4 | -2.6 | -4.1 | -5.0 | -5.7 |
| Class D | -3.2 | -2.1 | -2.2 | -4.2 | -4.7 | -5.0 |
| Class E | -3.0 | -2.4 | -2.2 | -4.8 | -7.5 | -6.1 |
| All | -2.4 | -1.6 | -1.8 | -3.7 | -4.4 | -4.8 |
| Enc Time[%] | 116% |  |  | 133% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The test results show that 2.4% and 3.7% bit rate saving on average, under all intra high efficiency and low complexity conditions, respectively. Up to 6.2% bit rate saving is achieved on sequences with rich textures.

# Conclusion

This document reports the experimental results of SDIP scheme for core Experiment 6 on intra prediction improvement. When integrated into the HM2.0 software, it shows 2.5% and 4.0% bit rate saving on average, under all intra high efficiency and low complexity conditions. Up to 6.4% bit rate saving is achieved on sequences with rich edges.

# References

- Y. Lin, C. Lai, J. Zheng, L. Liu, “Improved Intra prediction for positive directions in UDI”, Doc. JCTVC-D300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

- Mathias Wien and Achim Dahlhoff, “Adaptive Block Transforms”, Doc.VCEG-M62, Q.6/SG16 (VCEG), Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001

# Patent rights declaration(s)

Tsinghua University, University of Science and Technology of China, Microsoft, Huawei and HiSilicon may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).