| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E279 |
| --- | --- |

| Title: | Extensible High Layer Syntax for Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Stephan Wenger 433 Hackensack Ave. Hackensack, N.J. 07601 USA | Tel: +1-201-478-6145 Email: {jill, danny, stephan}@vidyo.com |  |
| Source: | Vidyo, Inc |  |  |

_____________________________

# Abstract

H.264/AVC high layer syntax was originally designed without considering the future extensions for scalability and multi-view coding.  When H.264/AVC was extended for SVC and MVC, the high layer syntax had to be retrofitted, in a backwards-compatible manner.  Considering possible extensions now should lead to a cleaner high layer syntax design for HEVC.  This contribution proposes the introduction of a new Dependency Parameter Set and two new SEI messages, as well as changes to the NAL unit header, Sequence Parameter Set, Picture Parameter Set, and slice header, for better support of scalability and future extensions.

# Introduction

The proposed high layer syntax changes are summarized in the list below, and detailed in the following section.  These changes are primarily directed at supporting scalability, but in some cases have applicability to multi-view as well.

- Dependency Parameter Set: Introduce a Dependency Parameter Set (DPS), above the Sequence Parameter Set, to tie together related sequences, and describe their dependency relationships.  Can be used for scalability and multi-view, or future extensions (e.g. multiple description coding, etc.).

- NAL Unit Header: Add a byte to NAL unit header for scalability information – spatial_id and quality_id, for certain NAL unit types.  Add store_ref_base_pic_flag and use_ref_base_pic_flag to support MGS key pictures.

- Sequence Parameter Set: Add information about temporal scalability layers.  Add the temporal_id_nesting_flag syntax element as proposed in JCTVC-D200.  Add sps_extension_flag for future extensions, including sequence level scalability parameters.

- Picture Parameter Set: Add temporal_layer_switching_point_flag, similar to the one proposed in JCTVC-D200, but with some modifications. Add pps_extension_flag for future extensions, including picture level scalability parameters.

- Temporal structure SEI message: New message which adds active # of temporal layers to the coding structure SEI message proposed in JCTVC-D200.

- Spatial and quality layer SEI message: New message containing the active # of spatial layers, and active # of quality layers in the highest active spatial layer.

- In slice header, put pic_parameter_set_id first, so that it is easier to find PPS, without having to entropy decode additional fields. Add slice_header_extension_flag for future extensions, including slice level scalability parameters.

# Proposal

## Dependency Parameter Set (DPS)

The SVC and MVC designs were complicated by the lack of an existing structure to associate multiple related sequences.  SVC includes a scalability information SEI message that provides some information about the relationships, but because it is optional, gateways cannot depend upon its availability.  Normative scalability relationship information will aid gateways to intelligently prune bitstreams.  Creation of a higher layer parameter set, above the sequence parameter set, is proposed to describe the relationship between related sequences.  A suggested name is the Dependency Parameter Set (DPS). It is immediately applicable to scalability, but could also be used for multi-view, or other future extensions involving multiple related sequences, for example multiple description coding.

Within the proposed DPS, a multiview_flag and an extension_flag are proposed, but their parameters not defined at this time.  A scalability_flag is proposed, along with proposed scalability parameters.  It is also proposed that the VUI information be divided between the DPS and SPS, so that the parameters that must be identical for all related sequences are sent in the DPS VUI, while the other parameters can remain in the SPS VUI.  Because the WD does not yet contain a description of the SPS VUI, this contribution doesn’t attempt to allocate parameters between the DPS VUI and the SPS VUI.

The scalability parameters in the DPS are utilized for spatial and quality scalability, but are not required for temporal scalability, as temporal layers are provided within a single spatial/quality layer.  Each spatial and quality layer has its own Sequence Parameter Set (SPS).

In this design, a spatial layer can be dependent on the spatial layer immediately preceding it, but not on other spatial layers.  This provides less flexibility than that allowed in SVC, which permits a spatial layer to be dependent on spatial layers other than the spatial layer immediately preceding it.  Reducing the flexibility simplifies the design and implementation, and reduces the amount of dependency signaling required.  If a spatial layer is dependent on a lower spatial layer that contains quality layers, it is dependent on the highest quality layer.

| dep_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| dep_parameter_set_id | u(2) |
| scalability_flag | u(1) |
| if( scalability_flag ) { |  |
| max_spatial_layers_minus1 | u(3) |
| for( i = 0; i <= max_spatial_layers_minus1; i++ ) { |  |
| dependency_flag[ i ] | u(1) |
| max_quality_layers_minus 1[ i ] | u(4) |
| } |  |
| } |  |
| multiview_flag | u(1) |
| if( multiview_flag ) { |  |
| // undefined |  |
| } |  |
| dps_vui_parameters_present_flag | u(1) |
| if( dps_vui_parameters_present_flag ) { |  |
| // undefined |  |
| } |  |
| extension_flag | u(1) |
| if( extension_flag ) { |  |
| // undefined |  |
| } |  |
| } |  |

dep_parameter_set_id identifies the dependency parameter set that is referred to by the sequence parameter set. The value of dep_parameter_set_id shall be in the range of 0 to 3, inclusive.

scalability_flag equal to 1 specifies that multiple spatial and/or quality layers may be present.   scalability_flag equal to 0 specifies that only a single spatial layer and single quality layer is present.

max_spatial_layers_minus1 + 1 specifies the maximum number of spatial layers that may be present. max_spatial_layers_minus1 shall be in the range of 0 to 7, inclusive.

dependency_flag[ i ] specifies whether inter-layer prediction from the highest quality layer of the coded sequence with spatial_id equal to i  may be used for decoding the coded slice with spatial_id equal to i + 1.  If dependency_flag[ i ] is equal to 1, inter-layer prediction may be used.  If dependency_flag[ i ] is equal to 0, inter-layer prediction may not be used.

max_quality_layers_minus_1 + 1 specifies the maximum number of quality layers that may be present for the spatial layer with spatial_id equal to i. max_quality_layers_minus_1 shall be in the range of 0 to 15, inclusive.

multiview_flag shall be equal to 0.

extension_flag shall be equal to 0.

## NAL unit header

Additional fields for spatial_id and quality_id are proposed for inclusion in the NAL unit header, for the NAL unit types allocated to spatial and quality layers, similar to the SVC design.

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_idc | u(2) |
| nal_unit_type | u(5) |
| NumBytesInRBSP = 0 |  |
| nalUnitHeaderBytes = 1 |  |
| if( nal_unit_type = = 1 ¦ ¦ nal_unit_type = = 5 ) { |  |
| temporal_id | u(3) |
| output_flag | u(1) |
| store_ref_base_pic_flag | u(1) |
| use_ref_base_pic_flag | u(1) |
| reserved_zero_2bits | u(2) |
| nalUnitHeaderBytes += 1 |  |
| } |  |
| if( nal_unit_type = = ? ¦ ¦ nal_unit_type = = ? ) { |  |
| spatial_id | u(3) |
| quality_id | u(4) |
| reserved_zero_bit | u(1) |
| nalUnitHeaderBytes += 1 |  |
| } |  |
| … |  |

temporal_id specifies a temporal layer identifier for the NAL unit.  temporal_id shall be in the range of 0 to max_temporal_layers_minus_1, inclusive.

store_ref_base_pic_flag equal to 1 specifies that, when the value of spatial_id as specified in the NAL unit header is equal to the maximum value of the spatial_id for the VCL NAL units of the current coded picture, an additional representation of the coded picture that may or may not be identical to the decoded picture is marked as “used for reference”.  This additional representation is also referred to as reference base picture and may be used for inter prediction of following pictures in decoding order, but it is not output.

The syntax element store_ref_base_pic_flag shall have the same value for all VCL NAL units of a dependency representation.  When nal_ref_idc is equal to 0, store_ref_base_pic_flag shall be equal to 0.

When max_num_ref_frames is less than 2 in the sequence parameter set that is referred by the associated NAL unit, store_ref_base_pic_flag shall be equal to 0.

use_ref_base_pic_flag equal to 1 specifies that reference base pictures (when present) and decoded pictures (when reference base pictures are not present) are used as reference pictures for inter prediction.  use_ref_base_pic_flag equal to 1 specifies that reference base pictures are not used as reference pictures for inter prediction.

The values of use_ref_base_pic_flag shall be the same for all NAL units of a spatial represention.

### spatial_id specifies a spatial layer identifier for the NAL unit.  spatial_id shall be in the range of 0 to max_spatial_layers_minus_1, inclusive.

quality_id specifies a quality layer identifier for the NAL unit.  quality_id shall be in the range of 0 to max_quality_layers_minus_1, inclusive.

## Sequence Parameter Set

A syntax element is added to refer to the Dependency Parameter Set id.  A syntax element is added to indicate the maximum number of temporal layers.

A temporal_id_nesting_flag syntax element is added, as proposed in JCTVC-D200.  If temporal_id_nesting_flag is set to 1, when a lower temporal_id picture is decoded, pictures with higher temporal_id values can no longer be used for prediction.   This is enforced by setting as “unused for reference” all pictures in the reference picture list with higher temporal_id values, which means that those pictures will be removed from reference picture memory.

An sps_extension_flag is included, but the parameters to be sent if it is set are not defined.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| seq_parameter_set_id | u(5) |
| dep_parameter_set_id | u(2) |
| reserved_zero_bit | u(1) |
| profile_idc | u(8) |
| level_idc | u(8) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| bit_depth_luma_increment | ue(v) |
| bit_depth_chroma_increment | ue(v) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| interpolation_filter_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { | u(1) |
| //undefined |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

dep_parameter_set_id refers to the active dependency parameter set. The value of dep_parameter_set_id shall be in the range of 0 to 3, inclusive.

max_temporal_layers_minus1 + 1 specifies the number of temporal layers present in the sequence.

temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the target access unit set.

Dependent on temporal_id_nesting_flag, the following applies.

– If temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

– Otherwise (temporal_id_nesting_flag is equal to 1), the following constraint shall be obeyed for layer representations with any spatial_id or quality_id values present in the target access unit set.

For each access unit auA with temporal_id equal to tIdA, an access unit auB with temporal_id equal to tIdB and tIdB  less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with

temporal_id equal to tIdC and tIdC less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order.

NOTE  – The syntax element temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific temporal_id tIdN to decoding up to a temporal_id tIdM > tIdN, is always possible.

sps_extension_flag shall be equal to 0.

## Picture Parameter Set:

This contribution proposes adding a temporal_layer_switching_point_flag, similar to that proposed in JCTVC-D200, but moving it from the NAL unit header to the PPS, and setting the value separately for each temporal level.   This allows a tradeoff in providing switching points vs. improving coding efficiency by allowing multiple reference pictures.  The decoding reference picture marking process is impacted, by marking as “unused for reference” those reference pictures that may not be referred to, eliminating the need to send dec_ref_pic_marking()/MMCO syntax elements to eliminate those reference pictures from the reference list.   The reference software has been modified to incorporate this proposed feature, and is available with the contribution.

A pps_extension_flag is added for extensions, including picture level scalability parameters.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | u(5) |
| seq_parameter_set_id | u(5) |
| entropy_coding_mode_flag | u(1) |
| temporal_layer_switching_params_present_flag | u(1) |
| if ( temporal_layer_switching_params_present_flag == 1) { |  |
| for( i = 0; i < max_temporal_layers_minus1; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| } |  |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) { |  |
| //undefined |  |
| } |  |
| } |  |

temporal_layer_switching_params_present_flag specifies if parameters for temporal switching point flags are present.   If temporal_id_nesting_flag is equal to 1, temporal_layer_switching_params_present_flag shall be equal to 0.

temporal_layer_switching_point_flag[ i ] specifies if the current access point is a temporal switching point allowing the decoding of higher temporal id layers following this access unit.  If temporal_id_nesting_flag is equal to 1, temporal_layer_switching_point_flag[ i ] shall be inferred to be equal to 1 .  If temporal_id_nesting_flag is equal to 0 and temporal_layer_switching_params_present_flag is equal to 0, temporal_layer_switching_point_flag[ i ] shall be inferred to be equal to 0 .

If temporal_switching_point_flag[ i]  is equal to 1, all pictures with values of temporal_id higher than i in the reference picture storage are marked as “unused for reference”, as specified in subclause X.

If temporal_id_nesting_flag is equal to 1, temporal_switching_point_flag[ i ] shall be equal to 1.

NOTE – When starting to decode a high temporal layer, availability of required reference pictures can be guaranteed immediately following an IDR, or a picture with a value of temporal_id lower than i and temporal_switching_flag equal to 1.

pps_extension_flag shall be equal to 0.

## Temporal structure SEI message

This SEI message indicates the number of active temporal layers in a bitstream.  If a gateway prunes a bitstream by removing one or more temporal layers, this fixed bit width syntax element can easily be modified.

This SEI message also optionally includes the coding structure that was proposed in JCT-D200.

| temporal_structure( payloadSize ) { | Descriptor |
| --- | --- |
| num_active_temporal_layers_minus1 | u(3) |
| coding_structure_flag | u(1) |
| if( coding_structure_flag ) { |  |
| num_pictures_in_sop_minus1 | ue(v) |
| num_sops_in_gop | ue(v) |
| for( i = 0; i < num_pictures_in_sop_minus1 + 1; i++ ) { |  |
| primary_pic_type[ i ] | u(2) |
| ref_flag[ i ] | u(1) |
| temporal_num[ i ] | u(3) |
| display_num[ i ] | ue(v) |
| } |  |
| } |  |
| coding_statistics_flag | u(1) |
| if( coding_statistics_flag ) { |  |
| average_frame_rate_flag | u(1) |
| average_bit_rate_flag | u(1) |
| if( average_frame_rate_flag ) |  |
| average_frame_rate | u(16) |
| if( average_bit_rate_flag ) |  |
| for( i = 0; i < NumTemporalLayers; i++ ) |  |
| average_bit_rate[ i ] | u(16) |
| } |  |
| } |  |

num_active_temporal_layers_minus1 + 1 specifies how many temporal layers are present in the bitstream.  num_active_temporal_layers_minus1 shall be in the range of 0 to max_temporal_layers_minus1, inclusive.

## Spatial and quality layers SEI message

This SEI message indicates the number of active spatial layers in a bitstream, and the number of quality layers per spatial layer.  If a gateway prunes a bitstream by removing one or more spatial or quality layers, the fixed bit width syntax elements can easily be modified.

| spatial_quality_layers( payloadSize ) { | Descriptor |
| --- | --- |
| num_active_spatial_layers_minus1 | u(3) |
| num_active_quality_layers_minus1 | u(4) |
| } |  |

num_active_spatial_layers_minus1 + 1 specifies how many spatial layers are present in the bitstream.  num_active_spatial_layers_minus1 shall be in the range of 0 to max_spatial_layers_minus1, inclusive.

num_active_quality_layers_minus1 + 1 specifies how many quality layers are present in the spatial layer with spatial_id equal to num_active_spatial_layers_minus1.   num_active_quality_layers_minus1 shall be in the range of 0 to max_quality_layers_minus1 [num_active_spatial_layers_minus1],  inclusive.

## Slice header

The pic_parameter_set_id field is moved to be the first element in the slice header, so that it can be more easily accessed by a gateway, without having to parse additional syntax elements.

| slice_header( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| first_tb_in_slice | ue(v) |
| slice_type | ue(v) |
| … |  |
| slice_header_extension_flag | u(1) |
| if( slice_header_extension_flag ) { |  |
| //undefined |  |
| } |  |
| } |  |

slice_header_extension_flag shall be equal to 0.

# Software

Software has been made available with the contribution for proposals #3 and 4, involving the temporal_id_nesting_flag in the SPS and temporal_layer_switching_point_flag in the PPS, which modify the reference picture marking process.  No experimental results have been made available, as the proposal is aimed at design cleanliness, not bitrate savings, by eliminating the need to send dec_ref_pic_marking() syntax elements for reference pictures that may no longer be used as references.

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).