| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E280 |
| --- | --- |

| Title: | Picture Orientation Information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger, Jill Boyce, Danny Hong, Adeel Abbas 433 Hackensack Ave. Hackensack, N.J. 07601 USA | Tel: +1-415-713-5473 Email: {stephan, jill,danny, adeel}@vidyo.com |  |
| Source: | Vidyo, Inc |  |  |

_____________________________

# Abstract

This contribution proposes the inclusion of picture orientation information in the video bitstream at the sequence level, which indicates to the decoder to rotate the image after the normal decoding process, at angles of 0, 90, 180 or 270 degrees.  The rotation information can be placed in the VUI or SPS, or in an SEI message.  GPUs normally provide hardware support for rotation, and are frequently used for post-decoding processing, such as color conversion and image scaling, so the decoder complexity increase for inclusion of this tool is low.  This feature is especially interesting for mobile phone videoconferencing applications, where overall system complexity can be reduced by shifting the rotation operation from pre-encoding to post-decoding as part of the rendering process.  Additionally, coding efficiency can be improved for some sequences by rotating images prior to encoding.  Rotation can cause coding losses for some sequences, but losses can be avoided by an intelligent encoder. Individual sequence gains were as high as 3.4%, with average gains for the test set of 0.7% Intra LoCo and 0.5% RA LoCo with an intelligent encoder, and 0% Intra LoCo and 0.3% RA LoCo gain for a naïve encoder that always rotated by 90 degrees. When slices with a maximum size of 1500 bytes were used, average gains were 0.7% Intra LoCo and 0.7% RA LoCo with an intelligent encoder, and 0.3% Intra LoCo and 0.6% RA LoCo for a naïve encoder.

# Introduction and Proposal

This contribution addresses a well-understood issue related to the correct rendering of pictures.  Specifically, modern handheld capture devices such as cellphones or digital cameras often include orientation sensors that measure the orientation of the device relative to the axis of gravity.  With this information available, it is trivial to include in the video bitstream information about the orientation of the device’s sensor.  Image sensors generally capture rectangular, not square images.

Modern still image cameras include, in their file format (for example in extensions to the EXIF format), an indication of the camera orientation.  Accordingly, software packages that read the coded pictures can rotate the pictures when they are being viewed on PC screens or other electronic viewing devices.

Users may choose to hold a mobile phone camera in a portrait orientation or a landscape orientation, hence capturing video that may be rotated. Many handheld devices contain orientation sensors, and can identify the need for rotation.  For proper viewing, the video should be inversely rotated accordingly before presentation to a viewer.  An end-to-end system could choose where the rotation function should be performed.  Including orientation signaling in the bitstream enables a system to select whether the rotation should be performed at the capture end, prior to encoding, or at the display end, after the decoding.

Graphical Processing Units (GPUs) have become very common in computers and mobile phones.  GPUs typically provide rotation operations that can be performed very efficiently.  GPUs are frequently used to render video after it has been decoded, and to perform color conversion and image scaling functions.  The figure below illustrates example end-to-end systems where the rotation function has been moved from the encoder-end to the decoder-end, where the rotation can be efficiently performed in the GPU, along with other functions.

If addition of the decoder rotation requirement is considered to be too computationally complex for some applications, this tool could be included in some profiles and excluded from others.

Orientation information can be sent in the VUI, in the SPS, or in an SEI message.  The authors consider VUI to be the most appropriate location, and propose the addition of an orientation_idc syntax element, to allow for 4 rotation positions – 0, 90, 180, and 270 degrees.

| vui_parameters ( ) { | Descriptor |
| --- | --- |
| … |  |
| orientation_idc | u(2) |
| … |  |

orientation_idc specifies if rotation be performed following video decoding.  orientation_idc equal to 0 specifies no rotation.  orientation_idc * 90 degrees specifies the clockwise rotation of the image following decoding, as detailed in subclause X.  orientation_idc shall be in the range of 0 to 3, inclusive.

# Experimental Results

While the primary motivation for this contribution is to allow for overall end-to-end system complexity allocation and reduction for certain applications, it has also been observed that coding efficiency savings can be achieved for some sequences when rotation is used.

Table 1 shows the results for a naïve encoder that always rotates the sequence 90 degrees, using the conditions specified in JCT-D600.  The HM2.0 software was not modified, but the images sequences were rotated prior to being encoded.  The sequences were rotated 90 degrees counter-clockwise (or 270 degrees clockwise) before encoding, so that 90 degree clockwise rotation would be required following decoding. Individual sequence coding gains as high as 3.4% were achieved (for the LoCo-RA Traffic sequence).  Many sequences showed losses.  The overall average gain compared to the HM2.0 anchors was 0% for IntraLoCo, 0.3% for RA LoCo, and 0% for HE Intra and 0.2% for HE RA.

A more complex encoder could avoid losses by not rotating the image sequence when it is disadvantageous to do so.  Also in Table 1, on the right side, an intelligent encoder was assumed, and the sequences with Y BD-rate losses were replaced with 0% loss in all color components.  The overall average gain increased to 0.7% for Intra LoCo, 0.4% for RA LoCo, 0.5% for HE Intra and 0.3% for HE RA.

Experiments were also run for sequences containing slices for the Low Complexity modes, using the HM-2.0-dev-slices code revision 611, and operating conditions described in CE4, using 1500 byte slices.  The rotated image sequences were encoded using slices, and compared to an anchor without rotation using slices.  Table 2 shows the naïve encoder results.  When slices were used, the sequences with relatively large losses in Table 1 typically had lower losses or even gains with rotation.  The overall average gain was 0/3 for Intra LoCo and 0.6% for RA LoCo.  On the right of Table 2, an intelligent encoder replaced losses by 0%, and the overall average gain increased to 0.7% for IntraLoCo and 0.7% for RA LoCo.

Table 1. Rotation

Table 2. Rotation with 1500 byte slices

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).