| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E284 |
| --- | --- |

| Title: | An Adaptive Interpolation Filtering Technique |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Nader Mahdi Hsan Guermazi Mohammed Ali Ben Ayed | Tel: Email: | faouzi@ebriskvideo.com nader@ebriskvideo.com hsan@ebriskvideo.com medali@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

In this contribution, an Adaptive Interpolation Filtering (AIF) technique is proposed for interpolation filtering of the luminance samples of video sequences. In the proposed technique, for each video picture, the encoder first generates two new 8-tap filters for each reference picture list. Then, the encoder selects, for each sub-sample position, either one/two HM 2.0 [1] predefined 8-tap filters or the corresponding newly-generated 8-tap filters. Finally, the encoder transmits up to four 8-tap filters per video picture. Compared to HM2.0, the proposed technique yields average BD-rate reductions of 1.6% for LD LoCo, 0.2% for LD, 0.9% for RA LoCo and 0.03% for RA, while still maintaining the same decoding times. The average encoding time, however, increases by approximately 25%.

# Introduction

This contribution describes an Adaptive Interpolation Filtering (AIF) technique for interpolation filtering during the motion compensation part of video encoding/decoding. For each video picture, the encoder first generates two 8-tap filters for each reference picture list: two (¼, ½) 8-tap filters for reference picture List0 and two (¼, ½) 8-tap filters for reference picture List1. Only the most referenced picture from the considered reference picture list is used for the generation of the two 8-tap filters and then the selection procedure (i.e., the encoder will use only the HM 2.0 predefined filter(s) when a motion vector is pointing to one of the other pictures in the list). Moreover, in the main motion estimation and compensation stages, and for each reference picture list, the newly-generated filters are used only when the new reference picture has the same QP value as that of the reference picture used in the generation and selection stages. In other words, one QP index for each reference picture list must be transmitted to the decoder.

# Proposed AIF Technique: Description

In this section, we present some details on the proposed AIF technique, including the generation of the AIF 8-tap filters, the selection procedure, and the transmission (if needed) of some or all of the newly-generated AIF filters.

## Generation of Filter Coefficients

There are many ways of generating, for each video picture, the coefficients of the four AIF 8-tap filters. The encoder is free to employ any method, as long as the newly-generated filters have the same structure shown in Table 2.

In this contribution, we employ the filter-coefficients estimation method used in [2]. This method consists of the following steps:

Step 1: The displacement vectors are first estimated using the HM2.0 8-tap filters.

Step 2: Estimate, for each reference picture list (List0 and List1), the most referenced picture by the current picture.

Step3: For each reference picture list, the values of the filter coefficients are estimated (through linear regression) by minimizing the energy of the prediction errors after performing motion-compensated prediction, for the most referenced picture, using the displacement vectors from Step 1.

## AIF Filter Selection

The proposed technique involves a selection, for each sub-position that belongs to one of the four groups shown in Figure 1 (i.e., Group_0, Group_1, Group_2 or Group_3), either one/two HM 2.0 predefined 8-tap filters or the corresponding newly-generated 8-tap filters. For example, for each sub-position that belongs to Group_3, the proposed technique involves the selection of the two HM 2.0 predefined 8-tap filters or the corresponding two newly-generated 8-tap filters. The HM 2.0 predefined filters are shown in Table 1. The corresponding newly-generated 8-tap filters are shown in Table 2.

The selection is based on the mean square error between the source sample values and the filtered sample values (using each set of 8-tap filters). The filter(s) with the lowest MSE is/are selected.

Figure 1: The four groups of sub-sample positions

Table 1: Predefined (HM2.0) 8-tap filters (6-bit)

| ¼ | {-1, 4, -10, 57, 19, -7, 3, -1 } |
| --- | --- |
| ½ | {-1, 4, -11, 40, 40, -11, 4, -1 } |

Table 2: Newly-generated 8-tap filters (6-bit)

| ¼ | {a0, a1, a2, a3, a4, a5, a6, a7} |
| --- | --- |
| ½ | {b0, b1, b2, b3, b3, b2, b1, b0} |

## Transmission of Filter Coefficients

In order to limit the precision of the intermediate data to 16 bits, the newly-generated filter coefficients are first clipped using the clip values shown in Table 3. Then, the clipped coefficients of each newly-generated 8-tap AIF filter are encoded using 6-bit quantization of the differences between the values of the newly-generated filter coefficients and the corresponding values of the HM2.0 filter coefficients.

Table 3: Clip values of the AIF filter coefficients

| Coefficient index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clip value | 7 | 15 | 31 | 63 | 63 | 31 | 15 | 7 |

## Syntax description

As described above, our objective is to allow the use of newly-generated 8-tap filters for the interpolation in the motion estimation/compensation process. Therefore, the syntax in the slice header is also changed as follows to support the proposed AIF technique. The use_generated_filter_flag is inserted in the slice header to indicate whether one/two/three/four newly-generated 8-tap filter(s) is/are used.

In the case where use_generated_filter_flag is equal to 1, for each reference picture list, use_generated_filter_list is added in the slice header to indicate whether one/two newly-generated 8-tap filter(s) is/are used for the considered list.

When the use_generated_filter_list flag of the considered list is equal to 1, the used_ref_qp_idx of the used reference and four additional flags should be read. The used_ref_qp_idx is sent to indicate to which reference within the reference picture list the newly-generated  8-tap filter(s) is/are to be applied. The four flags (use_generated_filter_group0_flag, use_generated_filter_group1_flag, use_generated_filter_group2_flag, and use_generated_filter_group3_flag) are sent to specify whether one/two newly-generated filter(s) is/are used for each of the considered groups. When one of the flags is equal to 1, it will be followed by coefficients of the newly-generated 8-tap filter(s) and the positions where such filter(s) should be applied. Note that the coefficients of the filter(s) that could be used for different sub-sample position groups are sent only once. For example, in the case where use_generated_filter_group0_flag is equal to 1 and use_generated_filter_group2_flag is also equal to 1, there will be no additional coefficients to be read for Group_2. In fact, in this case, the decoder jumps directly to read the positions where the newly-generated filter(s) will be applied since the required coefficient values are identical to those read for Group_0.

When the use_generated_filter_list flag of the considered list is equal to 2, which means the newly-generated 8-tap filter(s) has/have been selected for all sub-sample positions, only the used_reference_idx and the ¼ and ½ coefficients of the newly-generated 8-tap filter(s) are present in the bitstream.

Table 4: Syntax definition for the proposed technique

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| use_generated_filter_flag | 2 | u(1) |
| if (use_generated_filter_flag) { |  |  |
| for(list = 0; list < 2 ; list ++) { |  |  |
| use_generated_filter_list[list] | 2 | u(2) |
| if(use_generated_filter_list[list] == 1){ |  |  |
| used_ref_qp_idx[list] | 2 | u(2) |
| use_generated_filter_group0[list] | 2 | u(1) |
| if (use_generated_filter_group0[list]) { |  |  |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef_q[list][i] | 2 | se(v) |
| } |  |  |
| for (sub_pos =0; sub_pos < num_pos_in_group0; sub_pos ++ ) { |  |  |
| filter_index_group0[list][sub_pos] | 2 | u(1) |
| } |  |  |
| } |  |  |
| use_generated_filter_group1[list] | 2 | u(1) |
| if (use_generated_filter_group1[list]) { |  |  |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef_h[list][i] | 2 | se(v) |
| } |  |  |
| for (sub_pos =0; sub_pos < num_pos_in_group1; sub_pos ++ ) { |  |  |
| filter_index_group1[list][sub_pos] | 2 | u(1) |
| } |  |  |
| } |  |  |
| use_generated_filter_group2[list] | 2 | u(1) |
| if (use_generated_filter_group2[list]) { |  |  |
| if (! use_generated_filter_group0[list]) { |  |  |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef_q[list][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| for (sub_pos =0; sub_pos < num_pos_in_group2; sub_pos ++ ) { |  |  |
| filter_index_group2[list][sub_pos] | 2 | u(1) |
| } |  |  |
| use_generated_filter_group3[list] | 2 | u(1) |
| if (use_generated_filter_group3[list]) { |  |  |
| if (! use_generated_filter_group0[list]) { |  |  |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef_q[list][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| if (! use_generated_filter_group1[list]) { |  |  |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef_h[list][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| for (sub_pos =0; sub_pos < num_pos_in_group3; sub_pos ++ ) { |  |  |
| filter_index_group3[list][sub_pos] | 2 | u(1) |
| } |  |  |
| }else if(use_generated_filter_list[list] == 2){ |  |  |
| used_ref_qp_idx [list] | 2 | u(2) |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef_q[list][i] | 2 | se(v) |
| } |  |  |
| for (i =0; i < filter_size; i ++ ) { |  |  |
| diff_filter_coef_h[list][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| --- |  |  |
| } |  |  |

# Experimental Results

The simulation experiments were conducted following the testing conditions in JCTVC-D600 [3].

In the tested AIF method, a motion estimation stage is first used to generate four AIF filters. Then, for each sub-sample position and each reference picture list, a selection is performed that is based on the mean square error between the source sample values and the filtered sample values (using each set of 8-tap filters). The filter(s) with the lowest MSE is/are selected. In the RA configurations, the AIF filter(s) are never applied to non-reference pictures. In the LD configurations, the AIF filter(s) are also not applied to the highest-QP pictures (as illustrated in the example of Figure 2).

Figure 2: An example of an AIF assignment

The proposed AIF technique was compared to the HM2.0 technique. The coding efficiency (BD-Rate) numbers were obtained for the Low Delay (Low Complexity & High Efficiency) and Random Access (Low Complexity & High Efficiency) configurations (Table 5). The detailed BD-Rate results for the Luminance pictures are also shown in Table 6.

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @2.50 GHz.

- RAM: 4 Go

- OS:  Windows XP Professional.

It is clear, from the tables, that the proposed AIF method achieves coding efficiency gains over the HM2.0 fixed filtering technique, especially for the LC configurations. It is also important to note that the proposed AIF technique performs very well (by an average of more than 3%) for the new Test video sequences (i.e., Nebuta and SteamLocomotive).

The required additional complexity at the encoder and/or decoder can be quite small. Except for the decoding of at most two distinct AIF filters for each reference picture list (the ¼ and ¾ AIF filters employ the same coefficients, though in opposite order) for each video picture/slice, there is no additional decoder complexity. Moreover, except for the encoding of at most four distinct AIF filters for each video picture/slice, the added complexity at the encoder side depends on the application at hand. For example, the new technique can be used to send two special AIF filters (for each reference picture list) for each slice, picture, group of pictures or even for a set of video sequences that have similar characteristics (e.g., in the case of a special application space).

As shown in Table 5, even when we generate such filters on-line, the average additional encoding complexity needed for the generation, selection and application procedures is only 17% for Low Delay, 35% for Low Delay LoCo, 15% for Random Access and 30% for Random Access Loco.

Table 5: Simulation Results

|  | Random Access |  |  | Random Acces LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -1.0 | -1.2 | -2.7 | -1.9 | -2.0 |
| Class B | -0.1 | -0.1 | 0.0 | -0.9 | -0.2 | -0.4 |
| Class C | 0.1 | 0.0 | 0.0 | -0.2 | -0.1 | 0.0 |
| Class D | 0.6 | 0.3 | 0.4 | 0.4 | 0.3 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | -0.03 | -0.2 | -0.2 | -0.9 | -0.5 | -0.5 |
| Enc Time[%] | 120 % |  |  | 134 % |  |  |
| Dec Time[%] | 100 % |  |  | 100 % |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.0 | 0.1 | -2.1 | 0.4 | 0.6 |
| Class C | -0.3 | 0.1 | 0.0 | -1.1 | -0.3 | -0.3 |
| Class D | -0.1 | 0.2 | 0.0 | -0.4 | 0.1 | 0.1 |
| Class E | -0.1 | 0.3 | 0.6 | -2.7 | 0.3 | 0.8 |
| All | -0.2 | 0.1 | 0.1 | -1.6 | 0.1 | 0.3 |
| Enc Time[%] | 119 % |  |  | 130 % |  |  |
| Dec Time[%] | 100 % |  |  | 100 % |  |  |

Table 6: Detailed Luminance BD-Rate Results

|  | Sequences | Random Access | Random Access LoCo | LowDelay | LowDelay LoCo |
| --- | --- | --- | --- | --- | --- |
| S01 | Traffic | 0.1 | -1.4 |  |  |
| S02 | PeopleOnStreet | 0.1 | -0.9 |  |  |
| S0X | Nebuta | -2.2 | -4.6 |  |  |
| S0Y | SteamLocomotive | -0.8 | -4.0 |  |  |
| S03 | Kimono | -2.2 | -0.7 | -0.1 | -3.3 |
| S04 | ParkScene | -0.8 | 0.1 | -0.1 | -1.4 |
| S05 | Cactus | 0.1 | -0.5 | -0.1 | -1.4 |
| S06 | BasketballDrive | -2.2 | -0.9 | -0.5 | -1.4 |
| S07 | BQTerrace | -0.8 | -2.4 | -0.7 | -3.1 |
| S08 | BasketballDrill | 0.0 | 0.1 | -0.4 | -1.0 |
| S09 | BQMall | 0.1 | -0.1 | -0.3 | -1.3 |
| S10 | PartyScene | 0.3 | 0.4 | -0.2 | -0.5 |
| S11 | RaceHorses | -0.2 | -1.3 | -0.1 | -1.6 |
| S12 | BasketballPass | 0.3 | 0.2 | -0.4 | -1.2 |
| S13 | BQSquare | 0.8 | 0.6 | 0.0 | 0.0 |
| S14 | BlowingBubbles | 0.6 | 0.8 | -0.1 | 0.0 |
| S15 | RaceHorses | 0.7 | 0.1 | 0.0 | -0.6 |
| S16 | Vidyo1 |  |  | -0.7 | -1.4 |
| S17 | Vidyo3 |  |  | 0.2 | -4.4 |
| S18 | Vidyo4 |  |  | 0.2 | -2.2 |
| Average | Class A | -0.7 | -2.7 |  |  |
|  | Class B | -0.1 | -0.9 | -0.3 | -2.1 |
|  | Class C | 0.1 | -0.2 | -0.3 | -1.1 |
|  | Class D | 0.6 | 0.4 | -0.1 | -0.4 |
|  | Class E |  |  | -0.1 | -2.7 |
|  | All | -0.03 | -0.9 | -0.2 | -1.6 |

# Conclusion and recommendation

The proposed technique provides an average of 0.7% in BD-Rate reduction when compared to the HM 2.0 fixed filtering technique, while also requiring essentially the same decoding complexity. The additional encoder complexity can vary greatly, depending on the video application. In some cases, the new AIF filter(s) can be decided off-line, and the encoder will only need to encode and transmit at most two filters for each reference picture list. In other cases, the encoder will generate the coefficients as part of a pre-analysis phase. Even when we generate such filters on-line, the average additional encoding complexity needed for the generation, selection and application procedures is only ~25%.

We recommend that the JCT-VC adopt the proposed AIF technique into the HM.

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-2.0/

- S. Wittmann, T. Wedi, “Separable Adaptive Interpolation Filter,” ITU-T SG16/Q.6 Doc. T05-SG16-C-0219, Geneva, Switzerland, June 2007.

- F. Bossen, “Common TM Test Conditions,” JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).