| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E287 |
| --- | --- |

| Title: | Chroma ALF with reduced vertical filter size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | madhukar@ti.com sze@ti.com zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution analyzes implementation complexity of Chroma Adaptive loop filter (ALF) at decoder. Implementation complexity is determined by not just computations but also memory bandwidth/memory size (area). This contribution presents Chroma ALF filters that reduce vertical size of filter thereby reducing the line buffer/memory bandwidth requirements. Three Chroma ALF filters are proposed: 9x3, 7x3, and 5x3 rectangles. The proposed filters have a maximum vertical size of 3 and are reported to reduce memory bandwidth/memory size requirements by 50% when compared to HM 2.0 chroma ALF filter.  The worst case multiplications for HM 2.0 chroma (5x5), 5x3, 7x3, and 9x3 filters are reported to be: 13, 8, 11, 14 respectively. Among the three filters proposed in this contribution, filter 9x3 is asserted to provide the best trade-off in terms of complexity and coding performance. Filter 9x3 is reported to provide U-BD-Rate of: 0.1% HE_Intra, 0.3% HE_RA, -0.1% HE_LD, V-BD-Rate of: 0.1% HE_Intra, 0.4% HE_RA, -0.1% HE_LD, Y-BD-Rate of: 0.0% HE_Intra, 0.0% HE_RA, -0.1% HE_LD.

# Introduction

HM 2.0 ALF uses 5x5 square filter for chroma filtering as shown in Figure 1. The 5x5 filter is symmetric and requires 13 multiplications instead of 25. The same chroma filter is applied to both chroma planes. A two-bit chroma_idc field is used to signal which of the chroma plane(s) are filtered. Chroma ALF does not use CU adaptive filtering, ALF is applied on whole frame or not applied at all

5x5-Square

Figure 1: HM 2.0 Chroma ALF filter set.

ALF decode filtering can be carried out in either frame-based or LCU-based fashion. In frame-based ALF filtering, the deblocking filter output is first stored in a frame buffer. The deblocked filter output is then read to carry out ALF filtering. Figure 2 shows an example frame-based ALF filtering with 5x5 filter. The red-lines in Figure 2 are pixel lines. The deblock frame buffer needs to be read 25 times which is not feasible since the memory bandwidth requirements go up to 25 frame buffer reads. Clearly a caching mechanism is needed to reduce memory bandwidth requirements.

Figure 2: Frame-based ALF decode.

Filtering at (4, x)

Filtering at (5, x+1)

1

2

3

4

5

6

7

1

2

3

4

5

6

7

Figure 3: Frame-based ALF decode filtering with line buffer cache.

Figure 3 shows an example of frame-based ALF decode filtering with line buffer cache. The figure on left shows filtering of line 4. The figure on right shows filtering of line 5. From the Figure 3, one can see that Lines 3, 4, 5, 6 get used in filtering of both lines 4 and 5. Data on these lines can be cached in line buffers to avoid reading them again from deblock frame output buffer. With the line buffer caching mechanism shown in Figure 3, the deblock frame output buffer needs to be read only once (also written once). However, this is still expensive. Memory bandwidth requirements can be further reduced by using LCU-based filtering instead as shown in Figure 4.

Input

Output

Input

Lines that need to be cached

Output

Figure 4: LCU-based ALF decode filtering with line buffer cache.

In LCU-based filtering, ALF filtering is carried out on an LCU basis as and when the deblock filter output gets generated. The left part of Figure 4 shows ALF filtering of LCU(0,0) and the right part shows ALF filtering of LCU(1,0). The red lines show the deblock filter pixel lines (which is input to ALF filters) and the blue lines are the ALF filtered output. Since the ALF filter is a non-causal filter and uses right and bottom LCU data (which are not yet available), not all pixels in a LCU can be ALF filtered. The ALF filtered pixels are shown in solid blue line and the LCU pixels that cannot be ALF filtered when the LCU is generated are shown in dotted blue lines.

From Figure 4 it should be noted that even for LCU-based filtering, the previous 4 lines need to be cached in line buffers to avoid additional memory bandwidth as shown in Figure 4.

Assuming a 4Kx2K image and IBDI on, the line buffer memory size for Chroma is: 2*(4K/2) * 4 lines *12bits = 16 Kbytes which is a significant amount for single chip video codecs.

# ALF filters with reduced vertical size

For a given image size, the vertical size of ALF filters determines the size of line buffer/memory bandwidth requirements. These requirements can be reduced by reducing the vertical size of ALF filter. This contribution presents three ALF filters that reduce vertical size of chroma ALF filter. Figure 5 shows these filters. All three filters have a maximum vertical size of 3. Since these filters operate on only 3 lines instead of the original 5, the size of the line buffers goes down from 4 lines to 2 lines – a 50% reduction in memory bandwidth/memory size requirements.

9x3

7x3

5x3

Figure 5: ALF filters with reduced vertical size.

# Simulation results

Chroma ALF of HM 2.0 was modified to support rectangular size. Table below provides the BD-Rate performance of existing HM 2.0 ALF chroma filter, 9x3, 7x3 and 5x3 filter. It can be seen from tables below that the proposed chroma ALF filter sets capture most of the ALF coding gains while reducing line buffer size/memory bandwidth by 50%.

# Conclusions

This contribution analyzes implementation complexity of Chroma Adaptive loop filter (ALF) at decoder. Implementation complexity is determined by not just computations but also memory bandwidth/memory size (area). This contribution presents Chroma ALF filters that reduce vertical size of filter thereby reducing the line buffer/memory bandwidth requirements. Three Chroma ALF filters are proposed: 9x3, 7x3, and 5x3 rectangles. The proposed filters have a maximum vertical size of 3 and are reported to reduce memory bandwidth/memory size requirements by 50% when compared to HM 2.0 chroma ALF filter.  The worst case multiplications for HM 2.0 chroma (5x5), 5x3, 7x3, and 9x3 filters are reported to be: 13, 8, 11, 14 respectively. Among the three filters proposed in this contribution, filter 9x3 is asserted to provide the best trade-off in terms of complexity and coding performance. Filter 9x3 is reported to provide average BD-Rate in the range of U-BD-Rate: -0.1%-0.3%, V-BD-Rate: -0.1%-0.4%, Y-BD-Rate: -0.1%-0.0%, when compared to anchor. The proposed 9x3 chroma ALF filter sets capture most of the chroma ALF coding gains. Hence we recommend that the 9x3 filter be adopted in HM 3.0.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).