| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E288 WG11 Number: m19815 |
| --- | --- |

| Title: | Loop filtering with directional features |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents a block-based filter adaptation scheme with directional features (FAD). Blocks of 4x4 sizes are classified using four directional gradients. All 16 pixels within each 4x4 block are classified into the same class. For each class, the filter design follows the same procedure as the ALF in HM 2.0. The proposed filtering method will have up to 4 filters as compared to up to 16 as ALF in HM 2.0.

# Introduction

In the 4th JCT-VC meeting held at Daegu, Korea, a loop filtering scheme with directional similarity mapping was proposed in JCTVC-D221 [1]. However, it has been reported that the complexity associated with such design is very high [2]. This is due to the filter design which combines linear spatial filter part and non-linear directional similarity filter part: First of all, this type of combined filters utilizes the surrounding pixels twice, resulting in more coefficients thus higher computation cost. Secondly, the non-linear mapping operations in the directional filter part introduce additional complexity [3].

In this contribution, directional features will be used to classify block of pixels, similar to the procedure described in [1] and [3]. However the filter design remains the same as ALF in HM 2.0. In other words, only the method for filter adaptation is changed. Other components of ALF are not modified.

# Filter adaptation based on directions of local gradients

In JCTVC-D221 [1], pixels X(i,j) in a video frame are classified into four classes based on directional local gradients, each class adapts to a filter. In this contribution, 4x4 block-based filter adaptation is proposed: For each 4x4 block B(i,j), the pixel X(4i+1,4j+1) within is used to compute the four directional local gradients using [1, -1] weights as follows (note weight [-1, 2, -1] as in D221 can also be used):

gradhi,j=k=−LLl=−LLX4i+1+k,4j+1+l−X(4i+1+k,4j+1+l+1)gradvi,j=k=−LLl=−LLX4i+1+k,4j+1+l−X(4i+1+k+1,4j+1+l)graddi,j=k=−LLl=−LLX4i+1+k,4j+1+l−X(4i+1+k+1,4j+1+l+1)gradui,j=k=−LLl=−LLX4i+1+k,4j+1+l−X(4i+1+k+1,4j+1+l−1)

As can be seem, within the neighborhood of ±L pixels, grad_h measures the horizontal gradient, grad_v measures the vertical gradient, grad_d measures the gradient of 45 degree down, and grad_u measures the gradient of 45 degree up. Then all the pixel in block B(i,j) will be assigned as directional class D(i,j) according to the direction with minimum local gradient: min_g(i,j) = min{ grad_h(i,j), grad_v (i,j), grad_d(i,j), grad_u (i,j) }:

D(i,j) = 0 	if	 min_g(i,j) = grad_h(i,j)

D(i,j) = 1 	if	 min_g(i,j) = grad_v(i,j)

D(i,j) = 2 	if 	min_g(i,j) = grad_d(i,j)

D(i,j) = 3 	if 	min_g(i,j) = grad_u(i,j)

This will classify blocks into classes with similar gradient directions. In this contribution, L=1 is used. Thus for each direction, 9 gradient computations are required. All pixels within a 4x4 block are classified into the same class (i.e. block-based filter adaptation).

After classification, a Wiener filter will be designed for all blocks in each class, which is the same filter estimation process as in HM 2.0. Furthermore, CU-synchronous control map, adaptive filter tap decision, and filter coefficient coding methods, are all as in HM 2.0 ALF.

# Experimental results

Following the conditions in CE8 [4], results of the proposed method is as follows, where the gradients are computed using [1, -1] weights:

|  | Filter adaptation with directional features |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Up to 4 filters |  |  |  |  |  |  |  |  |  |  |  |
|  | IO | RA | LD |  |  |  |  |  |  |  |  |  |
| Class A | 0.6 | 0.8 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.5 | 0.7 | 0.3 |  |  |  |  |  |  |  |  |  |
| Class C | 0.3 | 0.1 | 0.0 |  |  |  |  |  |  |  |  |  |
| Class D | 0.2 | -0.1 | -0.2 |  |  |  |  |  |  |  |  |  |
| Class E | 0.6 |  | 0.2 |  |  |  |  |  |  |  |  |  |
| BD-rate | 0.4 | 0.4 | 0.1 |  |  |  |  |  |  |  |  |  |
| Enc. Time | 91% | 98% | 105% |  |  |  |  |  |  |  |  |  |
| Dec. time | 94% | 90% | 100% |  |  |  |  |  |  |  |  |  |

# References

- “Loop filter with directional similarity mapping (DSM)”, JCTVC-D221, Daegu, Jan. 2011.

- “CE8: Non-deblocking Loop Filtering - Summary Report”, JCTVC-E028, Geneva, Mar. 2011.

- “CE8 Subtest 3: Loop filter with directional similarity mapping”, JCTVC-E108, Geneva, Mar. 2011.

- “Description of CE8: Non-deblocking loop filtering”, JCTVC-D608, Daegu, Jan. 2011.

Patent rights declaration(s)

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).