| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E289 |
| --- | --- |

| Title: | Simplified Planar Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin, Lingzhi Liu, Changcai Lai, Jianhua Zheng | Tel: Email: | +86-10-82836279 yblin@huawei.com lzliu@huawei.com |
| Source: | HiSilicon Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document presents two simplified planar Intra prediction methods with lower calculation complexity. One is diagonal-based Intra prediction and other is sub-block based Intra prediction. Both of them can be easily implemented by only using addition and shift operations. Simulation results show the proposed methods reduce encoding time with similar performance as the existing planar Intra prediction.

# Introduction

Planar Intra prediction (PIP) has been investigated in CE6.e since the last JCTVC meeting [1]. As shown in Figure 1, PIP mode firstly predicts the right-bottom corner sample, then the samples at both right border and bottom border. Finally, bi-linear interpolation is performed to generate the prediction values of the remained inner samples with the reference of the prediction values in the previous two steps. PIP is suitable to encode smooth texture. It has been also reported that PIP improves objective and subjective performance.

Fig.1. Planar Intra prediction

This document aims to reduce the calculation complexity issue of planar Intra prediction and proposes two simplified planar prediction methods, i.e., diagonal-based Intra prediction and sub-block based Intra prediction.

# Simplified planar Intra prediction

## Diagonal-based Intra prediction

a) Prediction for the samples at diagonal line        b) Prediction for other samples

Fig. 2. Diagonal-based Intra prediction

Figure 2 shows the prediction process of diagonal-based Intra prediction. The samples at diagonal line of current block are firstly predicted by using its left and above reference samples. For example, as shown in Figure 2a, the prediction value of sample P is calculated as follows.

P’ = (LeftRef + AboveRef +1) >>1					(1)

Where, P’denotes the prediction value of sample P, LeftRef and AboveRef denote the corresponding samples at left and above reference arrays. For all the remained samples except those at diagonal line, e.g., the samples of p1 and p2, can be linearly interpolated based on the prediction values of the diagonal sample and its corresponding above/left reference sample. For example, as shown in Figure 2b, the prediction value of sample p1 can be calculated by using equation (2) through a linear interpolation.

p1’ = (AboveRef*d2 + P’*d1)/(d1+d2)			  	(2)

Where, p1’ is the prediction value of sample p1, d1 and d2 denote the distances from p1 to AboveRef and the sample P, respectively. A look-up table may be required to remove the division operation in the equation (2).

Diagonal-based Intra prediction has lower calculation complexity than the existing planar Intra prediction, since diagonal-based Intra prediction only employs linear interpolation rather than bilinear interpolation which is used in the existing planar Intra prediction.

## Sub-block based Intra prediction

Fig. 3. 2x2-based Intra prediction

A block can be divided into a series of 2x2 or 4x4 sub-blocks, and each sub-block is predicted with reference of the prediction value of its neighbor sub-blocks. The processing order for the divided sub-blocks can be from left to right and from top to bottom.

As shown in figure 3, a block of 16x16 is divided into 64 sub-blocks of 2x2. Each 2x2 sub-block is predicted as follows.

P3 = ( b + d +1) >>1 							(3)

P1 = ( P3 + b +1) >>1;  P2 = ( P3 + d +1) >>1; 			(4)

P0 = ( a + c + P1 + P2 +2) >>2; 					(5)

Where Pn (n=0,1,2,3) is the prediction value of the nth sample in current 2x2 sub-block, and a/b/c/d denotes the value of the neighbor reference sample of current 2x2 sub-block. The prediction value of current 2x2 sub-block depends on the prediction value of its neighbor left and above 2x2 sub-block. In this way, all sub-block can be predicted, and the residual of current block is also formed.

It is noticed that 2x2 based Intra prediction can be implemented with a simpler way than the existing planar Intra prediction. It only requires 2.5 additions and 1 shift operations per sample. As for the respect of dependency between different 2x2 sub-blocks, it is also possible for the different rows of 2x2 sub-blocks to be processed with pipelining.

# Test conditions and Results

The proposed Intra prediction methods have been integrated into the reference software provided by CE6.e. HM2.0 is used as anchor for the simulation.

The following running environment and compiler have been used for running simulations.

| Running environment | Compiler |
| --- | --- |
| Intel Xeon X7560 @2.27GHz, 256G RAM Windows Server 2003, 64 bits | VS2005 |

We have run both all Intra HE and LC cases using the same computer to make sure coherent encoding and decoding time. Common testing conditions are used for simulation [2][3]. The following tables summarize the simulation results. The detail of test results can be found from attachments.

Table 1. Performance of the existing Planar Intra prediction (JCTVC-D235)

Table 2. Performance of Diagonal-based Intra prediction

Table 3. Performance of 2x2-based Intra prediction

Table 1 shows the performance of the existing planar Intra prediction as described in JCTVC-D235, which achieves 0.7% bit-rate reduction over HM2.0 for both all Intra HE and LC configurations. Table 2 and Table 3 show the performance of the proposed diagonal and 2x2 based Intra prediction methods, respectively. It can be observed that the proposed prediction methods achieve the reduced encoding time and similar performance as the existing planar prediction. The reduction of encoding time can be explained by the simplification on calculation complexity.

Table 4. Performance of 4x4-based Intra Prediction

Table 4 further shows the performance of the 4x4 based Intra prediction methods, where sub-block size is 4x4 instead of 2x2, and each 4x4 sub-block employs the existing planar Intra prediction. It is observed that the proposed 4x4 based Intra prediction reduces encoding time while keeps almost the same performance as the existing planar Intra prediction.

# Conclusion

This document presents two simplified planar Intra prediction methods. Simulation results show that the proposed methods have lower calculation complexity and similar performance compared with the existing planar Intra prediction method.

# References

- Sandeep Kanumuri, TK Tan, Frank Bossen, "Enhancements to Intra Coding", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D235, 4th Meeting, Daegu, KR, Jan., 2011.

- Ali Tabatabai, Keiichi Chono, Muhammed Coban, etc. "Core Experiment 6: Intra Prediction Improvement", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D606, 4th Meeting, Daegu, KR, Jan., 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, 4th Meeting, Daegu, KR, Jan., 2011.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd and HiSilicon Technologies Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).