| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC- E291 |
| --- | --- |

| Title: | Cross-check Report for I2R’s Proposal JCTVC-E104 by MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Authors: | Zhi Zhou Shan Liu 2860 Junction Ave. San Jose, CA95134, USA | Emails: | zhi.zhou@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek USA |  |  |

_____________________________

# Abstract

The purpose of this document is to cross-check the I2R’s proposal JCTVC-E104 “RQT depth selection” [1]. The verification task has been done successfully and the BD-rates match those provided by I2R exactly.

# Introduction

I2R provided the HM2.0-based source code of their proposed algorithm.

Both QuadtreeTUMaxDepthInter and QuadtreeTUMaxDepthIntra are set to 2 in the six encoder configuration files.

All six configurations were tested for the provided code.

# Test Results

The BD-rates from our experimental results match those provided by I2R exactly. The detailed results are listed in JCTVC-E291.xls.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | -0.2 | 1.0 | 1.0 |
| Class B | 0.1 | -0.1 | -0.1 | -0.5 | 0.2 | 0.3 |
| Class C | 0.1 | -0.3 | -0.3 | -0.3 | -0.4 | -0.2 |
| Class D | 0.1 | -0.4 | -0.4 | -0.1 | -0.3 | -0.4 |
| Class E | 0.0 | 0.0 | 0.0 | -0.7 | 2.0 | 1.5 |
| All | 0.1 | -0.2 | -0.2 | -0.4 | 0.4 | 0.4 |
| Enc Time[%] | 81% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.4 | -0.3 | -0.3 | 1.5 | 1.4 |
| Class B | 0.3 | 0.2 | 0.0 | -0.3 | 0.8 | 0.6 |
| Class C | 0.2 | -0.1 | 0.1 | -0.2 | -0.2 | 0.1 |
| Class D | 0.3 | -0.2 | 0.0 | 0.0 | 0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.1 | -0.1 | -0.2 | 0.6 | 0.5 |
| Enc Time[%] | 90% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.9 | 0.9 | -0.2 | 0.3 | 0.5 |
| Class C | 0.2 | 0.4 | 0.4 | -0.2 | 0.1 | 0.0 |
| Class D | 0.2 | 0.5 | 0.0 | 0.0 | 0.4 | 0.0 |
| Class E | 0.0 | -0.2 | 1.3 | -0.2 | 1.4 | 0.8 |
| All | 0.2 | 0.5 | 0.6 | -0.2 | 0.5 | 0.3 |
| Enc Time[%] | 91% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

This report confirms the results provided by I2R in JCTVC-E104.

# References

- Y. H. Tan, C. Yeo, H. L. Tan, Z. Li, “RQT depth selection”, JCTVC-E104, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, January 2011.

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008.