| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E294 |
| --- | --- |

| Title: | CE1：Huawei report on DMVD in HM2.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | Sixin Lin Mingyuan Yang Haoping Yu Huawei Technologies Co., Ltd. Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86 755 28567191 linsx@huawei.com myyang@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

Spatial and Temporal Direct Mode (STDM) has been implemented into HM2.0 software and tested following the common test requirements. A memory constraint module is added into the STDM algorithm to address the serious concerns about memory access bandwidth that were brought up during the discussion in the last meeting. This contribution includes descriptions of coding algorithms and their implementation, coding performance, and complexity evaluation and analysis. According to the test results, an average bit-saving of 0.93% in RA-HE has been achieved with additional complexity, measured as encoding and decoding time, at about 1% and 5% more than that of the anchor, and in RA-LC case, an average bit-saving is about 0.7%, and the extra encoding time is 1% and the extra decoding time is about 6% comparing with the anchor.

# Algorithm Description

The basic Spatial and Temporal Direct Mode (STDM) algorithm is based on the JCTVC-D448, a joint proposal between Intel and Huawei. In response to the concerns about increased memory access traffic,  a memory constraint module is added into the STDM algorithm in this proposal to reduce memory access bandwidth.

After motion vector candidates have been set up as in JCTVC-D448, these candidates will be filtered based on their position, following the procedure described below.

- Based on the motion vector candidates, the center of these candidates will be calculated (shown as figure 1).

Fig. 1 Center calculation of motion vector candidates

- A window is set up around the center.

Fig. 2 Set up a window around the center

- The motion vector which falls outside the window is filtered out. Then a new set of motion vector candidates is set up.

- Using the new candidate set to derive the motion vector in encoder and decoder.

Using this memory constraint module, the memory access bandwidth can be reduced to a reasonable level. In current implementation, the size of memory which need to be accessed for current block is four times of the size of currently block.

- Software Implementation

STDM is applied to Skip mode) in B (NON-GPB) slices. So STDM mode only works for random access cases. Flag is needed to indicate whether STDM mode is used. For Skip mode only one bit is used to indicate whether STDM mode is used for all PUs.

CU-level rate-distortion (RD) evaluation is performed to decide whether STDM and TM-DMVD are chosen.

- Compression Performance and Complexity analysis

Table 1 and 2 shows the performance of STDM in skip mode compared to the performance of HM2.0 anchors. In high efficiency case, the average BD bitrate reduction for Luma component is about 0.9%  with about 1% increase in encoding time and about 5% increase in decoding time. In low complexity case, the average BD bitrate reduction for Luma component is about 0.7% with about 1% increase in encoding time and about 6% increase in decoding time.

Table 1. STDM in RA-HE

| Video Sequence | STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -1.0 | -1.3 | -1.5 |
| Class B | -0.8 | -0.9 | -0.9 |
| Class C | -0.9 | -1.1 | -1.1 |
| Class D | -1.1 | -1.1 | -1.0 |
| Total average | -0.9 | -1.1 | -1.1 |
| Encoding time | 101% | Decoding time | 105% |

Table 2. STDM in RA-LC

| Video Sequence | STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -0.8 | -1.0 | -0.9 |
| Class B | -0.7 | -0.8 | -0.7 |
| Class C | -0.6 | -0.7 | -0.7 |
| Class D | -0.7 | -0.8 | -0.8 |
| Total average | -0.7 | -0.8 | -0.8 |
| Encoding time | 101% | Decoding time | 106% |

- Reference

[1] Mingyuan Yang, Sixin Lin, Haoping Yu, “CE1：Huawei report on TMDMVD and STDM in HM”, JCTVC-D295, 4rd Meeting: Daegu, KR, 20-28 January, 2011;

# Patent rights declaration(s)

Huawei Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).