| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E296 |
| --- | --- |

| Title: | CE11: Adaptive coefficients scanning for inter-frame coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Song, Mingyuan Yang, Haitao Yang, Haoping Yu Huawei Technologies Co., Ltd. Huawei Building, 3 Xinxi Road, Haidian District,Beijing, China. | Tel: Email: | +86 10 82882583 andy.songjin@huawei.com yangmingyuan@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

In this document adaptive coefficients scanning with three scanning pattern is investigated and tested. In JCTVC-D311[1], the best scanning pattern for every Transform Unit (TU) is chosen based on the texture direction of reference block and no flag is sent to the decoder side. In this proposal a modified adaptive coefficients scanning is contributed. One flag is explicitly signaled to the decoder side to indicate whether the scanning pattern is zigzag or the pattern based on the texture direction. If the scanning pattern is not zigzag, the texture direction is detected after the parsing procedure to achieve the real residual block. Experimental results shows improved adaptive coefficients scanning provides 0.3% and 0.8 % BD-rate gain in high efficiency (HE) configurations respectively for random access, and low-delay test conditions.

# Adaptive Coefficients Scanning

Adaptive coefficient scanning was firstly introduced in D311 and consists of three scanning patterns: zigzag, horizontal and vertical patterns, where scanning pattern is chosen based on the texture direction of reference block.

The modified method puts the texture detection module into the “decompress” process, while this module was in the “parsing” process which would delay the parsing procedure and add the memory access function in the parsing procedure in D311. In the encoder side the texture direction of the reference block need to be detected firstly. Then the best scanning pattern is selected between zigzag and the pattern decided by texture direction based on the RD decision and explicitly signaled to decoder side. If the chosen pattern is vertical, the coefficients matrix will be transposed. Then the coding process goes on as usual. In the decoder side the flag of scanning pattern is parsed. If it isn’t zigzag scanning, horizontal scanning would be supposed until the parsing progress is finished. Then the texture direction of the reference block is detected. If the pattern based on the texture direction is vertical, the coefficients matrix would be transposed. After that the decoding process goes on as usual.

Moreover，in an improved method, luma scanning pattern of the same block is applied for chroma TU.

# Experimental results

The proposed algorithm is implemented on top of HM2.0 and the performance is evaluated according to CE11 description [2]. Both the original version and the modified version are compared to default scanning method scan in high efficiency configuration.

Following tables represent total average BD-rate saving for each testing configuration with encoder and decoder running time. The decoding time is tested in the same computer.

Tab.1 shows the results of original version.

Tab.1 original ACS

| BD-rate | Y | U | V | Enc | Dec |
| --- | --- | --- | --- | --- | --- |
| HE Random access | -0.3 | -0.1 | -0.1 | 102% | 103% |
| HE Low delay | -1.0 | -0.1 | -0.1 | 102% | 102% |

Tab.2 shows the results of modified version.

Tab.2 modified ACS

| BD-rate | Y | U | V | Enc | Dec |
| --- | --- | --- | --- | --- | --- |
| HE Random access | -0.3 | -0.1 | -0.1 | 106% | 101% |
| HE Low delay | -0.8 | -0.1 | -0.1 | 106% | 101% |

Tab.3 shows the results of improved version with chroma modification.

Tab.3 improved ACS

| BD-rate | Y | U | V | Enc | Dec |
| --- | --- | --- | --- | --- | --- |
| HE Random access | -0.3 | -0.4 | -0.4 | 107% | 97% |
| HE Low delay | -0.8 | -0.8 | -0.9 | 106% | 99% |

# Conclusion

Adaptive coefficient scanning method is proposed in this contribution according to the trade-off between complexity and performance. We recommend adopting this modified ACS as the consistent coefficient scanning method for high efficiency configuration in next version of HEVC Test Model.

# References

- J. Song, M. Yang, et al, “Adaptive coefficients scanning for inter-frame coding”, JCTVC-D311, JCT-VC Meeting, Daegu, Jan. 2010

- V. Sze, “CE11: Coefficient scanning and coding”, JCTVC-E031, JCT-VC Meeting, Geneva, March. 2010

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).