| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E298 |
| --- | --- |

| Title: | CE4 Subset1: Report on fine granularity slice partition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Qiu Shen Qingpeng Xie Haoping Yu Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | shenqiu@huawei.com alicxie@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution analyzes the necessary of supporting CU based slice partition, and reports the performance of the proposal which signals the slice start and end address in slice header.

# Introduction

Currently, HEVC [1] only supports slice boundaries on largest coding unit (LCU) which size is 64x64 in common condition [2]. We observed that the data sizes of LCUs are diversified for different video sequence, different bitrate and different coding mode. Take sequence RaceHorses_832x480_20.yuv at QP 22 for example, the LCU data sizes for intra coded frame and inter coded frame are greatly different. As shown in Figure 1, for intra mode the LCU data sizes are most in the range of 50 bytes to 1000bytes; while for inter mode the LCU data sizes are most in the range of 50 bytes to 500 bytes. On the other hand, the LCU data size gets smaller while QP getting bigger, take RaceHorses_832x480_20.yuv at QP 37 for example, the average LCU data sizes reduce to 106 bytes for intra mode and 20 bytes for inter mode (Figure 2). The tests are all down in the common condition of low complexity.

Figure 1 – LCU data size distribution for different coding mode (64x64 & QP 22)

Figure 2 – LCU data size distribution for different coding mode (64x64 & QP 37)

The problem is that it becomes difficult to control the data size of slices in terms of bytes in the case of big LCU size. To solve this problem, one choice is reducing the size of LCU to 32x32 or 16x16. The average LCU data size of intra modes of sequence RaceHorses_832x480_20.yuv at QP 22 reduces into 200 bytes for 32x32 (Figure 3), and into 48 bytes for 16x16 (Figure 4). However, smaller size of LCU may induces great loss of coding efficiency, and Table 1 gives the BD-rate loss of 32x32 LCU size compared to 64x64 LCU size.

Figure 3 – LCU data size distribution for different coding mode (32x32 & QP 22)

Figure 4 – LCU data size distribution for different coding mode (16x16 & QP 22)

Table 1 – BD rate loss of reducing the resolution of LCU from 64x64 to xxxxx

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.8 | 1.0 | 0.1 | 0.3 | 0.4 |
| Class B | 0.3 | 0.3 | 0.4 | 0.1 | 0.1 | 0.1 |
| Class C | 0.2 | 0.2 | 0.2 | -0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 |
| Class E | 0.3 | 0.3 | 0.4 | 0.0 | 0.2 | 0.2 |
| All | 0.2 | 0.3 | 0.4 | 0.0 | 0.1 | 0.1 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.7 | 3.6 | 3.2 | 5.7 | 5.4 | 5.2 |
| Class B | 2.6 | 1.7 | 1.6 | 4.1 | 4.2 | 3.9 |
| Class C | 1.0 | 0.7 | 0.7 | 0.9 | 1.0 | 1.2 |
| Class D | 0.5 | 0.1 | 0.4 | 0.5 | 0.4 | 0.7 |
| Class E |  |  |  |  |  |  |
| All | 2.0 | 1.5 | 1.5 | 2.9 | 2.8 | 2.8 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.9 | 2.0 | 2.0 | 3.1 | 3.2 | 3.6 |
| Class C | 0.9 | 0.8 | 0.7 | 0.6 | 0.8 | 0.5 |
| Class D | 0.2 | 0.4 | 0.0 | 0.2 | 0.3 | 0.3 |
| Class E | 8.9 | 4.9 | 5.3 | 11.4 | 12.5 | 11.9 |
| All | 2.8 | 1.8 | 1.8 | 3.3 | 3.6 | 3.6 |

# Fine granularity slice partition

Another solution of control slice data size more accurately is partitioning slice based on CU as shown in Figure 5. Slice 1 start from the 76th SCU in LCU 0 and end at the 155th SCU in LCU 2 (in this example, there are 256 SCU in each LCU).

Figure 5 – slice partition based on CU

To specify the start address accurately, we propose to write the index of the first LCU and SCU in the slice hierarchically in slice header. For example, the start address of slice 1 in Figure 3 is composed by the index of the first LCU in slice 1 (which is 0) and the relative index of the first SCU in slice 1 start from the first SCU in LCU 0 (which is 76). The syntax of slice start address can refer to Table 2.

Table 2 - slice start address in slice header

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| slice_start_is_pic_start_flag | 2 | u(1) |
| if(slice_start_is_pic_start_flag ==0) { |  |  |
| first_lcu_is_zero_flag | 2 | u(1) |
| if(first_lcu_is_zero_flag==1){ |  |  |
| first_scu_in_slice_minus_1 | 2 | u(v) |
| }else{ |  |  |
| first_lcu_in_slice_minus_1 | 2 | u(v) |
| first_scu_is_zero_flag | 2 | u(1) |
| if(first_scu_is_zero_flag == 0) { |  |  |
| first_scu_in_slice_minus_1 | 2 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

slice_start_is_pic_start_flag equal to 1 specifies the slice start from the first SCU in the first LCU in the picture. SliceStartLCUAddress and SliceStartSCUAddress both equal to 0.

first_lcu_is_zero_flag equal to 1 specifies the first LCU in the slice is the first one in the picture. first_lcu_is_zero_flag equal to 0 specifies the first LCU in the slice is not the first one in the picture, and the accurate address in the picture of the first LCU in the slice is specified by first_lcu_in_slice_minus_1.

first_lcu_in_slice_minus_1 specifies the address in the picture of the first LCU in the slice. SliceStartLCUAddress shall be computed as

SliceStartLCUAddress = first_lcu_in_slice_minus1+1

first_scu_is_zero_flag equal to 1 specifies the first SCU in the slice is the first one in the LCU. first_scu_is_zero_flag equal to 0 specifies the first SCU in the slice is not the first one in the LCU, and the accurate address in the LCU of the first SCU in the slice is specified by first_lcu_in_slice_minus_1.

first_scu_in_slice_minus_1 specifies the address in the picture of the first LCU in the slice. SliceStartSCUAddress shall be computed as

SliceStartSCUAddress = first_scu_in_slice_minus1+1

For context-based adaptive binary arithmetic coding (CABAC), it is necessary to indicate the end of slice. Similarly, we propose to signal the end address of hierarchically in slice header by writing the relative index the last LCU in slice start from the first LCU in slice (which is 2 in the example of slice 1) and the relative index of the last SCU in slice start form the first SCU in the last LCU (which is 155 in the example of slice 1). The syntax of slice end address can refer to Table 3.

Table 3 - slice end address in slice header

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| slice_end_is_pic_end_flag | 2 | u(1) |
| if(slice_end_is_pic_end_flag ==0) { |  |  |
| last_lcu_in_slice_minus_1 | 2 | u(v) |
| last_scu_is_zero_flag | 2 | u(1) |
| if(last_scu_is_zero_flag == 0) { |  |  |
| last_scu_in_slice_minus_1 | 2 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |

slice_end_is_pic_end_flag equal to 1 specifies the slice end at the last SCU in the last LCU in the picture. SliceEndLCUAddress equal to the number of LCU in one frame, and SliceEndSCUAddress equal to 0.

last_lcu_is_zero_flag equal to 1 specifies the last LCU in the slice is the first one in the picture. last_lcu_is_zero_flag equal to 0 specifies the last LCU in the slice is not the first one in the picture, and the accurate address in the picture of the last LCU in the slice is specified by last_lcu_in_slice_minus_1.

last_lcu_in_slice_minus_1 specifies the address in the picture of the last LCU in the slice. SliceEndLCUAddress shall be computed as

SliceEndLCUAddress = last_lcu_in_slice_minus1+1

last_scu_is_zero_flag equal to 1 specifies the last SCU in the slice is the first one in the LCU. last_scu_is_zero_flag equal to 0 specifies the last SCU in the slice is not the first one in the LCU, and the accurate address in the LCU of the last SCU in the slice is specified by last_lcu_in_slice_minus_1.

last_scu_in_slice_minus_1 specifies the address in the picture of the last LCU in the slice. SliceEndSCUAddress shall be computed as

SliceEndSCUAddress = last_scu_in_slice_minus1+1

Using the hierarchical address signaling, not only the CU based slice partition but also LCU based slice partition can be supported efficiently.

# Simulation results

This section reports the performance of slice partition based on LCU and CU respectively. In the experiments, the slices are set to be less than 1500bytes. Table 4 gives the BD-rate loss of slice partition based on 64x64 LCU compared to HM2.0, and Table 5 gives the BD-rate loss of slice partition based on 32x32 LCU compared to HM2.0. The BD-rate loss of slice partition based on CU is described in Table 6. It can be observed that partitioning slice based on CU reflect less on coding efficiency than reducing the size of LCU.

Table 4 -  BD-rate loss of slice partition based on LCU（64x64）

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.0 | 3.6 | 4.3 | 4.9 | 4.9 | 5.1 |
| Class B | 4.9 | 4.1 | 4.6 | 5.5 | 5.4 | 5.4 |
| Class C | 5.3 | 4.5 | 4.6 | 5.8 | 5.7 | 5.5 |
| Class D | 3.7 | 3.3 | 3.5 | 4.0 | 3.4 | 3.6 |
| Class E | 7.4 | 7.7 | 6.5 | 7.7 | 10.4 | 9.6 |
| All | 4.9 | 4.5 | 4.6 | 5.5 | 5.7 | 5.7 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.8 | 9.6 | 9.2 | 5.3 | 5.9 | 6.7 |
| Class B | 4.8 | 5.8 | 5.4 | 4.8 | 5.1 | 4.8 |
| Class C | 4.2 | 4.9 | 5.1 | 4.2 | 4.2 | 4.1 |
| Class D | 2.6 | 3.0 | 3.0 | 2.5 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 4.4 | 5.8 | 5.7 | 4.3 | 4.4 | 4.5 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.6 | 7.1 | 5.8 | 3.6 | 3.7 | 3.4 |
| Class C | 2.9 | 4.9 | 4.9 | 2.8 | 2.5 | 2.4 |
| Class D | 1.7 | 2.3 | 2.2 | 1.8 | 1.6 | 2.1 |
| Class E | 1.7 | 2.0 | 1.5 | 1.3 | 3.1 | 3.3 |
| All | 2.6 | 4.4 | 3.8 | 2.5 | 2.8 | 2.8 |

Table 5 -  BD-rate loss of slice partition based on LCU（32x32）

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.5 | 5.4 | 6.3 | 6.4 | 6.7 | 7.3 |
| Class B | 7.4 | 6.8 | 7.3 | 7.9 | 8.4 | 8.6 |
| Class C | 7.7 | 6.9 | 6.9 | 7.8 | 7.9 | 7.9 |
| Class D | 5.0 | 4.7 | 4.9 | 5.0 | 4.7 | 5.0 |
| Class E | 10.1 | 10.8 | 9.6 | 10.5 | 14.8 | 13.4 |
| All | 7.0 | 6.7 | 6.9 | 7.4 | 8.2 | 8.2 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 11.5 | 15.1 | 14.6 | 13.1 | 13.9 | 14.4 |
| Class B | 9.5 | 9.8 | 9.2 | 10.8 | 11.6 | 11.2 |
| Class C | 6.4 | 7.0 | 6.9 | 6.2 | 6.6 | 6.5 |
| Class D | 3.7 | 3.9 | 4.0 | 3.6 | 3.8 | 4.0 |
| Class E |  |  |  |  |  |  |
| All | 7.9 | 9.0 | 8.7 | 8.6 | 9.1 | 9.2 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 8.0 | 10.5 | 9.0 | 7.7 | 8.1 | 7.9 |
| Class C | 4.3 | 6.1 | 6.4 | 3.9 | 3.8 | 3.7 |
| Class D | 2.2 | 2.8 | 3.5 | 2.1 | 1.9 | 2.7 |
| Class E | 11.7 | 10.1 | 8.5 | 13.3 | 16.4 | 15.0 |
| All | 6.3 | 7.4 | 6.9 | 6.4 | 7.0 | 6.9 |

Table 6 -  BD-rate loss of slice partition based on CU

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.4 | 4.0 | 4.7 | 5.2 | 5.2 | 5.5 |
| Class B | 5.4 | 4.9 | 5.2 | 5.8 | 5.8 | 5.9 |
| Class C | 6.0 | 5.4 | 5.4 | 6.2 | 6.1 | 6.0 |
| Class D | 4.3 | 4.1 | 4.2 | 4.3 | 3.8 | 4.1 |
| Class E | 7.9 | 8.4 | 7.2 | 8.1 | 11.0 | 10.1 |
| All | 5.5 | 5.2 | 5.2 | 5.8 | 6.1 | 6.1 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.9 | 9.4 | 9.2 | 5.5 | 6.0 | 7.0 |
| Class B | 5.2 | 6.4 | 5.8 | 5.0 | 5.4 | 5.0 |
| Class C | 4.6 | 5.2 | 5.3 | 4.4 | 4.4 | 4.3 |
| Class D | 2.9 | 3.3 | 3.5 | 2.7 | 2.6 | 2.7 |
| Class E |  |  |  |  |  |  |
| All | 4.7 | 6.1 | 5.9 | 4.4 | 4.7 | 4.8 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.9 | 7.4 | 5.9 | 3.7 | 4.0 | 3.7 |
| Class C | 3.1 | 5.0 | 4.9 | 2.8 | 2.6 | 2.4 |
| Class D | 1.9 | 3.0 | 2.8 | 1.8 | 1.9 | 1.6 |
| Class E | 1.7 | 2.7 | 1.7 | 1.3 | 3.6 | 3.0 |
| All | 2.8 | 4.8 | 4.1 | 2.6 | 3.1 | 2.7 |

# Conclusion

Huawei recommends adopting fine granularity slice partition, and proposes to signal the slice start and end address in the slice header hierarchically.

- References

- "Test Model under Consideration", JCTVC-B205_draft005, Geneva, Switzerland, July 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-B300, Geneva, Switzerland, July 2010.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).