| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E301 |
| --- | --- |

| Title: | CE4 Subset1: Cross-check report for Ericsson’s fine granularity slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Qiu Shen Qingpeng Xie Haoping Yu Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | shenqiu@huawei.com alicxie@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the crosschecking procedures and results for Ericsson’s proposal in CE4 Subset1 activities (JCTVC-E260) [1].

# Test condition

The proposed fine granularity slices implemented into HM2.0-dev-slice reference software released by AHG-slice. The implementation conforms to the description in [1].

The test conditions specified in [1] are used for the simulation. VS2005 x32 compiler is used to generate the executable file from the provided source code. Simulations were done on computers running Windows Server 2008 x64 OS.

However, due to the limited computational resources, the encoding tasks are dispatched to machines of different type. So encoding and decoding time are not provided in the test results.

# Test results

The executables generated from the provided source code can be used for the encoding and decoding test. The decoded sequences match the reconstructed sequences.

The RD data provided by Ericsson match the results we obtained.

The results of case 8 and case 9 in CE4 Subset1 are presented in Table 1 and Table 2 respectively.

Table 1 – Cross-check results of case 8

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.0 | 3.6 | 4.3 | 4.9 | 4.9 | 5.2 |
| Class B | 5.0 | 4.3 | 4.7 | 5.6 | 5.5 | 5.6 |
| Class C | 5.5 | 4.8 | 4.8 | 5.9 | 5.8 | 5.7 |
| Class D | 3.9 | 3.6 | 3.7 | 4.1 | 3.6 | 3.8 |
| Class E | 7.5 | 7.8 | 6.7 | 7.8 | 10.7 | 9.7 |
| All | 5.1 | 4.6 | 4.7 | 5.6 | 5.8 | 5.8 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.7 | 9.3 | 8.5 | 5.3 | 5.9 | 6.7 |
| Class B | 4.9 | 6.0 | 5.5 | 4.9 | 5.2 | 4.9 |
| Class C | 4.3 | 5.0 | 5.0 | 4.3 | 4.2 | 4.1 |
| Class D | 2.6 | 2.9 | 3.0 | 2.6 | 2.4 | 2.8 |
| Class E |  |  |  |  |  |  |
| All | 4.4 | 5.8 | 5.5 | 4.3 | 4.5 | 4.7 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.7 | 7.1 | 5.7 | 3.6 | 3.7 | 3.5 |
| Class C | 2.9 | 4.9 | 4.8 | 2.8 | 2.5 | 2.3 |
| Class D | 1.7 | 2.6 | 2.2 | 1.8 | 1.8 | 2.1 |
| Class E | 1.6 | 2.4 | 1.1 | 1.3 | 3.2 | 2.8 |
| All | 2.6 | 4.5 | 3.8 | 2.5 | 2.9 | 2.7 |

Table 1 – Cross-check results of case 9

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.0 | 3.6 | 4.4 | - | - | - |
| Class B | 5.0 | 4.3 | 4.7 | - | - | - |
| Class C | 5.5 | 4.8 | 4.8 | - | - | - |
| Class D | 3.9 | 3.6 | 3.7 | - | - | - |
| Class E | 7.5 | 7.8 | 6.6 | - | - | - |
| All | 5.1 | 4.6 | 4.7 | - | - | - |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.7 | 9.0 | 8.7 | - | - | - |
| Class B | 4.9 | 6.0 | 5.5 | - | - | - |
| Class C | 4.3 | 5.0 | 5.1 | - | - | - |
| Class D | 2.6 | 2.9 | 3.1 | - | - | - |
| Class E |  |  |  | - | - | - |
| All | 4.4 | 5.7 | 5.6 | - | - | - |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  | - | - | - |
| Class B | 3.6 | 7.0 | 5.5 | - | - | - |
| Class C | 2.9 | 4.7 | 4.9 | - | - | - |
| Class D | 1.7 | 3.1 | 2.4 | - | - | - |
| Class E | 1.5 | 1.9 | 1.3 | - | - | - |
| All | 2.6 | 4.5 | 3.8 | - | - | - |

# Conclusion

Huawei has crosschecked the proposal of Ericsson for CE4 Subset1, and the RD performance provided by Ericsson match the results we obtained.

- References

- Rickard Sjöberg, “Fine Granularity slices”, JCTVC-C154, Guangzhou, China, October 2010.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).