| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E302 |
| --- | --- |

| Title: | Modification for robust parsing of AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhang, Qingpeng Xie, Qiu Shen, Haoping Yu Huawei Technologies Co., Ltd. Huawei Building, 3 Xinxi Road, Haidian District,Beijing, China. | Tel: Email: | +86 10 82882648 steven.zhanglei@huawei.com alex.xieqingpeng@huawei.com shenqiu@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

A method proposed for robust parsing in advanced motion vector prediction (AMVP). If there are three motion vector predictor (MVP) candidates (including left MVP, above MVP and temporal MVP) all existing, merge the spatial MVP candidates into only one spatial MVP candidate, and only one bit needed to be encoded for MVP index. Meanwhile if no spatial MVP exists, no bits should be coded into the stream. There is no parsing problem in AMVP, and the average loss is not too much either.

# Introduction

There is still a problem of error resilience existing in current version of HEVC. Decoding will be possibly crushed if the reference frames lost when current frame using temporal motion vector prediction (MVP). So a method is needed to solve this problem.

In this proposition, four solutions to solve the parsing problem have been tested. And one method using modified advanced motion vector prediction (AMVP) is proposed. The average loss compared to other methods is further reduced.

# Problem statement

In the current HM2.0 design, the number of MVP candidates has been reduced to three, including left MVP, above MVP and temporal MVP, referenced in JCTVC-D231[2].

The coding bitstream is related to MVP numbers and MVP index. If only one MVP candidate exists, MVP index is not coded, which can be inferred when decoding. If two MVP candidates exist, one bit marked MVP index should be coding into stream making sure which MVP is adopted. If three MVP candidates all exist, the stream should be coding as the following table.

| MVPIdx | MVPNum =1 | MVPNum=2 | MVPNum=3 |
| --- | --- | --- | --- |
| 0 | (inferred) | 0 | 0 |
| 1 |  | 1 | 10 |
| 2 |  |  | 11 |

(Reference: MVPIdx means MVP index, MVPNum means the number of MVP)

With the temporal MVP used in AMVP, the parsing problem still exists. For example, if reference frame is lost, the temporal MVP used in MVP candidates of current frame is possibly wrong, and which will influence the number of MVP candidates. If the number of MVP is wrong, decoder would not know how many bits on earth should be parsed, which may induce the parsing problem.

# Testing of robust solutions

Robust reference 1: “Spatial Left” – As three MVP candidates all exist, including left MVP, above MVP and temporal MVP, merge the two spatial MVP candidates (left MVP and above MVP) into one spatial MVP, the method is getting rid of above MVP. And only one spatial MVP and one temporal MVP exist now, so just one bit should be coded to tell the decoder the correct MVP index.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.5 | 0.5 | 0.8 | 0.7 | 0.6 |
| Class B | 0.3 | 0.4 | 0.4 | 0.8 | 0.7 | 0.7 |
| Class C | 0.6 | 0.6 | 0.6 | 1.0 | 0.9 | 0.9 |
| Class D | 0.7 | 0.6 | 0.8 | 1.1 | 0.9 | 1.0 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.5 | 0.6 | 0.9 | 0.8 | 0.8 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 0.7 | 0.3 | 1.3 | 0.6 | 0.8 |
| Class C | 0.7 | 0.9 | 0.7 | 1.2 | 0.7 | 0.8 |
| Class D | 0.8 | 1.0 | 0.6 | 1.3 | 0.7 | 0.7 |
| Class E | 0.4 | 0.7 | 1.2 | 4.2 | 2.4 | 3.2 |
| All | 0.6 | 0.8 | 0.6 | 1.8 | 1.0 | 1.2 |
| All wo E | 0.6 | 0.9 | 0.5 | 1.3 | 0.7 | 0.7 |

(Reference: CU merge non robust, HM2.0 default configuration, positive values indicate a loss)

Robust reference 2: “Spatial Above” – As three MVP candidates all exist, including left MVP, above MVP and temporal MVP, merge the two spatial MVP candidates (left MVP and above MVP) into one spatial MVP, the method is getting rid of left MVP. And only one spatial MVP and one temporal MVP exist now, so just one bit should be coded to tell the decoder the correct MVP index.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.6 | 0.4 | 0.9 | 0.8 | 0.8 |
| Class B | 0.4 | 0.4 | 0.5 | 0.9 | 0.7 | 0.7 |
| Class C | 0.6 | 0.5 | 0.6 | 0.9 | 0.8 | 0.8 |
| Class D | 0.7 | 0.6 | 0.8 | 1.1 | 1.0 | 1.1 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.5 | 0.6 | 0.9 | 0.8 | 0.9 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 0.7 | 0.6 | 1.3 | 0.6 | 0.6 |
| Class C | 0.6 | 0.9 | 0.9 | 1.1 | 0.8 | 0.7 |
| Class D | 0.7 | 0.4 | 0.5 | 1.2 | 0.9 | 0.5 |
| Class E | 0.3 | 0.8 | 1.1 | 4.2 | 2.5 | 3.1 |
| All | 0.5 | 0.7 | 0.7 | 1.8 | 1.1 | 1.1 |
| All wo E | 0.6 | 0.7 | 0.6 | 1.2 | 0.8 | 0.6 |

(Reference: CU merge non robust, HM2.0 default configuration, positive values indicate a loss)

Robust reference 3: “Spatial Average” – As three MVP candidates all exist, including left MVP, above MVP and temporal MVP, merge the two spatial MVP candidates (left MVP and above MVP) into one spatial MVP, the method is get the average of two spatial MVP candidates. And only one spatial MVP and one temporal MVP exist now, so just one bit should be coded to tell the decoder the correct MVP index.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.5 | 0.4 | 0.7 | 0.6 | 0.5 |
| Class B | 0.2 | 0.3 | 0.3 | 0.6 | 0.5 | 0.5 |
| Class C | 0.4 | 0.4 | 0.5 | 0.7 | 0.7 | 0.7 |
| Class D | 0.6 | 0.4 | 0.5 | 0.8 | 0.7 | 0.7 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | 0.4 | 0.4 | 0.7 | 0.6 | 0.6 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 0.7 | 0.4 | 1.3 | 0.7 | 0.7 |
| Class C | 0.6 | 0.8 | 0.6 | 1.0 | 0.7 | 0.6 |
| Class D | 0.6 | 0.7 | 0.6 | 1.2 | 0.7 | 0.4 |
| Class E | 0.2 | 0.6 | 0.7 | 4.2 | 2.9 | 3.3 |
| All | 0.5 | 0.7 | 0.6 | 1.7 | 1.1 | 1.1 |
| All wo E | 0.5 | 0.7 | 0.5 | 1.2 | 0.7 | 0.6 |

(Reference: CU merge non robust, HM2.0 default configuration, positive values indicate a loss)

# Proposal: description and results

The proposed method is based on robust reference 3 (Spatial Average)

If there is any spatial MVP existing, it should be coded MVP index, and if there is no spatial MVP existing in MVP candidates, the MVP index is not coded, which is inferred in the decoder. And it would not influence the decoder parsing even if any packets lost happen.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.3 | 0.0 | 0.5 | 0.4 | 0.4 |
| Class B | 0.0 | 0.2 | 0.2 | 0.4 | 0.4 | 0.4 |
| Class C | 0.2 | 0.2 | 0.3 | 0.5 | 0.5 | 0.5 |
| Class D | 0.3 | 0.3 | 0.4 | 0.5 | 0.5 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.2 | 0.2 | 0.5 | 0.4 | 0.4 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.5 | 0.3 | 0.8 | 0.4 | 0.7 |
| Class C | 0.2 | 0.6 | 0.3 | 0.6 | 0.5 | 0.3 |
| Class D | 0.2 | 0.3 | -0.1 | 0.6 | 0.2 | 0.0 |
| Class E | 0.3 | 0.8 | 0.9 | 3.6 | 2.9 | 3.0 |
| All | 0.2 | 0.5 | 0.3 | 1.2 | 0.8 | 0.9 |
| All wo E | 0.2 | 0.4 | 0.1 | 0.7 | 0.4 | 0.4 |

(Reference: CU merge non robust, HM2.0 default configuration, positive values indicate a loss)

# Conclusion

The proposed robust method not only solves the parsing problem, but also without too much performance loss. Consequently, we recommend the adoption of the modified advanced motion vector prediction for packets lost. Furthermore, the proposed robust method is just for AMVP parsing problem, meanwhile merge mode has the similar parsing problem to be solved.

# References

- JCT-VC, “Proposition for robust parsing with temporal predictor”, JCTVC-D197, JCT-VC Meeting, Daegu, January 2011

- JCT-VC, “CE9 3.2d Simplified Motion vector prediction”, JCTVC-D231, JCT-VC Meeting, Daegu, January 2011

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).