| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E303 |
| --- | --- |

| Title: | Cross-checking of an investigation on robust parsing from Microsoft |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Lei Zhang Qiu Shen Qingpeng Xie Haoping Yu Huawei Technologies Co., Ltd. Huawei Building, 3 Xinxi Road, Haidian District,Beijing, China. | Tel: Email: | +86 10 82882648 steven.zhanglei@huawei.com alex.xieqingpeng@huawei.com shenqiu@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This is a report of the verification tests that Huawei has performed on Microsoft’s proposals on robust bitstream parsing JCTVC-E148, and the main purpose of these tests is to confirm the reported robustness of decoder in handling corrupted bitstreams. The simulation results are consistent with the claims made by the proponents, and the decoder indeed didn’t crash while decoding the impaired video streams provided by Microsoft.

# Introduction

An error resilience issue still exists in the current HEVC design. As decoder uses the temporal information that might have been lost in error-prone channel to decode the current frame, the bistream parsing process may crash and causes decoder a catastrophic failure. To address this severe decoding problem, Microsoft proposes a few possible solutions. This contribution is to report the crosschecking results for two typical schemes of them.

# Test Conditions

Microsoft proposes robust parsing solutions for HM2.0; one of them is called as “TIDR mode” and another is “code full index”. Both of them are tested under the same testing conditions as described below:

- Sequence

BasketballDrill_832x480_50.yuv

- Case

Lowdelay and Lowdelay-LC

- Encoder provided by Microsoft

QP: 22		Number of frames: 500

- Data loss simulation in inter frames

Lose one frame each time, but keep the 192-bit slice header (Now one frame has only one slice). All of the original bitstreams can be decoded correctly by the provided decoder.

- Decoder provided by Microsoft

The decoder takes the lost frame number as an extra input

# Test Results

“TIDR” scheme from Microsoft

| Test Case | Frame Lost Number | Decode without collapse(Yes/No) |
| --- | --- | --- |
| Lowdelay | 1 | Yes |
|  | 2 | Yes |
|  | 3 | Yes |
|  | … | … |
|  | 499 | Yes |
| Lowdelay-LC | 1 | Yes |
|  | 2 | Yes |
|  | 3 | Yes |
|  | … | … |
|  | 499 | Yes |

By losing one inter-frame each time, we generated a total of 998 “bad” sequences in two cases (Lowdelay and Lowdelay-LC). The provided decoder can decode all of these bad bitstreams without any crash.

“Code full index” scheme from Microsoft

| Test Case | Frame Lost Number | Decode without collapse(Yes/No) |
| --- | --- | --- |
| Lowdelay | 1 | Yes |
|  | 2 | Yes |
|  | 3 | Yes |
|  | … | … |
|  | 499 | Yes |
| Lowdelay-LC | 1 | Yes |
|  | 2 | Yes |
|  | 3 | Yes |
|  | … | … |
|  | 499 | Yes |

By losing one inter-frame each time, we generated a total of 998 “bad” sequences in two cases (Lowdelay and Lowdelay-LC). The provided decoder can decode all of these bad bitstreams without any crash.

# Conculsion

Huawei has performed cross-checking tests on Microsoft’s proposals on robust bitstream parsing. It is confirmed that, under the proposed test conditions, the decoders didn’t crash while dealing with the impaired bitstreams for these two solutions.

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).