| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E0xx |
| --- | --- |

| Title: | Overlapped Block Motion Compensation for Regular Block Partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo, Peisong Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8125 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, overlapped block motion compensation (OBMC) has been applied to regular partitions (2NxN and Nx2N block partitions) in HM 2.0. Specifically, if a CU is 2NxN or Nx2N, OBMC is applied to the horizontal boundary of two 2NxN partitions, and the vertical boundary of two Nx2N partitions. In our simulation, it is observed a BD-rate reduction 0.54% on average for random access case, and 0.81% on average for low delay case. We observed there is no increase in encoding time and 1%-3% increase in decoding time.

# Introduction

OBMC [1] uses overlapped blocks in motion compensation to reduce blocking artifact along macroblock boundaries. The effective of OBMC on flexible motion partitions (e.g., Asymmetric Motion Partition and Geometry motion partition) has been demonstrated in previous JCTVC meetings [2] [3]. In this contribution, OBMC is extended to regular block partition (2NxN and Nx2N) in HM2.0 software. The introduction of OBMC can improve coding efficiency by smoothing out the partition boundaries and reducing the high frequency energy, resulting in better transform coding efficiency and visual quality.

# OBMC for Regular Partition Block

In HM software, a CU of size 2Nx2N can be partitioned into four different ways: a whole 2Nx2N partition, two 2NxN partitions, two Nx2N partitions and four NxN partitions. If the selected transform for this transform is 2Nx2N while the block partition is 2NxN, Nx2N or NxN, the block transform will cover pixels from multiple partitions. Since those partitions may have different motion vectors, the pixels at partition boundaries may have large discontinuity, which may generate visual artifacts and also reduced transform/coding efficiency.

Figure 1 OBMC for (a) 2NxN and (b) Nx2N block. Weight (3/4, 1/4) for “A” pixels, and (7/8, 1/8) for “B”pixels.

Figure 1 demonstrates OBMC for 2NxN (Fig. 1(a)) and Nx2N blocks (Fig. 1(b)). The white pixels are pixels belonging to Partition 0 and grey pixels are pixels belonging to Partition 2. The overlapped region in the luma component is defined as 2 rows (columns) of pixels on both sides of the horizontal (vertical) boundary. For pixels which are 1 row (column) apart from the partition boundary, i.e., pixels labeled as A in Figure 1, OBMC weighting factors are (3/4, 1/4). For pixels which are 2 rows (columns) apart from the partition boundary, i.e., pixels labeled as B in Figure 1, OBMC weighting factors are (7/8, 1/8).  For chroma components, the overlapped region is defined as 1 row (column) of pixels on both sides of the horizontal (vertical) boundary, and the weighting factors are (3/4, 1/4).

# Experimental results

We implement OBMC for regular block partition on top of HM 2.0. The coding efficiency is evaluated under Common conditions specified in [4]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time is measured on clusters with same CPU, and the decoding time is measured by decoding all sequences on a single PC.

Table 1 provides the BD rate reduction and the running time of the proposed scheme. On average there is a 0.53% rate reduction for Random Access cases, and 0.81% reduction for Low Delay cases. The maximum bit rate reduction can be 1.17% for Class E in Low Delay mode. We observe there is consistent encoding time reduction in every case. There is slight decoding time increase.

| BD-rate | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.30 | -0.33 | -0.32 | -0.47 | -0.81 | -0.63 |
| Class B | -0.47 | -0.64 | -0.65 | -0.42 | -0.51 | -0.59 |
| Class C | -0.77 | -1.18 | -1.13 | -0.67 | -0.94 | -1.08 |
| Class D | -0.69 | -0.90 | -0.87 | -0.52 | -0.92 | -0.78 |
| Class E |  |  |  |  |  |  |
| All | -0.56 | -0.86 | -0.83 | -0.51 | -0.78 | -0.76 |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
| BD-rate | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.56 | -0.61 | -0.65 | -0.48 | -0.46 | -0.27 |
| Class C | -0.87 | -0.90 | -0.75 | -0.79 | -0.80 | -1.16 |
| Class D | -1.04 | -0.91 | -1.15 | -0.82 | -0.84 | -0.70 |
| Class E | -1.17 | -1.43 | -0.72 | -0.98 | -0.85 | -0.79 |
| All | -0.87 | -0.91 | -0.81 | -0.74 | -0.71 | -0.70 |
| Enc Time[%] | 98% |  |  | 95% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

Table 1 BD rate reduction of the proposed scheme

# Conclusion

In this contribution, OBMC technique is applied to 2NxN and Nx2N block partition in HM2.0. We observe on average 0.54% bit rate reduction in random access case, and 0.81% for low delay case. There is slight encoding time reduction and decoding time increase. We recommend OBMC integrated into HM software.

# References

- P. Chen, R. Panchal, W. Chieng and M. Karczewicz, “Overlapped block motion compensation in TMuC”, JCTVC-C251, JCT-VC Meeting, Guangzhou, CN, Oct. 2010.

- L. Guo, P. Chen, and M. Karczewicz “CE2: Overlapped Block Motion Compensation for Geometry Partition Block”, JCTVC-D368, JCT-VC Meeting, Daegu, KR, Jan. 2011.

- M.-S. Cheon, I.-K. Kim, L. Guo, P. Chen, and M. Karczewicz,  “CE2: Asymmetric motion partition with overlapped block motion compensation”, JCTVC-D367, JCT-VC Meeting, Daegu, KR, Jan. 2011.

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC Meeting, Daegu, KR, Jan. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).