| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E305_r1 |
| --- | --- |

| Title: | Video Test Material Submission for "Screen Content" Coding Experiments: Scrolling Text, Overlays, Editing, and Window Switching |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gregory Cook (1) Wen Gao (1) Dong Wang (2) Jiantong Zhou (2) Haoping Yu (1) (1). Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 (2). Huawei Technologies Co., LTD Section B, Bantian, Longgang District Huawei Base, Shenzhen, China | Tel: +1 408 330 4816 Email: gregory.cook@huawei.com wen.gao@huawei.com dave.wangdong@huawei.com zhoujiantong@huawei.com haoping.yu@huawei.com |  |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution consists of a submission of nine test sequences in response to W11867, “Request for Video Test Material for ‘Screen Content’ Coding Experiments”.   Copyright statements as well as detailed description of the sequences and the creation process of the sequences were included. The sequences consist of camera-view content with text overlays and scrolling text, and camera-view content with document editing, scrolling and window switching.  By construction, the sequences are free of compression.  It was also noted that, compared to the HM2.0 anchor sequences, the compression of these sequences leads to quite varied results, including both a significant increase and decrease of the BD-rate.

# Introduction

This contribution is submitted in response to [1], a request for video test material for “Screen Content” Coding (SCC) experiments.   We are submitting two sets of sequences for consideration: the first set contains camera-view content with text overlays and scrolling text.  An example use of this kind of screen content is for television news, sports, or infotainment shows.  The second set of sequences contains camera-view content with document editing, scrolling and window switching. A telepresence system where video and documents are mixed together is a good example of the use of this kind of screen content.

Important considerations for this content are that it be uncompressed and also that a copyright release be easily obtained [1].   For the camera-view portion of the content we chose to use selected Class B, Class C, and Class D sequences of the HEVC test set [2].    With careful construction of the sequences we are able to produce sequences which are free of compression.  Adobe After Effects was used to generate uncompressed screen content for the first set of sequences.   For the second set, a digital capture card was employed; it is able to capture uncompressed YUV video at 1280x720 60 fps.  In both cases the original pixels of the HEVC test sequences are used, where appropriate, in the final sequence.

As a consequence, the content under submission is free of compression, and also has appropriate copyright such that it is usable for study and improvement of the effectiveness of the draft HEVC standard.

In addition, use of the HEVC test sequences as a basis of the SCC sequences allows a direct comparison of compression efficiency with the SCC counterparts.    We have included with this contribution an analysis of the sequences using the HM2.0 and Common Test Conditions as anchors.

# Sequence Description

## Camera-view content with text overlays and scrolling text

The sequences BasketballDrillText_832x480_50.yuv, BQMallText_832x480_60_yuv, PartySceneText_832x480_50.yuv, RaceHorsesText_832x480_30.yuv, BasketballDriveText_1920x1080_50.yuv, BQTerraceText_1920x1080_60.yuv, CactusText_1920x1080_50.yuv were created using text overlays and scrolling text over camera-view content.   All of the sequences are 10 seconds in length, and have a text overly in the upper right corner, and scrolling text along the bottom edge.

These sequences were created using the following process:

- The original sequence is converted to RGB PNG files using the available MATLAB script YUV2Image which in turn is based on the MATLAB Image Processsing Toolbox.

- These RGB files are imported into Adobe After Effects

- The “HEVC Test Sequence” overlay (also known as a “bug”) is a 200x100 image placed in the upper-right corner with an alpha blend of 75%.

- The scrolling text (also known as a “crawl”) uses an Ariel font, and is moved using After Effects functions from left-to-right at the constant rate of one screen width per eight seconds for all sequences.

- The background of the scrolling text was created using the Adobe Photoshop gradient tool, with colors Black (000000) and Blue (0066CC).

- The sequences rendered by Adobe After Effects are then converted back to YUV through the YUV2Image function.

- Finally, a C program is run on the sequences which retain the original pixels of the sequence if they were not affected by the overlay or the scrolling text.

Except for RGB/YUV colorspace conversions in those areas where the overlay is placed and where the scrolling text appears, the sequence is identical to the original and no compression is introduced.

## Camera-view content with document editing, scrolling and window switching

The sequences ScreenCaptureDocEditing_1280x720_60.yuv and ScreenCaptureWinSwitching_1280x720_60.yuv were created by mixing camera-view content with a screen capture of typical graphical user interface actions.   The first sequence has primarily document editing actions such as scrolling, insertion, and deletion, while the second sequence has primarily document window resizing, moving, and switching effects.  Both sequences are 10 seconds in length, and have camera-view video embedded within them.

The sequences were created using the following process:

- A Dell Studio XPS 9100 with an ATI Radeon HD 5800 Series graphics card running OS Microsoft Windows was put into “Duplicate Display” mode, with each DVI-D output set to 1280x720 at 60 Hz.

- One of the DVI-D outputs of the above computer was connected to a monitor.  The other output was connected to an NCast Corporation DCC 4.0 digitizing capture card.   This card is installed in another Dell Studio XPS 9100 running Centos 5.5 (x86_64), Linux kernel 2.6.18, and a Seagate Barracuda XT ST32000641AS hard drive.

- Using the application software “dcccapture” as supplied by NCast, the capture is accomplished by running the following command line:

./dcccapture --to-file /tmp/videoXX.yuv --fps 60

- To emulate document editing and other typical graphical user interface actions, various editing and window switching actions are then performed on the first Dell XPS while the capture is occurring on the second Dell XPS.

- A C program is then run to crop the sequence to 10 seconds, and also replace the pixels in the captured frames which represent camera-view video content with the actual pixels from the original uncompressed Class D sequence BQSquare_416x240_60.yuv.

The net effect of these steps is that there is no compression introduced into the capture process.

# Analysis of Sequences as Compared with the HM2.0 Common Test Condition Sequences

We have attached to this submission an Excel spreadsheet (JCTVC-E305.xls) containing a BD-rate [4] comparison of the sequences for consideration with their original test sequence counterpart using the Common Test Conditions [3] for the Intra, Random Access, and Low Delay cases.   Our observations and analysis are below.

## Difficulty of coding the camera-view content with text overlays and scrolling text

The scrolling text with gradient background is coded consistently in the sequences, and is relatively difficult to code compared to the Class C sequences.  This can be seen by comparing the BD-Rate with the average rate of the anchor sequences, which is used in this case as a measure of the difficulty of coding the sequences.   This average rate is computed using the same steps as the Bjontegaard Delta method [4], however, the average rate of the anchor sequence is found instead of the ratio of two average rates.

(a)

(b)

(c)

Figure 1 The BD-rate between the original and “Text” sequences plotted against the average rate of the original sequence for the Intra, Random access, and Low delay cases for the Class C sequences.

(a)

(b)

(c)

Figure 2 The BD-rate between the original and “Text” sequences plotted against the average rate of the original sequence for the Intra, Random access, and Low delay cases for the Class B sequences.

Figure 1 (a)-(c) demonstrates that the general trend of difficulty in coding the scrolling text holds for the Intra, Random Access, and Low delay cases.   In the case of Intra for the most difficult sequence, PartyScene, there is decrease in the BD-rate, while the other three sequences are clustered together with an increase in the BD-rate.  This indicates the scrolling text is somewhat easier to code in general than PartyScene, but more difficult than the other sequences.   This same observation holds for the Random Access case.  For the Low Delay case, Figure 1(c) indicates that the scrolling text was actually more difficult than all four sequences (including PartyScene) indicating that the Low Delay test condition has more difficulty with the scrolling text than the other two test conditions.

Figure 2 (a)-(c) also demonstrates that the general trend described previously holds, although in general the Class B sequences are more difficult to code than the Class C sequences, as compared to the scrolling text.  In the Random Access case (Figure 2 (b)), all three Class B sequences show a negative BD-rate, indicating the scrolling text is easier to code than the sequences themselves.

We may also make another observation.  Comparing the BD-rate for a single sequence for the Intra, Random Access, and Low Delay test conditions, the lowest BD-rate among the three conditions was for the Intra case, with the exception for the PartyScene sequence.  This indicates that the HM2.0 has the most difficulty with the scrolling text in Intra mode.   With the exception of PartyScene and BQMall, the Random Access condition had the highest BD-rate, indicating that this method was most effective in coding the scrolling text in the majority of the cases.

## Difficulty with chroma when coding camera-view content with text overlays and scrolling text

In general, the U and V BD-rates were much higher when compared with the Y BD-rate.  This was in general true for all three test conditions.  Visual inspection of the sequences indicates that this may be due to the lower contrast where the white text is over the blue background, as opposed to white text over the black background, which showed much fewer visual artifacts.

## Observations for camera-view content with document editing, scrolling and window switching

In general, visual inspection of the coded sequences indicates that, compared to the embedded video, for the Random access and Low delay test conditions, the document area looks reasonably good even for a QP of 37.   Close inspection of high frame rate effects such as document closing indicate that these two test conditions do not capture these effects very well.   For the Intra test condition, both the embedded video and document area indicated a large amount of flashing or flickering, and probably would not be acceptable in a telepresense system, for example.

In contrast to Section 3.1, the effectiveness of the Low Delay test condition is much better as compared to the Random access test condition, with a BD-rate of -20.8% and -16.4% for the DocEditing and WinSwitching cases, respectively.

# Conclusions

We may conclude from this analysis that scrolling text with gradient background is roughly equivalent in coding difficulty with the current test sequences.  There is some difficulty in coding the chroma components, and this would need to be addressed.  There was also a remarkable reversal in the effectiveness of the Random access test conditions versus the Low delay case for the camera-view content with document editing, scrolling and window switching case as compared to the scrolling text case.

It will be very important to judge the effectiveness of the HM for screen content both using the usual PSNR/rate methods and visual inspection, as changes in text can easily be discerned by the human visual system.

In general these sequences provide a different and unique test for the HM, and we recommend that they be adopted as Class F test sequences.

# References

- “Request for Video Test Material for "Screen Content" Coding Experiments”, ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Audio, W11867, January 2011.

- “Joint Call for Proposals on Video Compression Technology”, ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Audio, W11113, January 2010.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, 4th JCT-VC Meeting, Daegu, KR, January 2011.

- Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

# Patent rights declaration(s)

Huawei Technologies (USA) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The following copyright statement accompanies each of the following submitted sequences: BasketballDrillText_832x480_50.yuv, BQMallText_832x480_60_yuv, PartySceneText_832x480_50.yuv, RaceHorsesText_832x480_30.yuv, BasketballDriveText_1920x1080_50.yuv, BQTerraceText_1920x1080_60.yuv, ScreenCaptureDocEditing_1280x720_60.yuv and ScreenCaptureWinSwitching_1280x720_60.yuv.

Other than the copyrighted sequences used as a basis for the creation of these sequences, these sequences and all intellectual property rights therein remain the property of Huawei Technologies (USA).    Copyright as claimed by the original owners, in this case NTT DOCOMO, INC, remains with the owners.  These sequences may only be used for the purpose of developing, testing and promulgating  technology standards.   Huawei Technologies (USA) makes no warranties with respect to the sequences and expressly disclaims any warranties regarding their fitness for any purpose.

The following copyright statement accompanies the following submitted sequence: CactusText_1920x1080_60.yuv

Other than the copyrighted sequences used as a basis for the creation of these sequences, these sequences and all intellectual property rights therein remain the property of Huawei Technologies (USA).    Copyright as claimed by the original owners, in this case RAI, remains with the owners.  These sequences may only be used for the purpose of developing, testing and promulgating  technology standards.   Huawei Technologies (USA) makes no warranties with respect to the sequences and expressly disclaims any warranties regarding their fitness for any purpose.