| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E309 |
| --- | --- |

| Title: | Parameter set updates using conditional replacement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger 433 Hackensack Ave. Hackensack, N.J. 07601 USA | Tel: +1-415-713-5473 Email: stephan@vidyo.com |  |
| Source: | Vidyo, Inc |  |  |

_____________________________

# Abstract

Proposed is a mechanism to allow updates of existing parameter sets, without requiring the transmission of the complete parameter set data.  The mechanism can be implemented without syntax changes relative to H.264/AVC’s high level syntax, though a change in the semantics is required.  The key motivation behind this change is the presumption that the use of ALF slice parameter sets, as proposed in companion document JCTVC-E281, may require frequent parameter set transmissions where the parameter sets differ only slightly in content.  Therefore, an efficient “delta” transmission of individual parameters of the parameter set is considered beneficial by the authors. Also proposed is a parameter set maintenance mechanism, one of whose incarnation is copy.

# Introduction, Overview, and Motivation

In this proposal, it is assumed that there is a use case for (slice?) parameter sets that contain content that could be efficiently—that is, by using fewer bits than an outright transmission of the parameter set—derived from a previously transmitted and stored (slice) parameter set.  While the concept is applicable to any form of parameter set (and should presumably allowed for all, for orthogonality reasons), the main use case is the update of ALF coefficients.

In H.264/AVC, parameter sets are “intra” information.  If one sends a parameter set, out of band or in the bitstream, the sent parameter set overwrites the information at the location specified in the parameter set.  If the optional parts of a parameter set (such as HRD or VUI information in the sequence parameter set) are empty, then the values of these parameters are undefined, even if they were defined before at the same parameter set location.  The authors believe that this was a sensible design choice for H.264/AVC, as parameter sets there are both small and infrequently sent.

Conceivably, mechanisms such as the ALF coefficient transmission may change this picture.  Being content adaptive, there is no way to know how many ALF coefficients are generated in relation to the number of coded slices (or coded pictures, if the adaptivity were restricted to the picture level).  Accordingly, we are potentially looking at many parameter set transmissions even within a GOP, of potentially similar data (the data is similar by nature—ALF coefficients—and may be similar in values as well).

Video coding has long ago addressed this problem by the transmission of difference information; conditional replacement was introduced as early as ITU _T Rec. H.120, and Inter Mode is known since at least ITU-T Rec. H.261.

This contribution advocates changing the Intra-only behavior of parameter set transmissions to conditional replacement, or Inter parameter sets.  That is, when a parameter set is sent with an optional part not present, the information for this optional part available in the storage location remains valid.  (More complex mechanisms, that would allow, for example, the delta update of values in the parameter set instead of conditionally replacing them, may be an overkill.)

In JCT-VC, we have the option to structure parameter sets such that many, if not all, parameters in the set can be “optional” (in the sense that their presence is indicated by a flag), making the semantic change much more useful.  The overhead would be one bit per parameter in the finest possible granularity, but coarser granularity steps are equally possible.

This idea came up when thinking about the transmission of ALF coefficients in slice parameter sets, as proposed in companion document JCTVC-E281.  In this context, there may be use cases where parameter set maintenance messages may become useful.  We write “may” here not because of a copy-paste from patent lingo :-), but because we are unsure whether this is indeed a strong use case. The author of this paragraph, not “living in the loop”, admits lack of detailed understanding of the ALF mechanisms currently in the draft or proposed.  Insofar, the following is meant as a basis for a discussion, rather than a concrete proposal.  Perhaps the ALF experts can advise.

A parameter set update mechanism becomes immediately more useful if there is a way to avoid sending, to all used parameter set locations, and intra parameter set.  A “copy” command would be useful that copies the data of one specified parameter set to another location.  For architectural cleanliness and extensibility, we suggest to introduce a parameter set maintenance NAL unit, one of which incarnations could be “copy”.

# Proposal #1: Semantic change to support conditional replacement

The following, conceptual proposal assumes that there were a slice parameter set that includes only n ALF coefficients.  How these coefficients are ordered, entropy coded, used, and whether they belong to a single filter, a single filter set, or multiples thereof, is irrelevant for this proposal.

Below is a simplified syntax of a hypothetical slice parameter set to support ALF not allowing for conditional replacement:

“Intra” Slice PS:

Slice PS Reference	ue(v)		// the storage location for this slice PS

PPS Reference 		ue(v)		// backlink to the PPS

ALF_coeff 0		ue(v)		// first ALF coefficient (quantized etc.)

ALF_coeff 1		ue(v)		// second ALF coefficient

ALF_coeff 2		ue(v)

…

ALF_coeff n		ue(v)		//nth ALF coefficient

In order to use “inter” parameter set updates, two conceptual additions are required.

First, at a granularity to be decided, flags have to be introduced to label the present/absent information of the parameter set.

Second, and optionally, it may make sense (and has to be decided) to allow, with a single sending of a parameter set, to update more than one storage location.

The resulting “inter/intra” slice PS could look as follows:

Slice PS Reference		ue(v)		// the storage location for this slice PS

// perhaps here a counter for additional

// Slice PS references, and those refs

PPS Ref_present_flag		u(1)

if (Ref_present_flag) {

PPS Reference 			ue(v)		// backlink to the PPS

}

ALF_coeff_0_present_flag	u(1)		// first coefficient present?

If (ALF-coeff_0_present_flag) {

ALF_coeff 0			ue(v)		// first ALF coefficient (quantized etc.)

}

ALF_coeff_12_present_flag)	u(1)		// coeffs 1, 2 present?

If (ALF_coeff_12_present_flag) {

ALF_coeff 1			ue(v)		// second ALF coefficient

ALF_coeff 2			ue(v)

}

…

ALF_coeff n			ue(v)		//nth ALF coefficient

Extension_flag			u(v)		// must be zero

Using this syntax, it is trivial to override only those coefficients (and, more abstractly speaking, parameter set data) that are changing, without wasting bits for those pieces of data that are not changing.  Further, changes could possibly be made to more than one parameter set at the same time.

# Proposal #2: Parameter Set maintenance and copy

The conditional replacement mechanism above allows for the efficient modification of an existing parameter set, without retaining a copy of the original.  However, using the loopfilter-example once more, perhaps it makes sense to have more than one adaptively generated set of loopfilter coefficients available.  In this case, and assuming that similar content yields similar (but not necessarily identical) loopfilter coefficients, it may make sense to allow to populate a previously unused parameter set location with data from a different parameter set, so to be able to update it using the “inter” mechanism above.  The parameter set copy command allows for this functionality.

The parameter set copy is probably best implemented using a currently unassigned NAL unit type for “parameter set maintenance” NAL units.  Copy would be one codepoint for this NAL unit type, but there may be more the author has not thought about.

One implementation could look as follows:

Nal unit type “parameter set maintenance”

Number_of_maintainance_items	ue(v)

For (i=0; i<number_of_maintenance_items) {

Command				ue(v)		//maintenance command:

// 0== slice-PS-copy,

// others reserved

source_slice_ps			ue(v)

destin_slice_ps			ue(v)

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).