| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E312 |
| --- | --- |

| Title: | CE12 Subset 2: Cross-Verification of Panasonic’s Parallel Deblocking Contribution (JCTVC-E224) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | G. Van der Auwera, I. S. Chong, and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: geertv@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The cross-verification of Panasonic’s parallel deblocking contribution, JCTVC-E224, confirms Panasonic’s rate-distortion results. The encoding and decoding times are similar with respect to HM2.0.

# Introduction

This document reports the cross-verification results of Panasonic’s parallel deblocking contribution JCTVC-E224 and also JCTVC-D214 (Panasonic), which describes the same method. The method computes deblocking filtering decisions based on reconstructed samples (unfiltered), which separates these decisions from the vertical and horizontal filtering.

# Objective Verification Results

## Rate-Distortion Results

Table 1 enumerates the rate-distortion results as BD-rate differences. The common test conditions are those specified in JCTVC-D600. The conclusion is that the BD-rate differences match those reported by Panasonic in JCTVC-E224.

Table 1 Cross verification results for Panasonic parallel deblocking

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.3 | 0.0 | 0.0 | 0.1 | -0.1 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class E | -0.1 | 0.0 | 0.1 | 0.0 | -0.3 | 0.1 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

## Encoding and Decoding Times

Table 1 reports the encoding and decoding times for Panasonic’s parallel deblocking.

The encodings were executed on a Linux computing cluster consisting of machines with various computing performance and memory configurations. The encoder executable was built using GCC version 4.1.2. The differences in encoding times between our measurements and Panasonic’s are likely attributable to the variation in execution times on our computing cluster.

The decodings were executed on a PC with the following configuration: Windows 7, 6 GB memory, Intel Xeon 2.79GHz (W3530), Microsoft Visual Studio 2008. The decoding times are similar compared to the decoding times of the anchors.

# Source Code Evaluation

The source code provided by Panasonic applies the deblocking filtering on LCU basis. Each LCU quadtree is traversed CU per CU in z-scan order. Each CU is deblocked in three stages, which separate the decisions from the filtering. The first stage computes and stores the decisions for the horizontal edges (luma). The second stage computes decisions for the vertical filtering and executes that filtering (luma and chroma). The third stage executes the horizontal filtering (luma and chroma) by making use of the stored decisions for luma in stage one and the filtered output of stage two. Two buffers of sizes 8×8 and 8×64 (UInt) store the decisions.

# Subjective Quality

The conclusion from an informal viewing on a PC monitor of the sequences specified in Table 2 is that the subjective quality is similar to the corresponding HM2.0 sequences.

Table 2 Informal subjective quality evaluation result

| Class | Sequence | Test Condition | Subjective Quality vs. HM2.0 |
| --- | --- | --- | --- |
| B | Kimono | Low-delay, low-complexity, QP32 | Similar |
| B | Kimono | Random-access, high-efficiency, QP37 | Similar |
| C | BQ_Mall | Low-delay, low-complexity, QP32 | Similar |
| C | BQ_Mall | Random-access, high-efficiency, QP37 | Similar |
| E | Vidyo3 | Low-delay, low-complexity, QP32 | Similar |
| E | Vidyo3 | Low-delay, high-efficiency, QP37 | Similar |

# Conclusion

This cross-verification matches the rate-distortion results reported in Panasonic’s parallel deblocking proposal JCTVC-E224. The encoding and decoding times are similar to those from HM2.0.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).