| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E314 |
| --- | --- |

| Title: | CE12 Subset 2: Cross-Verification of Sony’s Parallel Deblocking Contribution (JCTVC-E181) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | G. Van der Auwera, I. S. Chong, and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: geertv@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The cross-verification of Sony’s parallel deblocking contribution, JCTVC-E181, confirms Sony’s rate-distortion results. The encoding and decoding times are similar to the HM2.0 anchor.

# Introduction

This document reports on the cross-verification results of Sony’s parallel deblocking contribution JCTVC-E181. Two configurations of the source code provided by Sony are verified, referred to by “Item 2” and “Item 3”, with the following macro settings:

- Item 2: “para_only”

  - SONY_DF_FPARA		ON

  - SONY_DF_JPARA 		OFF

  - SONY_DF_LINE_JUDGE 	OFF

- Item 3: “para-only + line judge”

  - SONY_DF_FPARA		ON

  - SONY_DF_JPARA 		OFF

  - SONY_DF_LINE_JUDGE 	ON

In “Item 2”, the horizontal and vertical deblocking filtering is separated while the deblocking decisions are unchanged with respect to HM2.0. In “Item 3”, the deblocking decisions are modified in addition to the changes in “Item 2”.

# Objective Verification Results

## Rate-Distortion Results

Table 1 enumerates the rate-distortion results as BD-rate differences for Sony’s “Item 2”, while Table 2 reports the results for Sony’s “Item 3”. The common test conditions are those specified in JCTVC-D600. The conclusion is that the BD-rate differences for “Item 2” and “Item 3” match those reported by Sony in JCTVC-E181.

Table 1 Cross verification results for Sony’s “Item 2”

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | -0.3 |
| Class D | -0.1 | 0.0 | 0.1 | 0.0 | -0.1 | 0.1 |
| Class E | 0.0 | -0.4 | 0.3 | 0.0 | -0.3 | 0.1 |
| All | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 2 Cross verification results for Sony’s “Item 3”

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | 0.0 | -0.1 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 |
| Class E | -0.2 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 |
| All | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.1 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class B | -0.3 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class C | -0.2 | -0.1 | -0.1 | -0.2 | -0.1 | 0.0 |
| Class D | -0.1 | 0.0 | -0.1 | -0.1 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | 0.0 | 0.0 | -0.2 | -0.1 | 0.0 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.1 | -0.4 | -0.4 | -0.2 | 0.0 |
| Class C | -0.3 | 0.0 | 0.0 | -0.3 | -0.1 | -0.3 |
| Class D | -0.2 | -0.2 | 0.0 | -0.2 | -0.1 | 0.5 |
| Class E | -0.8 | -0.6 | 0.0 | -0.3 | -0.3 | -0.2 |
| All | -0.4 | -0.2 | -0.1 | -0.3 | -0.2 | 0.0 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

## Encoding and Decoding Times

Table 1 and Table 2 report the encoding and decoding times for Sony’s “Item 2” and “Item 3”.

The encodings were executed on a Linux computing cluster consisting of machines with various computing performance and memory configurations. The encoder executable was built using GCC version 4.1.2. The differences in encoding times between our measurements and Sony’s are likely attributable to the variation in execution times on our computing cluster.

The decodings were executed on a PC with the following configuration: Windows 7, 6 GB memory, Intel Xeon 2.79GHz (W3530), Microsoft Visual Studio 2008. Compared to the decoding times of the anchors, the decoding times of Sony’s “Item 2” and “Item 3” are similar.

# Source Code Evaluation

In “Item 2”, Sony implemented the separate vertical and horizontal deblocking on LCU basis. This means that each LCU is first deblocked vertically and then horizontally (luma and chroma). The LCU is traversed CU per CU in z-scan order. In the source code, there is overhead associated with traversing the LCU quadtree twice.

In “Item 3”, in addition to the separation of vertical and horizontal deblocking, Sony modified the decision (luma only) to deblock an edge (length 8 pixels), which in HM2.0 is measured with Laplacians on both sides of the edge at edge positions 3 and 5.  The modification computes the Laplacians at each location along the edge. In summary, the decision to deblock an edge (length 8 pixels) is modified to deciding whether to deblock line by line (luma).

# Subjective Quality

The conclusion from an informal viewing on a PC monitor of the sequences specified in Table 3, is that the subjective quality of “Item 2” and “Item 3” is similar to the corresponding HM2.0 sequences.

Table 3 Informal subjective quality evaluation result

| Class | Sequence | Test Condition | Item 2: Subjective Quality vs. HM2.0 | Item 3: Subjective Quality vs. HM2.0 |
| --- | --- | --- | --- | --- |
| B | Kimono | Low-delay, low-complexity, QP32 | Similar | Similar |
| B | Kimono | Random-access, high-efficiency, QP37 | Similar | Similar |
| C | BQ_Mall | Low-delay, low-complexity, QP32 | Similar | Similar |
| C | BQ_Mall | Random-access, high-efficiency, QP37 | Similar | Similar |
| E | Vidyo3 | Low-delay, low-complexity, QP32 | Similar | Similar |
| E | Vidyo3 | Low-delay, high-efficiency, QP37 | Similar | Similar |

# Conclusion

This cross-verification matches the rate-distortion results reported in Sony’s parallel deblocking proposal JCTVC-E181. The encoding and decoding times are similar compared to HM2.0.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).