| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E317 |
| --- | --- |

| Title: | CE6.e: Nokia report on planar intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Oguz Bici Jani Lainema Kemal Ugur | Email: | ext-oguz.bici@nokia.com jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document presents Nokia results within the core experiment CE6.e on planar intra prediction. Two implementations of planar prediction generating identical bitstreams are introduced: First one is bilinear interpolation as previously introduced. The other one is a multiplication-free implementation by averaging two linear interpolations. The experimented variables aiming for objective improvements consist of bottom-right sample prediction method (DC prediction or averaging the last elements in the reference blocks), planar mode indication (replacing DC mode or indicating separately) and transform coding (DCT or DST). The other experiment aimed for subjective improvement includes CU level indication, signaling bottom-right value information, disabling transform coding and favoring planar mode based on smoothness.

# Introduction

At the JCT-VC meeting in Daegu, in January 2011, Core Experiment 6.e [1] was established to measure performance of different configurations and combinations of the proposed planar intra prediction methods. Three configurations were defined: 1) Planar predictions proposed by the proponents, 2) Prediction error coding (DCT or DST) and 3) Indicating planar mode (Replacing DC mode, indicating planar mode separately or CU level indication). In configuration 1, 1.a and 1.e are based on Nokia’s contribution on planar coding (D326 [2]). In configurations 1.g and 1.h, planar prediction process is same as 1.e except for the prediction of bottom-right sample. In this document, experimental results of 1.a, 1.e, 1.g and 1.h in CE6.e  are presented for various combinations of the variables in configurations 2 and 3.

# Planar prediction

1.a planar prediction is implemented as in D326 document [2] which consists of reconstructing bottom-right sample and applying bilinear interpolation to interpolate inner block samples. Bottom-right sample is set as the DC prediction value of the picture block. 1.e introduces a multiplication-free implementation of 1.a. It is based on average of two linear interpolations similar to the planar prediction implementation in 1.c (D235 [3]). 1.a and 1.e generate identical bitstreams.

1.g prediction is same as the 1.e except for the reconstruction of bottom-right sample process. In 1.g, rather than using the DC prediction for the bottom-right sample, average of the last (farthest) samples in the top and left reference blocks is used. 1.h case is obtained by applying the simplified refinement introduced in 1.f configuration and then applying the 1.h prediction. 1.g and 1.h cases are explained in more detail in the unified planar intra prediction contribution [4].

# Results

1.a and 1.e results are not presented separately as the resultant BD-rate values are identical and 1.e is a less complex implementation. The variables used with the 1.e prediction from the CE description are as follows:

2.a: DCT is used for prediction error coding.

2.b: DST is used for prediction error coding.

3.a: Planar mode is indicated such that DC mode is replaced by the planar mode.

3.b: Planar mode is indicated as a separate mode.

3.c: CU level indication, signaling bottom-right value information, disabling transform coding and favoring planar mode based on smoothness as described in [2]. Different from [2], planar deblocking is disabled. The main goal of 3.c configuration is to achieve subjective improvement, rather than BD-rate. The reconstructed sequences for the 3.c case will be available during the meeting for subjective evaluations.

Results for 1.g and 1.h cases are provided for 2.a and 2.b cases separately. Planar indication as in 3.b configuration is used for all 1.g and 1.h experiments.

The results compared to HM 2.0 anchors are provided in Tables 1 through 9.

Table 1 1.e + 2.a + 3.a

Table 2 1.e + 2.b + 3.a

Table 3 1.e + 2.a + 3.b

Table 4 1.e + 2.b + 3.b

Table 5 1.e + 3.c

Table 6 1.g + 2.a + 3.b

Table 7 1.h + 2.a + 3.b

Table 8 1.g + 2.b + 3.b

Table 9 1.h + 2.b + 3.b

# References

[1] A. Tabatabai, K. Chono, M. Coban, K. Kei, K. Sugimoto, A. Tanizawa “Description of Core Experiment 6 (CE6): Intra Prediction Improvement,” JCT-VC Doc. JCTVC-D606, Daegu, Jan 2011.

[2] J. Lainema, K. Ugur and O. Bici, “Planar intra coding for improved subjective video quality,” JCT-VC Doc. JCTVC-D326, Daegu, Jan 2011.

[3] S. Kanumuri, TK Tan, F. Bossen, “Enhancements to Intra Coding,” JCT-VC Doc. JCTVC-D235, Daegu, Jan 2011.

[4] O. Bici, J. Lainema, K. Ugur, “Unified Planar Intra Prediction,” JCT-VC Doc. JCTVC-E174, Geneva, Mar 2011.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).