| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E319 |
| --- | --- |

| Title: | CE5: Improvements on transform coefficients coding in LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Report |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

In this report, the coding results of improvements on transform coefficients coding in LCEC proposed in JCTVC-D261 under the test conditions of CE5 are reported. First, differentiated tables for run-mode coding in Intra blocks are presented. The BD-rate savings are 0.3% for Intra, 0.1% for random access and 0.1% for low delay. Second, the full coding of 16x16 and 32x32 coefficients are extended and the corresponding results are reported. The BD-rate savings are 2.4% for Intra, 1.5% for random access and 0.5% for low delay.

# Introduction

In HM2.0, the coding for transform coefficients encodes up to 8x8 transform coefficients in LCEC, which means 4x4 and 8x8 blocks are encoded entirely, and 16x16 and 32x32 Transform Units (TU) are truncated to 8x8 coefficients. These coefficients are scanned to 1-D array and encoded in three steps: 1) last nonzero coefficient position, 2) run mode and 3) level mode. In HM2.0 implementation, run mode shares the same mapping for both 8x8 blocks and truncated 8x8 blocks out of 16x16 and 32x32 blocks. The solution in HM2.0 has the following drawbacks: 1) one single model cannot describe the diverse properties of “run” for 8x8, 16x16 and 32x32; 2) Simple truncation in coefficients of large TU is sub-optimal in a RD fashion.

To address the disadvantages of the current HM2.0, solutions are provided in this proposal to address each individual drawback. First, differentiation of sorting tables in terms of TU size is introduced. Second, the encoding of entire TU with size of 16x16 and 32x32 was extended.

# Proposed solution

In the JCT-VC meeting in Daegu, solutions were proposed [1] to improve the transform coding in LCEC.

## Differentiated tables for run-mode

Dedicated mapping tables for Luma of 4x4, 8x8 and 16x16/32x32 are introduced for Intra blocks, while Luma of 4x4, 8x8 and 16x16/32x32 in Inter blocks as well as Chroma use the same mapping as in HM2.0. So, there are 3 more sorting tables used in addition to the single sorting table of HM2.0.

## Full coding of 16x16 and 32x32

In this solution, the coefficient coding of 4x4 and 8x8 blocks remains the same as in HM2.0. Encoding of entire TUs with size of 16x16 and 32x32 is implemented as in [1], following the 3 steps: 1) last nonzero coefficient position, 2) run mode and 3) level mode. The tables for last nonzero coefficient of 16x16 and 32x32 are extended to support large ranges of last nonzero position. For run-mode coding, the number of run-mode tables has been reduced to only one compared to that in [1]. Here it shares one single table for Intra, Inter and Chroma blocks. Level-mode encoding reuses the level encoding from 4x4 and 8x8 blocks.

# Simulation results

## Differentiated tables for run-mode coding

The summary of coding results under CE5 test conditions [2] for Section 2.1 is listed as follows. An average gain of 0.3% for Intra, 0.1% for random access and 0.1% for low delay is reported in Table-1. Detailed results are available in the attached spreadsheet.

Table-1: Results for Differentiated Tables in run-mode coding

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.3 | -0.4 |
| Class B | -0.2 | -0.4 | -0.4 |
| Class C | -0.1 | -0.2 | -0.2 |
| Class D | -0.1 | -0.2 | -0.2 |
| Class E | -0.6 | -0.5 | -0.5 |
| All | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 |
| Class E |  |  |  |
| All | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.1 | 0.0 | 0.2 |
| Class C | -0.1 | 0.0 | -0.1 |
| Class D | 0.0 | -0.1 | 0.3 |
| Class E | -0.1 | 0.0 | 0.5 |
| All | -0.1 | -0.1 | 0.2 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

## Full coding of 16x16 and 32x32

The summary of coding results under CE5 test conditions [2] for Section 2.2 is listed below. On the average, a 2.4% gain for Intra only, 1.4% gain for random access and 0.5% gain for low delay are reported in Table-2. We observed that there are 12.5% and 7.6% gain for Luma of “Nebuta” and “SteamLocomotive” under intra configuration, 7.9% and 3.4% gain for Luma of “Nebuta” and “SteamLocomotive” under random access configuration, respectively. However, we also observed that there is a huge BD-Rate loss in Chroma for these new Class A sequences under random access configuration. The detailed results are available in the attached spreadsheet.

Table-2: Results for Full coding of 16x16 and 32x32 blocks

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -6.1 | 0.6 | 0.2 |
| Class B | -2.0 | -0.9 | -0.7 |
| Class C | -0.7 | -0.4 | -0.3 |
| Class D | -0.9 | -0.3 | -0.6 |
| Class E | -2.3 | -1.3 | -1.4 |
| All | -2.4 | -0.4 | -0.5 |
| Enc Time[%] | 120% |  |  |
| Dec Time[%] | 97% |  |  |
|  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.3 | 11.9 | 9.6 |
| Class B | -1.5 | 0.5 | 0.2 |
| Class C | -0.6 | 0.1 | 0.4 |
| Class D | -0.4 | 0.3 | -0.3 |
| Class E |  |  |  |
| All | -1.5 | 3.0 | 2.3 |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.9 | 1.4 | 1.6 |
| Class C | -0.6 | 0.5 | 0.5 |
| Class D | -0.2 | 0.4 | 0.5 |
| Class E | -0.3 | 1.3 | 1.4 |
| All | -0.5 | 0.9 | 1.0 |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

This report presented the coding results of improvements on coefficient coding in LCEC. The results state that the coding efficiency is improved by the proposed solutions with reasonable cost. Abnormal behaviors for both Luma and Chroma are observed for new Class A sequences. Based on the reported results, we recommend further study on coefficients coding in LCEC, especially for new Class A sequences.

# Reference

- J. Xu, M. Haque, A. Tabatabai, “Improvements on transform coefficients coding in LCEC”, JCTVC-D261, Jan. 2011

- X. Wang, “Low Complexity Entropy Coding Improvements”, JCTVC-D605_r2, Feb. 2011.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).