| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E322_r1 |
| --- | --- |

| Title: | CE5: Cross-verification of Qualcomm’s coefficient coding for LCEC by Sony |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N 1st Street San Jose, CA 95112 | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This report presents cross-verification test results on coefficient coding in LCEC proposed by Qualcomm [JCTVC-E383] under the test environment defined by CE5. The Cross-verification results confirmed the BD-rate performance in JCTVC-E383. Due to different simulation environments, the encoding and decoding time vary.

# Introduction

In Daegu meeting, some improvements in Low Complexity Entropy Coding (LCEC) were reported in various contributions. The Core Experiment 5 (CE5) [1] evaluates all such techniques. In this report, Sony provides the cross-verification results for Qualcomm’s coefficient coding in LCEC [2].  The solution provided includes supporting of full coding of 16x16 and 32x32 Transform Units. In the software for cross-checking, the techniques for 4x4 and 8x8 run-mode mapping for Inter and Chroma are provided to replace the table used in HM2.0. Also, the encoding of last nonzero coefficients is observed to be different from HM2.0 for 8x8 and larger blocks.

# Cross- verification simulations

## Coding conditions

The conditions are the same as specified in JCTVC-D600 for low complexity settings.

## Test set

- The same set as specified in JCTVC-D600 is used.

## Evaluation criteria

The criteria are to report the coding efficiency improvement relative to the reference software and to evaluate the coding complexity.

## Cross-verification Results

The summary of cross-verification results for 32x32 full coding is listed in Table-1. A detailed spreadsheet is attached.

Table-1: Cross-verification results for JCTVC-E383 full coding

|  | Results from Qualcomm |  |  | Results from SONY |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Intra LoCo |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -7.2 | 1.3 | 1.4 | -7.2 | 1.3 | 1.4 |
| Class B | -2.6 | -0.6 | -0.8 | -2.6 | -0.6 | -0.8 |
| Class C | -1.0 | -0.4 | -0.6 | -1.0 | -0.4 | -0.6 |
| Class D | -1.1 | -0.5 | -0.7 | -1.1 | -0.5 | -0.7 |
| Class E | -2.5 | -2.0 | -1.8 | -2.5 | -2.0 | -1.8 |
| All | -2.9 | -0.4 | -0.5 | -2.9 | -0.4 | -0.5 |
| Enc Time[%] | 113% |  |  | 122% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  |  | Random access LoCo |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.6 | 11.9 | 11.9 | -3.6 | 11.9 | 11.9 |
| Class B | -1.1 | 0.5 | 0.1 | -1.1 | 0.5 | 0.1 |
| Class C | -0.5 | 0.2 | 0.1 | -0.5 | 0.2 | 0.1 |
| Class D | -0.3 | 0.2 | -0.1 | -0.3 | 0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -1.4 | 3.0 | 2.8 | -1.4 | 3.0 | 2.8 |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 105% |  |  |
|  | Low delay LoCo |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.2 | 1.7 | 1.6 | -1.2 | 1.7 | 1.6 |
| Class C | -1.1 | 0.5 | 0.1 | -1.1 | 0.5 | 0.1 |
| Class D | -0.5 | 0.0 | 0.2 | -0.5 | 0.0 | 0.2 |
| Class E | -0.9 | 0.5 | 0.7 | -0.9 | 0.5 | 0.7 |
| All | -1.0 | 0.7 | 0.7 | -1.0 | 0.7 | 0.7 |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |

The summary of cross-verification results for 16x16 partial coding is listed in Table-2. A detailed spreadsheet is attached.

Table-2: Cross-verification results for JCTVC-E383 partial coding

|  | Results from Qualcomm |  |  | Results from SONY |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Intra LoCo |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -6.0 | 0.2 | -0.2 | -6.0 | 0.2 | -0.2 |
| Class B | -2.4 | -1.0 | -1.3 | -2.4 | -1.0 | -1.3 |
| Class C | -0.9 | -0.5 | -0.6 | -0.9 | -0.5 | -0.6 |
| Class D | -1.0 | -0.5 | -0.7 | -1.0 | -0.5 | -0.7 |
| Class E | -2.3 | -2.1 | -1.9 | -2.3 | -2.1 | -1.9 |
| All | -2.5 | -0.7 | -0.9 | -2.5 | -0.7 | -0.9 |
| Enc Time[%] | 109% |  |  | 125% |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |
|  |  | Random access LoCo |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.7 | 6.9 | 5.4 | -2.7 | 6.9 | 5.4 |
| Class B | -1.0 | 0.1 | -0.3 | -1.0 | 0.1 | -0.3 |
| Class C | -0.5 | 0.1 | 0.1 | -0.5 | 0.1 | 0.1 |
| Class D | -0.3 | 0.1 | -0.2 | -0.3 | 0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -1.1 | 1.7 | 1.2 | -1.1 | 1.7 | 1.2 |
| Enc Time[%] | 105% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay LoCo |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.0 | 1.3 | 1.1 | -1.0 | 1.3 | 1.1 |
| Class C | -1.0 | 0.3 | 0.1 | -1.0 | 0.3 | 0.1 |
| Class D | -0.5 | 0.1 | 0.0 | -0.5 | 0.1 | 0.0 |
| Class E | -0.9 | -0.5 | 0.0 | -0.9 | -0.5 | 0.0 |
| All | -0.8 | 0.4 | 0.4 | -0.8 | 0.4 | 0.4 |
| Enc Time[%] | 105% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  |

# Conclusion

In this report, the cross-verification results for JCTVC-E383 are provided by Sony. The Coding efficiency in terms of BD-rates matches with the results from Qualcomm. The encoding and decoding time vary due to different simulation environments.

# References

- X. Wang, “Low Complexity Entropy Coding Improvements”, JCTVC-D605_r2, Feb. 2011.

- Qualcomm, “CE5: coefficient coding with LCEC for large block”, JCTVC-E383, Feb. 2011.