| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E323 |
| --- | --- |

| Title: | CE8 Subtest 2: Block based adaptive loop filter (ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | I. S. Chong1 M. Karczewicz1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Tomoo Yamakage3 Takeshi Chujoh3 Takashi Watanabe3 | Emails: | ichong@qualcomm.com martak@qualcomm.com chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com tomoo.yamakage@toshiba.co.jp takeshi.chujoh@toshiba.co.jp takashi39.watanabe@toshiba.co.jp |
| Source: | Qualcomm1 , MediaTek Inc.2, and TOSHIBA Corporation3 |  |  |

_____________________________

# Abstract

This contribution reported the results of CE8 Subtest 2: Block based adaptive loop filter (ALF). A novel pixel classfication method is proposed in this contribution. Frist, block based classification (i.e., 4x4 block based) is proposed instead of the pixel based classification as in the current ALF in HM2.0. Moreover, directional information is harmonized with the Laplacian activity metric based classification method in HM2.0. No syntax changes are made and the pixel classfication method in HM2.0 (i.e., Laplacian based activity metric) is replaced with the method propsed in this contribution. In comparison with the JCTVC-D600 anchor, the proposed method reportedly achieved 0.2%, 0.5%, and 0.4% bit rate reductions for HE-AI, HE-RA, and HE-LD, respectively, with roughly unchanged encoding time (i.e., 2% increase) and 6-8% decoding time reduction. And results of some variants are also provided, i.e., block based ALF without directional harmonization and merging this proposal with MediaTek’s CE8 subset2 proposal. This contribution is cross checked by JCTVC-E109 and E131.

- Introduction

In the previous Daegu metting, during the discussion of JCTVC-D119 [1], block based pixel classfications are considered. Also in JCTVC-D116 [4], directional information combined with activity metric was proposed as a pixel classification method, which produces a noble gain. Therefore, in Core Experiment 8 (CE8) [3] Subtest 2, block based ALF using activity metric combined with directional information was tested.

In the implementation, pixel classfication parts are changed without any syntax changes. The encoding algorithm was kept the same as the anchor. The rest of this document is organized as follows. The proposed pixel classification will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

In the current HM2.0, ALF classifies each pixel into 16 different categories using Laplacian based activity metric. In the newly proposed metric, for block based ALF, Laplacian based activity metrics of 4x4 blocks are averaged, and each 4x4 block has one Laplacian activity value.

Figure 1. Block based activity metric

Additionally directional information is computed using 1D lapalcian activity metric. First vertical/horizontal activity for each block is computed as follows.

Vertical activity = sumi,j| (R(i,j)<<1) –R(i-1,j)-R(i+1,j)| with i,j =0…3

Horizontal activity = sumi,j| (R(i,j)<<1) –R(i,j-1)-R(i,j+1)| with i,j=0…3

Then directional information for each block is decided as follows.

If Vertical activity > threshold*Horizontal activity, Direction = 1

Else if Horizontal activity > threshold* Vertical activity, Direction = 2

Else Direction = 0

Note that Vertical/Horizontal activity computation can be done during Laplacian activity computation, because Laplancian activity is the sum of Vertical and Horizontal activity.

With Direction information, new Activity metric for each block is derived as follows.

Activity Metric = 3*Laplacian Activity + Direction

Now based on this new metric, the same encoding algorithm as in HM2.0 is applied to derive filter coefficients (i.e., merging groups and derive one set of filter coefficients for each merged group). Note that the maximum set of filter coefficients are the same as in HM2.0 (up to 16 set of filters).

Note that coding structures are modified in the decoder side to support CU based pixel classfications, which also can be applied to the current anchor.

# Simulation results

As requested in JCTVC-D608 , the high efficiency (HE) test configurations in JCTVC-D600  were tested. Experimental results are shown below. The proposed block based ALF with new metric can achieve 0.2%, 0.5%, and 0.4% bit rate reductions for HE-AI, HE-RA, and HE-LD, respectively. The encoding time is almost unchanged. The decoding time is reduced by 6-8%. Generally speaking, the proposed method can achieve better compression performance with less decoding time than the anchor. And this contribution is cross checked by JCTVC-E109 and E131.

|  | Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | -0.2 |  | 0.0 |  | 0.0 |
| Class B | -0.2 |  | 0.1 |  | 0.1 |
| Class C | -0.3 |  | 0.2 |  | 0.2 |
| Class D | -0.1 |  | 0.2 |  | 0.2 |
| Class E | -0.2 |  | 0.2 |  | 0.2 |
| All | -0.2 |  | 0.1 |  | 0.1 |
| Enc Time[%] | 102% |  |  |  |  |
| Dec Time[%] | 93% |  |  |  |  |
|  |  |  | Random access |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | -0.5 |  | 0.1 |  | -0.1 |
| Class B | -0.7 |  | 0.0 |  | 0.0 |
| Class C | -0.4 |  | 0.1 |  | 0.1 |
| Class D | -0.2 |  | 0.2 |  | 0.1 |
| Class E |  |  |  |  |  |
| All | -0.5 |  | 0.1 |  | 0.0 |
| Enc Time[%] | 102% |  |  |  |  |
| Dec Time[%] | 92% |  |  |  |  |
|  | Low delay |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |  |
| Class B | -0.4 |  | -0.1 |  | 0.0 |
| Class C | -0.4 |  | 0.2 |  | 0.3 |
| Class D | -0.2 |  | 0.2 |  | -0.4 |
| Class E | -0.5 |  | -0.1 |  | 0.6 |
| All | -0.4 |  | 0.0 |  | 0.1 |
| Enc Time[%] | 102% |  |  |  |  |
| Dec Time[%] | 94% |  |  |  |  |

Additionally, results with block based Laplacian metric is reported as follows.

|  | Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  |  | U BD-rate |  |  |  | V BD-rate |
| Class A | 0.0 |  |  | 0.0 |  |  |  | 0.0 |
| Class B | 0.0 |  |  | 0.0 |  |  |  | 0.0 |
| Class C | 0.2 |  |  | 0.0 |  |  |  | 0.0 |
| Class D | 0.1 |  |  | 0.0 |  |  |  | 0.0 |
| Class E | 0.0 |  |  | -0.1 |  |  |  | -0.1 |
| All | 0.1 |  |  | 0.0 |  |  |  | 0.0 |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 95% |  |  |  |  |  |  |  |
|  |  |  | Random access |  |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  |  |  | V BD-rate |  |
| Class A | 0.2 |  | 0.2 |  |  |  | -0.1 |  |
| Class B | 0.1 |  | 0.1 |  |  |  | 0.1 |  |
| Class C | 0.1 |  | 0.0 |  |  |  | 0.0 |  |
| Class D | 0.1 |  | 0.0 |  |  |  | 0.1 |  |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.1 |  | 0.1 |  |  |  | 0.0 |  |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 95% |  |  |  |  |  |  |  |
|  | Low delay |  |  |  |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  |  | V BD-rate |  |  |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.2 |  | 0.1 |  |  | -0.1 |  |  |
| Class C | 0.1 |  | 0.1 |  |  | 0.1 |  |  |
| Class D | -0.1 |  | 0.1 |  |  | -0.1 |  |  |
| Class E | -0.2 |  | -0.5 |  |  | 0.1 |  |  |
| All | 0.0 |  | 0.0 |  |  | 0.0 |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |  |  |
| Dec Time[%] | 93% |  |  |  |  |  |  |  |

Also efforts are made to merge this proposal with MediaTek’s “CE8 Subtest 2: Adaptation between Pixel-based and Region-based Filter Selection”. Note that those two proposals block based and no syntax changes are made except one slice level flag for switching between region-based and block-based. And the initial number of groups of pixels for block based ALF is reduced from 48 to 16 in this merged version. The results are as follows. Note that we found that results in r2 was not the right one for Intra case, so we fix the table here and uploaded new excel sheet in r3 version.

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | -0.1 | 0.0 |  | 0.0 |
| Class B | -0.1 | 0.1 |  | 0.1 |
| Class C | -0.2 | 0.1 |  | 0.1 |
| Class D | 0.0 | 0.1 |  | 0.1 |
| Class E | -0.1 | 0.1 |  | 0.1 |
| All | -0.1 | 0.1 |  | 0.1 |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  |  | Random access |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | -0.5 | 0.1 |  | 0.0 |
| Class B | -0.7 | 0.1 |  | 0.1 |
| Class C | -0.5 | 0.2 |  | 0.2 |
| Class D | -0.2 | 0.1 |  | 0.1 |
| Class E |  |  |  |  |
| All | -0.5 | 0.1 |  | 0.1 |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 91% |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |
| Class B | -0.9 | -0.1 |  | 0.0 |
| Class C | -0.8 | 0.2 |  | 0.1 |
| Class D | -0.3 | -0.3 |  | -0.1 |
| Class E | -1.3 | 0.1 |  | 0.0 |
| All | -0.8 | 0.0 |  | 0.0 |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 93% |  |  |  |

# Conclusions

In CE8 Subtest 2, Qualcomm tested an block based adaptive loop filter with a novel activity metric which combines Laplacian metric with directional information. It is reported that bit rate reductions are 0.2%, 0.5%, and 0.4% for HE-AI, HE-RA, and HE-LD, respectively. The encoding time is about the same as the JCTVC-D600 anchor, and the decoding time is reduced by 6-8%. It is proposed to adopt this tool into HM and enable it for HE configurations.

# Patent rights declarations

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, T. Yamakage, T. Chujoh, and T. Watanabe, “CE8 Subtest2: A Joint Proposal on Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba,” Document of Joint Collaborative Team on Video Coding, JCTVC-D119, Jan. 2011.

[2] T. Yamakage, Y.-W. Huang, I. S. Chong, T. Ikai, M. Narroschke, W. L. Lai, A. Tabatabai, M. Budagavi, and F. Kossentini, “Description of CE8: Non-deblocking Loop Filtering” Document of Joint Collaborative Team on Video Coding, JCTVC-D608, Jan. 2011.

[3] F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.

[4] T. Ikai and Y. Yasugi, “Region-based adaptive loop filter using two-dimensional feature,” Document of Joint Collaborative Team on Video Coding, JCTVC-D116, Jan. 2011.