| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E324 WG11 Number: m19852 |
| --- | --- |

| Title: | Joint Algorithm-Architecture Optimization of CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Massachusetts Institute of Technology Anantha P. Chandrakasan Massachusetts Institute of Technology | Email: Email: | sze@alum.mit.edu anantha@mit.edu |
| Source: | Massachusetts Institute of Technology |  |  |

_____________________________

# Abstract

To address the increasing demand for higher resolution and frame rates, processing speed and area cost should be considered in the development of HEVC. Accordingly, both algorithm and architecture should be taken into account during video codec design. This contribution proposes joint optimization of both the algorithm and architecture to ensure that high coding efficiency can be achieved in conjunction with high processing speed and low area cost. Specifically, it presents two optimizations that can be performed on CABAC, a form of entropy coding in AVC. First, subinterval reordering is proposed for the arithmetic decoder to increase the processing speed by 14 to 22% with no impact on coding efficiency. Second, modification of the mvd context selection is proposed to reduce memory requirements (i.e. area cost) by 50% with negligible coding efficiency impact (coding gains of 0.007% and 0.028% in HM-2.0).  The results have bee cross-checked by HHI.

# Introduction

Two joint algorithm and architecture optimizations are presented which increase the speed and reduce the area cost of CABAC. The optimizations were implemented on HM-2.0 to measure their coding efficiency impact under common conditions defined by the software AhG.

# Joint algorithm and architecture optimizations

## Subinterval Reordering

In the arithmetic decoder of AVC CABAC, the interval is divided into two subintervals based on the probabilities of the least probable symbol (LPS) and most probable symbol (MPS). The range of MPS (rMPS) is compared to the offset to determine whether the bin is MPS or LPS. rMPS is computed by first obtaining The range of LPS (rLPS) from a 64x4 look up table (LUT) (using bits [7:6] of the current 9-bit range and the 6-bit probability state from the context) and then subtracting it from the current range. Depending on whether an LPS or MPS is decoded, the range is updated with their respective subintervals. To summarize, the interval division steps in the arithmetic decoder are

- obtain rLPS from the 64x4 LUT

- compute rMPS by subtracting rLPS from current range

- compare rMPS with offset for bin decoding decision

- update range based on bin decision.

If the offset was compared to rLPS rather than rMPS, then the comparison and subtraction to compute rMPS can occur at the same time. Furthermore, the updated offset is computed by subtracting rLPS from offset rather than rMPS. Since rLPS is available before rMPS, this subtraction can also be done in parallel with range-offset comparison. Fig. 1 shows the difference between the subinterval order of AVC CABAC and the proposed subinterval reordering. The two orderings of the subintervals are mathematically equivalent in arithmetic coding; thus changing the order has no impact on coding efficiency. This was verified with HM-2.0.

An arithmetic decoder was implemented in RTL for each subinterval ordering. Both implementations were synthesized to obtain their area-delay trade-off in a 45-nm CMOS process. For the same area, subinterval reordering reduces the critical path delay by 14 to 22%.

Fig. 1: Impact of subinterval reordering for CABAC decoding.

Subinterval reordering has similar benefits for the arithmetic encoder of AVC CABAC.  Rather than comparing offset to rLPS or rMPS, the bin to be encoded is compared to MPS. Depending on whether the bin equals MPS, the range is updated accordingly. Switching the order of subintervals allows the bin-MPS comparison to occur in parallel with the rMPS subtraction in the CABAC encoder as shown in Fig. 2. The lower bound can also be updated earlier since it depends on rLPS rather than rMPS.

Fig. 2: Impact of subinterval reordering for CABAC encoding

## Modified Context Selection of MVD

To make use of the spatial correlation of neighboring data, context selection can depend on the values of the top and left blocks as shown in Fig. 3. Consequently, a line buffer is required in the CABAC engine to store information pertaining to the previously decoded row. The depth of this buffer depends on the width of the frame being decoded which can be quite large for high resolution (e.g. 4kx2k) sequences. The bit-width of the buffer depends on the type of information that needs to be stored per block or macroblock in the previous row. We propose reducing the bit-width of this data to reduce the overall line buffer size of the CABAC.

Fig. 3: For position X, context selection is dependent on A and B (4x4 blocks for mvd) a line buffer required to store the previous row of decoded data

Specifically, this contribution proposes modifying the context selection for motion vector difference (mvd) is used to reduce the number of bits required to represent motion information. Rather than transmitting the motion vector, the motion vector is predicted from its neighboring 4x4 blocks and only the difference between motion vector prediction (mvp) and motion vector (mv), referred to as mvd, is transmitted.

mvd = mv - mvp

A separate mvd is transmitted for the vertical and horizontal components. The context selection of mvd depends on neighbors A and B as shown in Fig. 3.  In AVC, neighboring information is incorporated into the context selection by adding a context index increment (between 0 to 2 for mvd) to the calculation of the context index. The mvd context index increment, χmvd, is computed in two steps:

Step 1: Sum the absolute value of neighboring mvd

e(A,B,cmp)=|mvd(A,cmp)|+|mvd(B,cmp)|

where A and B  represent the left and top neighbor and cmp indicates whether it is a vertical or horizontal component.

Step 2: Compare e(A,B,cmp) to thresholds of 3 and 32

χmvd(cmp)=0,if e(A,B,cmp)<3        1,if 3<e(A,B,cmp)≤32 2,if e(A,B,cmp)>32

With the upper threshold set to 32, a minimum of 6-bits of the mvd has to be stored per component per 4x4 block in the line buffer. For 4kx2k, there are (4096/4)=1024 4x4 blocks per row, which implies 6x2x1024=12,228 bits are required for mvd storage.

To reduce the memory size, rather than summing the components and then comparing to a threshold, this contribution proposes separately comparing each component to a threshold and summing their results. In other words,

Step 1: Compare the components of mvd to a threshold

threshA(cmp)=|mvd(A,cmp)|>16

threshB(cmp)=|mvd(B,cmp)|>16

Step 2: Sum the results threshA and threshB from Step 1

χmvd(cmp)=threshA(cmp)+threshB(cmp)

A single threshold of 16 is used. Consequently, only a single bit is required to be stored per component per 4x4 block; the size of the line buffer for mvd is reduced to 1x2x1024=2,048 bits. In AVC, the overall line buffer size of the CABAC required for all syntax elements is 20,480 bits. The modified mvd context selection reduces the memory size by 50%, from 20,480 bits to 10,240 bits.

# Experiment Results

The HM-2.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

The proposed modifications have negligible impact on coding efficiency (coding gain up to 0.03%) as shown in Table 1. Detailed results in HM2-MIT-results.xls

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.000 | 0.000 | 0.000 |
| Class B | 0.000 | 0.000 | 0.000 |
| Class C | 0.000 | 0.000 | 0.000 |
| Class D | 0.000 | 0.000 | 0.000 |
| Class E | 0.000 | 0.000 | 0.000 |
| All | 0.000 | 0.000 | 0.000 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.020 | 0.120 | -0.038 |
| Class B | -0.017 | 0.029 | 0.046 |
| Class C | -0.039 | -0.049 | -0.080 |
| Class D | 0.010 | 0.029 | 0.123 |
| Class E |  |  |  |
| All | -0.007 | 0.032 | 0.014 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.008 | 0.088 | -0.086 |
| Class C | 0.018 | 0.172 | -0.054 |
| Class D | -0.071 | -0.334 | -0.127 |
| Class E | -0.065 | -0.123 | -0.156 |
| All | -0.028 | -0.036 | -0.102 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 106% |  |  |

Table 1: Average coding gain due to subinterval reordering and modified mvd context selection.  Positive numbers in the table means coding loss.

# Text for Working Draft

## Subinterval Reordering

Arithmetic decoding process for a binary decision

Inputs to this process are ctxIdx, codIRange, and codIOffset.

Outputs of this process are the decoded value binVal, and the updated variables codIRange and codIOffset.

Figure 9-3 shows the flowchart for decoding a single decision (DecodeDecision):

- The value of the variable codIRangeLPS is derived as follows.

- Given the current value of codIRange, the variable qCodIRangeIdx is derived by

qCodIRangeIdx =( codIRange >> 6 ) & 3 (9-25)

- Given qCodIRangeIdx and pStateIdx associated with ctxIdx, the value of the variable rangeTabLPS as specified in Table 9-44 is assigned to codIRangeLPS:

codIRangeLPS = rangeTabLPS[ pStateIdx ][ qCodIRangeIdx ]

- The variable codIRangeMPS is set equal to codIRange - codIRangeLPS and the following applies.

- If codIOffset is greater than or equal to codIRangeLPS, the variable binVal is set equal to valMPS,

codIOffset is decremented by codIRangeLPS, and codIRange is set equal to codIRangeMPS.

- Otherwise, the variable binVal is set equal to 1-valMPS, and codIRange is set equal to codIRangeLPS.

Encoding process for a binary decision (informative)

This subclause does not form an integral part of this Recommendation | International Standard.

Inputs to this process are the context index ctxIdx, the value of binVal to be encoded, and the variables codIRange, codILow and BinCountsInNALunits.

Outputs of this process are the variables codIRange, codILow, and BinCountsInNALunits.

Given the current value of codIRange, codIRange is mapped to the index qCodIRangeIdx of a quantised value of codIRange by using Equation 9-25. The value of qCodIRangeIdx and the value of pStateIdx associated with ctxIdx are used to determine the value of the variable rangeTabLPS as specified in Table 9-44, which is assigned to codIRangeLPS. The value of codIRange − codIRangeLPS is assigned to codIRangeMPS.

In a second step, the value of binVal is compared to valMPS associated with ctxIdx. When binVal is same as valMPS, codIRange is added to codILow and codIRange is set equal to the value codIRangeMPS; otherwise, codIRange is set equal to the value codIRangeLPS and codILow is unchanged.   Given the encoded decision, the state transition is performed as specified in subclause 9.3.3.2.1.1. Depending on the current value of codIRange, renormalization is performed as specified in subclause 9.3.4.3. Finally, the variable BinCountsInNALunits is incremented by 1.

## Modified Context Selection of MVD

Derivation process of ctxIdxInc for the syntax elements mvd_l0 and mvd_l1
….

The variable ctxIdxInc is derived as follows

- If (absMvdCompA) is less than 16, and (absMvdCompB) is less than 16, ctxIdxInc is set equal to 0.

- Otherwise, if (absMvdCompA) is greater than 16, and (absMvdCompB) is less than 16, ctxIdxInc is set equal to 1.

- Otherwise, if (absMvdCompA) is less than 16,and  (absMvdCompB) is greater than 16, ctxIdxInc is set equal to 1.

- Otherwise, if ( bsMvdCompA) is greater than 16,and  (absMvdCompB) is greater than 16, ctxIdxInc is set equal to 2.

# Summary

This contribution presents two joint algorithm and architecture optimizations of the CABAC engine, subinterval reordering and modified context selection of mvd, which increase processing speed by 14 to 22% and reduce memory size by 50% with no coding loss. The results have been cross-checked by HHI.  This contribution proposes that these modifications be adopted into the HEVC test model.

# References

- V. Sze, A. P. Chandrakasan, "Joint Algorithm-Architecture Optimization of CABAC to Increase Speed and Reduce Area Cost," IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), to appear May 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

# Patent rights declaration(s)

Massachusetts Institute of Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).