| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 |  |  | Document: JCTVC-E326 WG11 Number: m19854 |  |  |
| --- | --- | --- | --- | --- | --- |
| Title: | CE11: Cross-verification of HHI's (JCTVC-D336) Coding of transform coefficient levels with Golomb-Rice codes |  |  |  |  |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Core Experiment |  |  |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. | Email: |  | sze@ti.com |  |
| Source: | Texas Instruments Inc. |  |  |  |  |

_____________________________

# Abstract

This contribution describes the cross-check results for HHI’s proposal JCTVC-D336.  Two Ver. 2ases were provided (Ver. 1 was made available on the CE11 deadline 2/23/11 and Ver. 2 was made available after the deadline on 3/7/11).  The Ver. 2ases have different threshold values.  The coding efficiency results measured in HM-2.0 with the high efficiency configuration between at QP=22,27,32,37 as specified in the common conditions.  Ver. 1 has on average no impact on coding efficiency (Intra 0.02%, Low Delay 0.03%, Random Access -0.02%).  Ver. 2 a coding efficiency of Intra -0.05%, Low Delay 0%, Random Access 0%.  The bin distribution statistics were also measured.  For both Ver. 1 and Ver. 2, the number of bypass bins increase from Intra 14% to 19%, Low Delay 13% to 15%, Random Access 11% to 12%, with negligible change in total number of bins (between -1% and 0%).  Results match those reported by HHI.

# Experiment Results

The TMuC-0.9 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.08 | 0.01 | 0.04 |
| Class B | 0.02 | 0.00 | -0.01 |
| Class C | 0.01 | 0.00 | 0.01 |
| Class D | -0.03 | -0.08 | -0.07 |
| Class E | 0.01 | 0.02 | -0.04 |
| All | 0.02 | -0.01 | -0.01 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.11 | 0.06 | -0.36 |
| Class B | 0.00 | 0.01 | -0.05 |
| Class C | -0.01 | -0.09 | -0.09 |
| Class D | 0.01 | -0.32 | -0.05 |
| Class E |  |  |  |
| All | 0.03 | -0.08 | -0.13 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.01 | 0.08 | 0.15 |
| Class C | -0.04 | 0.08 | 0.08 |
| Class D | -0.05 | 0.07 | -0.45 |
| Class E | 0.02 | -0.04 | 0.57 |
| All | -0.02 | 0.05 | 0.06 |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 97% |  |  |

Table 1  Coding gains of Ver. 1.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.08 | -0.15 | -0.08 |
| Class B | -0.05 | -0.08 | -0.09 |
| Class C | -0.01 | -0.04 | -0.04 |
| Class D | -0.06 | -0.12 | -0.08 |
| Class E | -0.07 | -0.08 | -0.17 |
| All | -0.05 | -0.09 | -0.09 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.03 | 0.33 | -0.22 |
| Class B | -0.03 | -0.06 | 0.03 |
| Class C | -0.03 | -0.03 | -0.12 |
| Class D | 0.01 | -0.26 | -0.01 |
| Class E |  |  |  |
| All | -0.01 | -0.01 | -0.07 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.01 | 0.25 | -0.13 |
| Class C | 0.01 | -0.01 | 0.00 |
| Class D | -0.05 | -0.09 | -0.35 |
| Class E | -0.20 | -0.85 | 0.82 |
| All | -0.05 | -0.11 | 0.02 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2  Coding gains of Ver. 2.

|  |  |  |  |  |  |  |  | ANCHOR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  |  |  | Intra |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass | % sig | % last | % coeff | % reg | % bypass |
| Class A | -1% | 31% | 15% | 26% | 78% | 22% | 30% | 14% | 27% | 85% | 15% |
| Class B | -1% | 32% | 14% | 23% | 81% | 19% | 32% | 14% | 23% | 86% | 14% |
| Class C | 0% | 28% | 14% | 22% | 82% | 18% | 28% | 14% | 23% | 85% | 15% |
| Class D | -1% | 27% | 15% | 24% | 81% | 19% | 27% | 15% | 24% | 85% | 15% |
| Class E | -1% | 27% | 13% | 22% | 82% | 18% | 27% | 13% | 23% | 87% | 13% |
| All | -1% | 29% | 14% | 23% | 81% | 19% | 29% | 14% | 24% | 86% | 14% |
|  | Random access |  |  |  |  |  | Random access |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 29% | 11% | 17% | 83% | 16% | 28% | 11% | 17% | 86% | 14% |
| Class B | 0% | 28% | 10% | 15% | 84% | 15% | 28% | 10% | 16% | 87% | 13% |
| Class C | 0% | 24% | 10% | 15% | 84% | 15% | 23% | 10% | 15% | 86% | 13% |
| Class D | 0% | 23% | 10% | 15% | 85% | 15% | 23% | 10% | 15% | 86% | 13% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| All | 0% | 26% | 10% | 15% | 84% | 15% | 26% | 10% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  | Low delay |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0% | 29% | 10% | 13% | 87% | 13% | 29% | 10% | 13% | 88% | 11% |
| Class C | 0% | 26% | 9% | 13% | 86% | 14% | 26% | 9% | 13% | 87% | 12% |
| Class D | 0% | 26% | 9% | 12% | 87% | 13% | 26% | 9% | 12% | 88% | 12% |
| Class E | 0% | 23% | 7% | 8% | 88% | 9% | 23% | 7% | 8% | 89% | 9% |
| All | 0% | 27% | 9% | 12% | 87% | 12% | 27% | 9% | 12% | 88% | 11% |

Table 3  Bin distribution of Ver. 1.

|  |  |  |  |  |  |  | ANCHOR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  |  |  | Intra |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass | % sig | % last | % coeff | % reg | % bypass |
| Class A | -1% | 31% | 15% | 26% | 78% | 22% | 30% | 14% | 27% | 85% | 15% |
| Class B | -1% | 32% | 14% | 23% | 81% | 19% | 32% | 14% | 23% | 86% | 14% |
| Class C | 0% | 28% | 14% | 22% | 82% | 18% | 28% | 14% | 23% | 85% | 15% |
| Class D | -1% | 27% | 15% | 24% | 81% | 19% | 27% | 15% | 24% | 85% | 15% |
| Class E | -1% | 27% | 13% | 22% | 82% | 18% | 27% | 13% | 23% | 87% | 13% |
| All | -1% | 29% | 14% | 23% | 81% | 19% | 29% | 14% | 24% | 86% | 14% |
|  | Random access |  |  |  |  |  | Random access |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 29% | 11% | 17% | 83% | 16% | 28% | 11% | 17% | 86% | 14% |
| Class B | 0% | 28% | 10% | 15% | 84% | 15% | 28% | 10% | 16% | 87% | 13% |
| Class C | 0% | 24% | 10% | 15% | 84% | 15% | 23% | 10% | 15% | 86% | 13% |
| Class D | 0% | 23% | 10% | 15% | 85% | 15% | 23% | 10% | 15% | 86% | 13% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| All | 0% | 26% | 10% | 15% | 84% | 15% | 26% | 10% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  | Low delay |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0% | 29% | 10% | 13% | 87% | 13% | 29% | 10% | 13% | 88% | 11% |
| Class C | 0% | 26% | 9% | 13% | 86% | 14% | 26% | 9% | 13% | 87% | 12% |
| Class D | 0% | 26% | 9% | 12% | 87% | 13% | 26% | 9% | 12% | 88% | 12% |
| Class E | 0% | 23% | 7% | 8% | 88% | 9% | 23% | 7% | 8% | 89% | 9% |
| All | 0% | 27% | 9% | 12% | 87% | 12% | 27% | 9% | 12% | 88% | 11% |

Table 4 Bin distribution of Ver. 2.