| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E329 WG11 Number: m19857 |
| --- | --- |

| Title: | Removal of cabac_zero_word for error detection/resilience |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yasutomo Matsuba Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA | Email: Email: Email: | matsuba@ti.com sze@ti.com zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The decoder can detect an error in the slice layer by comparing the number decoded bits for the slice with the current NAL unit size. A mismatch indicates that there is an error in the decoded bitstream. In AVC, the encoder inserts a variable number of cabac_zero_words in the slice layer, after end_of_slice_flag=1, which changes the number bits per NAL unit and the above method of error detection cannot be directly used.  Before doing a comparison, the decoder must also parse through the bytes after end_of_slice_flag=1 to ensure that they match the cabac_zero_word pattern.  Furthermore, cabac_zero_word can cause false alarms for error detection (i.e. when bytes up to and including end_of_slice_flag=1 are correct, but an error occurs in cabac_zero_word).  This contribution proposes removing cabac_zero_word from the rbsp_trailing_bits(), and recommends inserting the stuffing bytes outside of the slice_layer (e.g. place a startcode delimiter between slice layer and stuffing bytes).

# Impact of cabac_zero_word in error detection

Error detection is necessary as bit errors can occur during transmission. One method of error detection in the slice layer is to compare the number decoded bits for the slice with the current NAL unit size. In AVC [1], the encoder inserts a variable number of cabac_zero_words in the slice_data, after end_of_slice_flag=1, which changes the number bits per NAL unit and the above method of error detection cannot be directly used.  Before doing a comparison, the decoder must also parse through the bytes after end_of_slice_flag=1 to ensure that they match the cabac_zero_word pattern.  Furthermore, cabac_zero_word can cause false alarms for error detection (i.e. when bytes up to and including end_of_slice_flag=1 are correct, but an error occurs in cabac_zero_word).

# Recommendation to remove cabac_zero_word from the rbsp_slice_trailing_bits()

This contribution proposes the removal cabac_zero_word in HEVC syntax.  From the HEVC working draft [2]:

rbsp_slice_trailing_bits( ) {

rbsp_trailing_bits( )

if( entropy_coding_mode_flag )

while( more_rbsp_trailing_data( ) )

cabac_zero_word /* equal to 0x0000 */

}

Removal of cabac_zero_word enables a simple method of error detection by comparing the number decoded bits for the slice with the current NAL unit size.  It also prevents false alarms which can be caused by errors being detected in the cabac_zero_word; the slice will be deemed corrupted even if there are no errors before end_of_slice_flag=1.

# Summary

In this contribution proposal to remove cabac_zero_word from rbsp_slice_trailing_bits() in the HEVC syntax. This can improve both error detection and error resilience in HEVC.

# References

- ITU-T Recommendation H.264 (03/2009), Advanced video coding for generic audiovisual services

- JCTVC-D503, HEVC Specification Working Draft 2 (WD2)

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).