| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E330 WG11 Number: m19858 |
| --- | --- |

| Title: | Reduced neighboring dependency in context selection of significant_coeff_flag for parallel processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-567-4005 sze@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution proposes reducing the neighboring dependencies in the context selection of significant_coeff_flag for positions in the top-most row and left-most column of a transform (i.e. the top and left edge). While the simplified context selection method in JCTVC-D260 (adopted for HM-2.0), eliminates the neighboring dependency on the most recently decoded significant_coeff_flag within a diagonal, this contribution eliminates the dependency when wrapping from one diagonal to another during zig-zag scan. This modification was implemented in HM-2.0 and its coding efficiency was evaluated on HM-2.0 with a coding loss 0.1% for Intra, Low delay and Random Access in high efficiency.  This contribution has been cross-checked by Sony (JCTVC-E382).

# Introduction

For improved coding efficiency, HM-2.0 uses a highly adaptive context selection approach for significant_coeff_flag in the significance map, where the context selection can depend on the value of the coefficients of the neighbouring positions (typically in the left, top-left and top directions) in the transform.  A context selection method was proposed in JCTVC-D260 (adopted for HM-2.0) that eliminates the neighboring dependency to the diagonal neighbours (as shown in Fig. 1), which is typically the most recently decoded significant_coeff_flag.  However, when the processing significant_coeff_flags that are near the edge of the transform, the most recently decoded bin may not be the diagonal neighbour, but the immediate top or left neighbour (as shown in Fig. 2).  To minimize speculative computations for parallel bin decoding, it would be desirable to avoid dependency on the previously decoded bin.

B

E

F

H

I

X

Fig.1 – Simplified context selection for significant map in HM-2.0.  Context for position X depends on B, E, F, H and I.

…

…

Fig.2 – When processing the significant_coeff_flag in positions highlighted in blue (every other position in first row and every other position in first column), there is dependency on previously decoded bin highlighted in grey. These dependencies occur at the edges of the transform.  This dependency will be a challenge for parallel decoding of two bins (or more) at a time.

# Simplified Context Selection for Significance Map

To enable parallel processing of the bins, this contribution proposes reducing the context dependencies for the significant_coeff_flag in positions near the edge of the transform.  The dependency on I (the immediate left neighbor) is removed for positions in the first row, and dependency on F (the immediate top neighbor) is removed for positions in the first column.  Fig. 3 shows these modifications.

First column: Context selection not dependent on F. Note: H, E, and I do not exist for these positions.

First row: Context selection not dependent on I. Note: B, E, and F do not exist for these positions.

B

E

F

H

oesX

B

E

H

I

X

Fig.5 – For parallel processing of at least two bins, remove the dependency to the immediate top neighbour for the positions in the first column; also remove the dependency to the immediate left neighbour for the positions in the first row.

Note dependency removal in Fig.3 is helpful even when attempting to decode more than 2 bins in parallel.  As shown in contribution JCTVC-D244, the number of speculative computation grows exponentially with the number of parallel bins.  The dependency removal in Fig. 3 would delay the start of the exponential growth by one position (i.e. it could potentially half the number of speculative computations).

# Experiment Results

The TMuC-0.9 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

Removing dependency for 2-bin parallelism.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A | 0.2 | 0.1 | 0.2 |
| Class B | 0.2 | 0.2 | 0.2 |
| Class C | 0.1 | 0.1 | 0.1 |
| Class D | 0.1 | 0.0 | 0.1 |
| Class E | 0.2 | 0.3 | 0.3 |
| All | 0.1 | 0.1 | 0.2 |
| Enc Time[%] |  |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random access |  |  |
|  | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A | 0.2 | 0.1 | -0.2 |
| Class B | 0.1 | 0.1 | 0.0 |
| Class C | 0.1 | 0.0 | 0.0 |
| Class D | 0.1 | 0.1 | 0.1 |
| Class E |  |  |  |
| All | 0.1 | 0.0 | 0.0 |
| Enc Time[%] |  |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A |  |  |  |
| Class B | 0.1 | 0.2 | 0.2 |
| Class C | 0.1 | 0.2 | 0.1 |
| Class D | 0.0 | -0.1 | -0.4 |
| Class E | 0.1 | -0.3 | 0.7 |
| All | 0.1 | 0.0 | 0.1 |
| Enc Time[%] |  |  |  |
| Dec Time[%] | 99% |  |  |

Table 1 Average loss from reducing dependency at the edge of the transform to help enable parallel processing. Positive numbers in the table means coding loss.

# Text for Working Draft

Note: no text has been written for ctxIdxInc derivation in working draft 2

Derivation process of ctxIdxInc for the syntax elements significant_coeff_flag

….

The variable ctxIdxInc is derived as follows

If levelListIdx % TransformSize equals 0, top neighbour (levelListIdx- TransformSize ) is set to 0 for ctxIdxInc calculation.

If levelListIdx less than TransformSize, left neighbour (levelListIdx -1) is set to 0 for ctxIdxInc calculation.

# Summary

This contribution presents a modification to the context selection of significant_coeff_flag by reducing the dependency at position near the edge of the transform, where dependencies are to the top and left rather than diagonal. The dependency reduction results in an average coding loss of 0.1% for Intra, 0.1% for Low delay and 0.1% for Random Access.

# Patent rights declaration(s)

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).