| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E332 |
| --- | --- |

| Title: | CE13: Cross-Verification of MediaTek’s adaptive offset (JCTVC-E049) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | I. S. Chong and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-845-5477 Email: ichong@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports verification results of JCTVC-E049 for CE13.  Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Encoding is almost the same as proponent’s data and decoding time is 3~4% different. Considering the difference of operating the environments, results are confirmed.

# Introduction

This contribution is a report of verification results of JCTVC-E049, which is a contribution to CE13. Verification is performed under the CE13 test conditions that refer to JCTVC-D600.  In addition, source code is reviewed.

# Experimental Results

Our encoding environment is using 64bit linux cluster and decoding was done on sinlge 64 bit windows PC. There was no mismatch between encoder and decoder. There is no mismatch of overall BD-rate between the verification results and the proponent’s results. Two cases (i.e., Case 1 and Case 2) are tested and results are reported as follows.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class D | -0.3 | 0.1 | 0.1 | -0.3 | 0.1 | 0.1 |
| Class E | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| All | -0.2 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 104% |  |  | 108% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | 0.2 | 0.0 | -0.4 | -0.1 | 0.0 |
| Class B | -0.2 | 0.1 | 0.1 | -0.2 | 0.1 | 0.1 |
| Class C | -0.2 | 0.0 | 0.1 | -0.2 | 0.1 | 0.1 |
| Class D | -0.6 | 0.2 | 0.2 | -0.4 | 0.2 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | 0.1 | 0.1 | -0.3 | 0.1 | 0.1 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.2 | -0.3 | -0.4 | 0.0 | 0.2 |
| Class C | -0.3 | 0.1 | -0.1 | -0.4 | 0.0 | -0.1 |
| Class D | -0.7 | 0.4 | 0.4 | -0.5 | 0.4 | 0.6 |
| Class E | -1.0 | -0.2 | 0.1 | -0.7 | 0.2 | 0.2 |
| All | -0.5 | 0.2 | 0.0 | -0.5 | 0.1 | 0.2 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 107% |  |  | 109% |  |  |

Table 1. Adaptive offset Case 1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.0 | 0.0 | -0.5 | 0.0 | 0.0 |
| Class B | -0.3 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
| Class C | -0.6 | 0.0 | 0.0 | -0.5 | 0.0 | 0.0 |
| Class D | -0.4 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
| Class E | -0.5 | 0.0 | 0.0 | -0.6 | 0.0 | 0.0 |
| All | -0.4 | 0.0 | 0.0 | -0.4 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 106% |  |  | 112% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | 0.1 | 0.0 | -2.0 | -0.5 | -0.6 |
| Class B | -1.0 | -0.1 | -0.1 | -1.2 | -0.1 | -0.1 |
| Class C | -1.1 | -0.1 | -0.1 | -0.7 | -0.3 | -0.2 |
| Class D | -0.8 | -0.1 | 0.0 | -0.1 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | 0.0 | 0.0 | -1.0 | -0.2 | -0.2 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 110% |  |  | 109% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | -0.2 | -0.2 | -1.4 | 0.5 | 0.7 |
| Class C | -1.9 | 0.2 | -0.1 | -1.9 | 0.0 | -0.3 |
| Class D | -1.6 | 0.3 | 0.3 | -0.9 | 0.3 | 0.5 |
| Class E | -0.6 | -0.2 | 0.8 | -0.1 | 1.0 | 1.4 |
| All | -1.3 | 0.0 | 0.1 | -1.2 | 0.4 | 0.5 |
| Enc Time[%] | 101% |  |  | 105% |  |  |
| Dec Time[%] | 110% |  |  | 110% |  |  |

Table 2. Adaptive offset Case 2

Encoding is almost the same as proponent’s data and decoding time is 3~4% different. Considering the difference of operating the environments, results are confirmed.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).