| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E334 WG11 Number: m19862 |
| --- | --- |

| Title: | CE8.1: Cross-verification of proposal on deblocking parameter adjustment for 1-input ALF (E193) by Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution is a part of the core experiment on non-deblocking loop filtering (CE8) and it cross-verifies proposal E193 (Toshiba/MediaTek/Qualcomm) on deblocking parameter adjustment for 1-input ALF. The adjustments to the deblocking parameters for configurations using ALF are sent in the slice header. The proposed technique reportedly results in average BD-rate reduction of 0.6% for three high efficiency configurations. The results are confirmed by the cross-verifier. The subjective video quality on a majority of the studied sequences is reportedly similar to that of the anchor except two cases when the proposal is subjectively worse than the anchor.

# Software and cross-verification

The software has been compiled successfully with gcc 4.3.2 on Linux and Visual Studio for Windows.

## Algorithm and syntax changes

It is proposed in E193 [1] to adjust deblocking tables for configurations using ALF. This is achieved by sending offsets for Tc and Beta values in the slice header. The current deblocking filtering in HM2.0 uses beta values for the filtering decision over the block boundary and tc for clipping (to limit the amount of filtering). The clipping parameter tc is read from the table as tc[QP + tcOffset] and beta as beta[QP], where tcOffset is equal to 4 when one of the blocks adjacent to the block boundary is INTRA and 0 otherwise.

The following changes are proposed in E193 to the values of tc and beta.

tc[QP + tcOffset + new_tc_offset] and

beta[QP + tcOffset + new_beta_offset].

The proposed offsets new_tc_offset and new_beta_offset are sent in the slice header if the loop filter is enabled. The encoder in E193 reads the offsets from the configuration file. The following parameters are used in the simulations.

LoopFilterNewBetaOffset : -2

LoopFilterNewTcOffset : -4

# Objective performance

The objective performance of the cross-verified method is shown in the table below. The decoding time on a single computer is reported in the next section. The scheme has been compared against the HM2.0 anchor for three high-efficiency configurations. The comparison results are reported in Table 1. The BD-rate [3] reduction is -0.6% on average for three high-efficiency configurations. The obtained results perfectly match the results provided by the proponents.

Table 1 Comparison of E193 vs HM2.0 as anchor

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | 0.0 | 0.0 |
| Class B | -0.8 | 0.0 | 0.0 |
| Class C | -0.6 | 0.0 | 0.0 |
| Class D | -0.5 | 0.0 | 0.0 |
| Class E | -0.8 | 0.0 | 0.0 |
| All | -0.7 | 0.0 | 0.0 |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | 100% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | 0.0 | -0.1 |
| Class B | -0.6 | 0.0 | 0.0 |
| Class C | -0.3 | 0.0 | 0.0 |
| Class D | -0.2 | 0.0 | 0.2 |
| Class E |  |  |  |
| All | -0.5 | 0.0 | 0.0 |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.6 | -0.2 | -0.4 |
| Class C | -0.2 | 0.3 | -0.1 |
| Class D | -0.3 | 0.5 | 0.2 |
| Class E | -1.1 | -0.4 | -0.1 |
| All | -0.5 | 0.1 | -0.1 |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | 100% |  |  |

# Complexity considerations

In order to evaluate the complexity changes of the E193, the decoding time on a single machine has been measured for both the proposal and the anchor. The time has been obtained without writing decoded pictures on the disk in order to minimize the influence of the I/O time.

Table 2 Decoding time of E193 vs HM2.0 decoding time

| Configuration | Dec. time proposal [s] | Dec. time HM2.0 [s] | Dec. time proposal [%] |
| --- | --- | --- | --- |
| Random access | 3598.15 | 3588.59 | 100.3% |
| Low delay | 2513.90 | 2514.19 | 100.0% |
| All intra | 7240.00 | 7235.87 | 100.1% |
| Average |  |  | 100.1% |

Table 2 reports that the cross-verified method has similar complexity as HM2.0 anchor.

# Subjective performance

The subjective quality of proposal E193 has been evaluated. A number of sequences coded with the low-delay and high-efficiency configurations at QP 32 and 37 have been compared to the anchor. The proposal and the anchor were repeated a few times one after another without knowing the identities. After deciding which one is better, the identities were revealed.

Most of the sequences were similar to the anchor when watched in motion with very small differences between the anchor and proposal (it was difficult to decide if the proposal or the anchor is better). Two sequences were subjectively worse than the anchor. Those were Vidyo3 and Vidyo4 coded with the low-delay configuration at QP37. In Vidyo3, the anchor looks cleaner (e.g., it leaves less noise around the letters and the drawings on the white board). In Vydyo4 the proposal has more dragging after the moving objects and leaves a color artifact on a chair after the person.

# References

- T. Yamakage, S. Asaka, T. Watanabe, T. Chujoh, C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, "CE8.1: Deblocking parameter adjustment for 1-input ALF", JCTVC-E193, Geneva, Mar. 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C600, JCT-VC Meeting, Daegu, Jan. 2010.

- Gisle Bjontegaard, “Calculation of average PSNR differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.