| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E338 |
| --- | --- |

| Title: | CE11: Parallel Context Processing for the significance map in high coding efficiency |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Sole, R. Joshi, M. Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-845-2429 Email: joels@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In JCTVC-D262, a technique for the parallelization of context processing was proposed. The method separates syntax elements of the significance map to facilitateparallel processing. Significance coefficient flags and last significant coefficient flags are separated by encoding the position of the last significant coefficient before the position of the other significant coefficients within a block. The position of the last coefficient is encoded explicitly by signaling its X and Y coordinates with a unary code.

This contribution is part of CE11 and shows the results of the technique in HM2.0. The BD-rate for the high efficiency intra, random access, and low-delay configurations is -0.03℅, 0.01℅, and  -0.10℅, respectively. For chroma, the BD-rate is -0.02% for intra, -0.85% for random access and -1.90% for low delay.

- Introduction

Parallelization of significance map was proposed in JCTVC-D262 [1]. This contribution analyses JCTVC-D262 within the scope of CE11.

The significance map encoding in HM2.0 is carried out by interleaving the bin indicating the significance of a coefficient (significant_coeff_flag)  and the bin indicating whether the coefficient is the last significant within the block (last_significant_coeff_flag) when the flag significant_coeff_flag equals 1. The drawback of this approach is that the decision regarding the encoding of the last_significant_coeff_flag for a particular transform coefficient is dependent on the value of the significant_coeff_flag. This makes the encoding of the significance map difficult to parallelize [2].

## Algorithm description

JCTVC-D262 proposed a separation of the encoding of the significant_coeff_flag and the last_significant_coeff_flag. This is accomplished by first encoding the position of the last significant coefficient. Once the last nonzero coefficient is determined, then the significant_coeff_flags are coded. The position of the last coefficient in the block is coded by explicitly signaling its (X, Y)-coordinates. ‘X’ indicates the column number and ‘Y’ the row number (see Fig. 1). Both coordinates are coded independently as a unary code starting from position 0 and moving to the next column (or row) until the column (or row) of the last coefficient is met. The unary number is encoded with CABAC and for each block size the context depends solely on the column (or row) number.

Figure 1. Explicit encoding of X and Y coordinates for the last significant coefficient

- Performance Measurement

The proposed tool has been implemented in HM2.0. In this section, performance measurements as specified in core experiment 11 [3] are provided.

## Coding Performance Measurements

Simulation results are presented for the full sequences with high efficiency common test conditions. Table 1 shows the results. The BD-rate is slightly better than the anchor, being -0.03℅, 0.01℅ and -0.10℅ for intra, random access and low delay configurations, respectively (negative numbers indicate BD-rate gains). BD-rate for chroma of the proposal is -0.02% for intra,-0.85% for random access and -1.90% for low delay. Detailed BD-rate and BD-PSNR results are provided in an accompanying excel file.

Table 1. BD-rate for the proposal (high efficiency)

| BD-rate | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.17 | -0.22 | -0.14 | -0.07 | -1.10 | -0.92 |  |  |  |
| Class B | -0.07 | 0.06 | 0.07 | -0.02 | -0.62 | -0.46 | -0.07 | -1.36 | -1.74 |
| Class C | 0.11 | 0.10 | 0.06 | 0.07 | -0.94 | -0.83 | -0.06 | -1.51 | -1.40 |
| Class D | -0.06 | -0.05 | 0.10 | 0.05 | -1.15 | -0.86 | -0.19 | -2.87 | -3.07 |
| Class E | 0.09 | -0.05 | -0.18 |  |  |  | -0.11 | -2.26 | -1.06 |
| All | -0.03 | -0.03 | -0.01 | 0.01 | -0.94 | -0.75 | -0.10 | -1.94 | -1.86 |
| Enc T[%] | 100% |  |  | 100% |  |  | 98% |  |  |
| Dec T[%] | 100% |  |  | 100% |  |  | 100% |  |  |

## Complexity Measurements

### Encoding and Decoding Times

Regarding complexity, no meaningful difference in encoding and decoding times are observed with respect to the anchor. Table 1 shows the percentage of coding time. Detailed results are also provided in the accompanying excel file.

### Bin Counts

As per the CE11 requirements, the following bin counts are provided:

- The total number of bins and bin reduction/increase due to proposed technique.

- A breakdown of the number of bins coded in regular and bypass mode for CABAC.

- A breakdown of the number of bins for sig flag, last flag and absolute coefficient level.

Table 2 summarizes the relevant bin counts for this proposal, while an accompanying excel file provides the full counts. The proposed method reduces the average total number of bins by 5% for Intra, and 2% for random access.

Table 2. Average bin counts of the proposal with respect to the anchor

| % bins | Total | Signif | Last |
| --- | --- | --- | --- |
| Intra | 95% | 94% | 77% |
| Random access | 98% | 96% | 93% |
| Low delay | 100% | 96% | 115% |

### Parallelization Aspects

This method benefits parallelization in the following ways:

- It benefits the implementation of parallel algorithms by the separation of the significant coefficient and the last coefficient planes (see Fig. 2; for more detail compare to Fig. 9 in [2]).

- Additionally, the X and Y component are encoded independently, and thus, allowing further parallelization within the coding of the last coefficient position.

- Parallelization and speculative computation of the significant coefficients become easier because the stopping point (i.e., last significant coefficient of the block) is known beforehand.

- The worst-case scenario for the number of bins for the last flag is reduced. For an N×N block, the maximum number of bins for the last flag goes from N*N-1 down to 2*N-2 bins.

  - For 32×32 blocks, the worse case is reduced from 1023 bins to 62.

  - The average case is also reduced (table 2).

In HM2.0, the significance map represents, on average, 43% of the bins for intra, 35% for random access and 36% for low delay. These elements represent the highest percentage of bins and they are encoded interleaved. By the proposed method, both elements are partitioned allowing parallelization and an increase in throughput due to the elimnation of this bottleneck, as shown in table 3.

Figure 2. Parallelization of context processing for significance map

Table 3. Distribution of the bins for the significance map

| bin distribution | PCP SigMap |  | HM2.0 |
| --- | --- | --- | --- |
|  | Sig | Last | Sig + Last |
| Intra | 29% | 11% | 43% |
| Random Access | 26% | 9% | 36% |
| Low Delay | 26% | 10% | 36% |

### Contexts

The proposed method doesn’t use the current contexts for the last flags (16 contexts per block size). Instead, it uses contexts for the X and Y position depending on the column and row number and on the block size. The number of contexts is distributed as follows:

- 4×4: 6 contexts (3 for X and 3 for Y)

- 8×8: 14 contexts (7+7)

- 16×16: 24 contexts (12+12)

- 32×32: 32 contexts (16+16)

For the significance flag, the current contexts are re-used if the last significant coefficient is after the 10th coefficient in scanning order. If the last coefficient is before the 10th, then a context depending on the position is used, i.e., 9 additional contexts are used per block size.

- Draft Text

This section describes the algorithm to parse the position of the last significant coefficient in more detail. The contexts for the last position depend on the row/column number. The context is derived using the following table:

g_uiCtxXY[ 31 ] =

{0, 1, 2, 3, 4, 5, 6, 7, 8, 8, 9, 9, 10, 10, 11, 11, 12, 12, 12, 12, 13, 13, 13, 13, 14, 14, 14, 14, 15, 15, 15};

Note also that each block size has its own set of contexts. X and Y do not share same contexts.

1. Parse x location (truncated unary code):

1.1 Set x=0

1.2 For x=0...Width-2

1.2.1 Decode bin LAST using g_uiCtx[x] context for x

1.2.2 If( LAST=0 )  x++

1.2.3 Otherwise, go to 1.3

1.3 If( LAST=0 ) x++

2. Parse y location (truncated unary code):

2.1 Set y=0

2.2 For y=0...Width-2

2.2.1 Decode bin LAST using g_uiCtx[y] context for y

2.2.2 If( LAST=0 )  y++

2.2.3 Otherwise, go to 2.3

2.3 If( LAST=0 )  y++

3. If ( ScanType = Vertical ) swap(x,y)

4. Set (x,y) coefficient as significant

5. Set context type for significance map

5.1 If Last Coefficient (x,y) is in the first 10 coefficients in scan order

5.1.1 Set ctxLow=1

5.2 Otherwise, set ctxLow=0

6. For scan_position = (0,0)...(x,y)-1

6.1 Encode significance of coefficient

6.1.1 If ctxLow=1, use context depending on scan_position

6.1.2 Otherwise, use regular context (currently in HM2.0)

- Conclusions

An algorithm for parallelization of the significance map encoding is proposed. The parallelization advantage of the algorithm comes at no performance cost. The luma BD-rate of the algorithm is -0.03℅, 0.01℅ and -0.10℅ for intra, random access and low delay configurations, respectively.

- References

- J. Sole, R. Joshi, I.-S. Chong, M. Coban, M. Karczewicz, “Parallel Context Processing for the significance map in high coding efficiency”, JCTVC-D262, 4th JCT-VC Meeting, Daegu, KR, January 2011.

- M. Budagavi, M. U. Demircin, “Parallel context processing techniques for high coding efficiency entropy coding in HEVC”, JCTVC-B088, 2nd JCT-VC Meeting, Geneva, CH, July 2010.

- V. Sze et al, “CE11: Coefficient Scanning and Coding”, JCTVC- D611, 4th JCT-VC Meeting, Daegu, KR, January 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).