| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E339 WG11 Number: m19867 |
| --- | --- |

| Title: | On num_reorder_frames and max_dec_frame_buffering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg KI/EAB/TVV/V 164 80 Stockholm Sweden | Tel: Email: | +46761153563 rickard sjoberg@ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document discusses max_dec_frame_buffering and num_reorder_frames and proposes to move them from the optional VUI parameter part of the sequence parameter set and instead make them mandatory for every sequence parameter set.

# Proposal

The details of max_dec_frame_buffering and num_reorder_frames are not yet present in Working Draft 2 but looking in the AVC specification the following can be found:

MaxDpbFrames is calculated from the picture sizes by the following formula:

MaxDpbFrames = Min( MaxDpbMbs / ( PicWidthInMbs * FrameHeightInMbs ), 16 )

where MaxDpbMbs is given by the level. It limits the amount of memory a decoder needs to allocate to decode the bitstream.

max_dec_frame_buffering is part of the VUI parameters and is therefore an optional codeword in the SPS. It can be used to lower the MaxDpbFrames value. It has no normative impact on the decoding process but it informs the decoder that it does not need to allocate the maximum number of DBP frames as given by the MaxDpbFrames. If this codeword is not present, the value of it is inferred to be equal to MaxDpbFrames.

num_reorder_frames is also part of the VUI parameters. It is used to signal to the decoder the maximum number of frames that precede any frame in decoding order but follow it in display order. In other words, it tells the decoder how big the picture interleaving depth is. If this codeword is not present, the value of it is inferred to be equal to MaxDpbFrames.

Consider a low delay encoding of BQSquare which is a 60Hz 416x240 sequence. Assume that the encoder uses one reference picture and that the display order is the same as decoding order for all pictures. A 416x240@60Hz sequence has a MaxMBPS of 23400 and would therefore need AVC level 3. MaxDpbFrames then becomes 16. If max_dec_frame_buffering or num_reorder_frames is not set, they are both inferred to 16. This means that the decoder needs to allocate 16 picture buffers in order to be sure that it can decode the bitstream correctly. In order to achieve a sometimes nice decoder property to return one picture to display per decoded picture twice the number of picture buffers is needed to be prepared for worst case. The amount of redundant memory allocation and the increase of decoding delay caused by not signaling max_dec_frame_buffering and num_reorder_frames is huge.

We therefore propose to make both these codewords mandatory in the HEVC SPS.

## AVC specification

The specifications of num_reorder_frames and max_dec_frame_buffering are here cited for convenience

| seq_parameter_set_data( ) { | C | Descriptor |
| --- | --- | --- |
| <snip> |  |  |
| vui_parameters_present_flag | 0 | u(1) |
| if( vui_parameters_present_flag ) |  |  |
| vui_parameters( ) | 0 |  |
| } |  |  |

| vui_parameters( ) { | C | Descriptor |
| --- | --- | --- |
| <snip> |  |  |
| bitstream_restriction_flag | 0 | u(1) |
| if( bitstream_restriction_flag ) { |  |  |
| motion_vectors_over_pic_boundaries_flag | 0 | u(1) |
| max_bytes_per_pic_denom | 0 | ue(v) |
| max_bits_per_mb_denom | 0 | ue(v) |
| log2_max_mv_length_horizontal | 0 | ue(v) |
| log2_max_mv_length_vertical | 0 | ue(v) |
| num_reorder_frames | 0 | ue(v) |
| max_dec_frame_buffering | 0 | ue(v) |
| } |  |  |
| } |  |  |

num_reorder_frames indicates the maximum number of frames, complementary field pairs, or non-paired fields that precede any frame, complementary field pair, or non-paired field in the coded video sequence in decoding order and follow it in output order. The value of num_reorder_frames shall be in the range of 0 to max_dec_frame_buffering, inclusive. When the num_reorder_frames syntax element is not present, the value of num_reorder_frames value shall be inferred as follows.

- If profile_idc is equal to 44, 86, 100, 110, 122, or 244 and constraint_set3_flag is equal to 1, the value of num_reorder_frames shall be inferred to be equal to 0.

- Otherwise (profile_idc is not equal to 44, 86, 100, 110, 122, or 244 or constraint_set3_flag is equal to 0), the value of num_reorder_frames shall be inferred to be equal to MaxDpbFrames.

max_dec_frame_buffering specifies the required size of the HRD decoded picture buffer (DPB) in units of frame buffers. The coded video sequence shall not require a decoded picture buffer with size of more than Max( 1, max_dec_frame_buffering ) frame buffers to enable the output of decoded pictures at the output times specified by dpb_output_delay of the picture timing SEI messages. The value of max_dec_frame_buffering shall be greater than or equal to max_num_ref_frames. An upper limit for the value of max_dec_frame_buffering is specified by the level limits in subclauses A.3.1, A.3.2, G.10.2.1, and H.10.2.

When the max_dec_frame_buffering syntax element is not present, the value of max_dec_frame_buffering shall be inferred as follows.

– If profile_idc is equal to 44, 86, 100, 110, 122, or 244 and constraint_set3_flag is equal to 1, the value of max_dec_frame_buffering shall be inferred to be equal to 0.

– Otherwise (profile_idc is not equal to 44, 86, 100, 110, 122, or 244 or constraint_set3_flag is equal to 0), the value of max_dec_frame_buffering shall be inferred to be equal to MaxDpbFrames.

## Proposed working draft text

We propose to move num_reorder_frames and max_dec_frame_buffering from the VUI parameters and place them last in seq_parameter_set_rbsp(), right before rbsp_trailing_bits().

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| <snip> |  |
| num_reorder_frames | ue(v) |
| max_dec_frame_buffering | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

We propose the following NAL unit semantics in section 7.4.2.1 of the WD:

num_reorder_frames indicates the maximum number of frames that precede any frame in the coded video sequence in decoding order and follow it in output order. The value of num_reorder_frames shall be in the range of 0 to max_num_ref_frames, inclusive.

max_dec_frame_buffering specifies the required size of the HRD decoded picture buffer (DPB) in units of frame buffers. The coded video sequence shall not require a decoded picture buffer with size of more than Max( 1, max_dec_frame_buffering ) frame buffers to enable the output of decoded pictures at the output times specified by dpb_output_delay of the picture timing SEI messages. The value of max_dec_frame_buffering shall be greater than or equal to max_num_ref_frames.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).