| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E342 |
| --- | --- |

| Title: | Adaptive Loop Filtering Using Two Filter Shapes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Hsan Guermazi Nader Mahdi Mohammed Ali BenAyed | Email: | faouzi@ebriskvideo.com hsan@ebriskvideo.com nader@ebriskvideo.com medali@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This contribution presents the advantages of utilizing two different filter shapes in the Adaptive Loop Filtering part (that is applied to the luminance samples) of the HM-2.0 encoder/decoder. More specifically, we show that the simultaneous use of a 7×7 modified-diamond shape and a 13×7 cross shape yields good complexity-performance trade-offs.

# Introduction

In the HM-2.0 implementation [1], 5×5 and 7×7 diamond-shaped symmetrical filters, as well as 9×7 modified-diamond-shaped symmetrical filters, are used in the Adaptive Loop Filtering (ALF) of the reconstructed luminance samples [2]. While all such filters are limited to seven lines, they can still be costly in terms of both side information and computations. For example, the 9×7 modified-diamond-shaped symmetrical filter requires the use of 20 coefficients. Since the HM-2.0 encoder is allowed to send to the decoder as many as 16 filters, as many as 320 coefficients may be required to be sent to the decoder. Moreover, as shown in Figure 1, the ALF of a single sample may require as many as 20 multiplications and 39 additions (38 additions + one addition for the offset) when a 9×7 modified-diamond-shaped symmetrical filter is employed. Finally, the use of three different filter sizes (namely 5×5, 7×7 and 9×7) can be computationally demanding at the encoder.

Figure 1 – Filtering using a 9×7 modified-diamond-shaped symmetrical filter

# Proposed Technique

In ALF, for a given filter shape, there is a fundamental relationship between the filter size (i.e., number of filter coefficients) and the resulting complexity-performance trade-offs. By decreasing the filter size, the encoding/decoding process will be less computationally demanding, however, the coding efficiency can also decrease substantially. On the other hand, increasing the filter size will generally yield an increase in both computational requirements and coding efficiency. The filter size that will yield the best complexity-performance trade-offs will vary depending on video content and other parameters. Therefore, the HM-2.0 encoder/decoder allows the use of three different sizes (namely, 5×5, 7×7 and 9×7).

However, by using different shapes, we are able to use a single size for each considered shape, while maintaining good complexity-performance trade-offs. In this document, we propose an ALF technique that allows the use of 5×7 modified-diamond-shaped and 13×7 cross-shaped filter sets (see Figure 2), where each filter set can include as many as 16 distinct filters (as in the case of the HM-2.0). Since the proposed 5×7 diamond-like-shaped and 13×7 cross-shaped symmetrical filters employ only 10 coefficients, the proposed ALF technique requires a maximum of 10 multiplications and 19 additions (18 additions + one addition for the offset) for the ALF filtering of each luminance sample of the reconstructed picture (Figure 3 & Figure 4). Note that the numbers of lines of these two candidates (i.e., numbers of coefficients in the vertical direction) are also equal to 7, maintaining memory requirements that are similar to those of the HM 2.0.

Figure 2 – Diamond-like-shaped & cross-shaped symmetric filters

Figure 3 Filtering using a 5×7 diamond-like-shaped symmetrical filter

Figure 4 - Filtering using a 13×7 cross-shaped symmetrical filter

The HM-2.0 ALF algorithm consists of three steps as shown in Figure 5. Steps 1 and 2 are performed using a set of 5×5 diamond-shaped filters. In Step 3, the 7×7 and 9×7 modified-diamond-shaped filters are tested.

Figure 5 – HM 2.0 technique

In the proposed technique, Step 3 is removed and Step 1 is applied twice as shown in Figure 6. Note that the first step can be executed simultaneously using a set of 5×7 diamond-like-shaped filters and a set of 13×7 cross-shaped filters. In other words, the first step could be performed in parallel, since there is no dependency between the two paths.

After applying Step 1 for each of the two shapes, a decision between the two filtered frames is made and the candidate that gives the best rate-distortion trade-off will be considered. Therefore, Step 2 will be performed using the winner; either the 5×7 diamond-like-shaped filters or the 13×7 cross-shaped filters.

Figure 6 - Proposed technique

In this proposed technique, the new ALF filtering is applied to only the luminance component, and the same HM-2.0 ALF filtering is applied to the chrominance components.

# Simulation Results

The simulation experiments were conducted following the testing conditions in JCTVC-D600. The coding efficiency (BD-Rate) numbers were obtained for the Low Delay (Table 1), Random Access (Table 2) and Intra Only (Table 3) High-Efficiency profiles. The detailed luminance results for all video sequences are shown in Table 4.

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core 2 Quad Q8300 @2.50 GHz.

- RAM: 4 Go

- OS:  Windows XP Professional.

Our proposal has been cross verified by TI and the BD rates matched exactly. Run-times are slightly different (3% better at encoder and 1% better at decoder: see numbers in blue).

Table 1 – Simulation Results: Low Delay (High-Efficiency)

|  | Low delay |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.4 | 0.1 | 0.0 |
| Class C | 0.1 | 0.0 | 0.0 |
| Class D | -0.2 | -0.3 | -0.6 |
| Class E | 0.6 | -0.7 | 0.3 |
| All | 0.2 | -0.2 | -0.1 |
| Enc Time[%] | 100% (95%) |  |  |
| Dec Time[%] | 93% (94%) |  |  |

Table 2 – Simulation Results: Random Access (High-Efficiency)

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.0 | 0.0 |
| Class B | 0.4 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | -0.1 |
| Class D | -0.2 | 0.1 | 0.1 |
| Class E |  |  |  |
| All | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 101% (94%) |  |  |
| Dec Time[%] | 94% (88%) |  |  |

Table 3 – Simulation Results: Intra Only (High-Efficiency)

|  | Intra Only |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.0 | 0.0 |
| Class B | 0.2 | 0.0 | 0.0 |
| Class C | 0.4 | -0.1 | -0.1 |
| Class D | 0.0 | -0.1 | -0.1 |
| Class E | 0.5 | -0.1 | -0.1 |
| All | 0.3 | -0.1 | -0.1 |
| Enc Time[%] | 99% (103%) |  |  |
| Dec Time[%] | 87% (90%) |  |  |

Table 4 – Detailed Results (Luminance Only)

|  | Sequences | Intra Only | Random Access | LowDelay |
| --- | --- | --- | --- | --- |
| S01 | Traffic | 0.3 | 0.3 |  |
| S02 | PeopleOnStreet | 0.4 | 0.2 |  |
| S0X | Nebuta | 0.4 | 0.4 |  |
| S0Y | SteamLocomotive | 0.2 | 0.2 |  |
| S03 | Kimono | 0.1 | -0.1 | 0.1 |
| S04 | ParkScene | 0.3 | 0.1 | 0.0 |
| S05 | Cactus | 0.2 | 0.2 | 0.3 |
| S06 | BasketballDrive | 0.3 | 0.5 | 0.7 |
| S07 | BQTerrace | 0.4 | 1.0 | 1.1 |
| S08 | BasketballDrill | 1.5 | 1.0 | 1.1 |
| S09 | BQMall | 0.2 | -0.1 | -0.1 |
| S10 | PartyScene | 0.0 | -0.5 | -0.3 |
| S11 | RaceHorses | 0.0 | -0.3 | -0.3 |
| S12 | BasketballPass | 0.1 | 0.1 | 0.2 |
| S13 | BQSquare | 0.1 | -0.8 | -0.4 |
| S14 | BlowingBubbles | -0.1 | -0.2 | -0.4 |
| S15 | RaceHorses | 0.0 | 0.0 | -0.2 |
| S16 | Vidyo1 | 0.5 |  | 0.5 |
| S17 | Vidyo3 | 0.6 |  | 0.8 |
| S18 | Vidyo4 | 0.3 |  | 0.5 |
| Average | Class A | 0.3 | 0.3 |  |
|  | Class B | 0.4 | 0.4 | 0.4 |
|  | Class C | 0.0 | 0.0 | 0.1 |
|  | Class D | 0.0 | -0.2 | -0.2 |
|  | Class E | 0.5 |  | 0.6 |
|  | All | 0.3 | 0.1 | 0.2 |

In order to demonstrate the ability of the proposed technique in decreasing worst-case decoder complexity, an evaluation against the highest-complexity HM-2.0 (i.e., where the ALF employs only 9×7 modified-diamond-shaped filters) has been conducted on Class D & Class C sequences, and in this case, we found that our proposed technique reduces the average decoding time by approximately 15 % (Table 5).

Table 5 – Decoding times in the worst case

|  | Dec Time[%] – Class D | Dec Time[%] – Class C |
| --- | --- | --- |
| Low Delay | 83% | 80% |
| Random access | 80% | 83% |
| Intra Only | 89% | 84% |
| Average | 84% | 82% |

# Syntax and semantics

There are no changes in the syntax elements. In fact, realfiltNo_flag is used to identify the filter shape. In HM-2.0, this flag is used to specify the filter tap and since the filter tap decision has been removed, this flag has been exploited to indicate the selected filter.

realfiltNo_flag: 1 means that cross-shaped filters are used; 0 means that diamond-like-shaped filters are used.

# Conclusions

The proposed technique employs both diamond-like-shaped and cross-shaped filters. Our simulation experiments show that the proposed technique does not have a big impact on the coding efficiency while decreasing significantly the decode times.

# Reference

[1]	 https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-2.0/

[2] 	M. Karczewicz, P. Chen, R. Joshi, X. Wang, W. Chien, R. Panchal, “Video coding technology  proposal by Qualcomm Inc.”, JCTVC-A121, April 2010.

[3]	 T. Chujoh, N. Wada, T. Watanabe, G. Yasuda and T. Yamakage, “Specification and experimental results of Quadtree-based Adaptive Loop Filter”, ITU-T Q.6/SG16 Doc., VCEG-AK22, Yokohama, April 2009.

[4]	 http://iphome.hhi.de/suehring/tml/download/KTA/jm11.0kta2.7.zip

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).