| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E344 |
| --- | --- |

| Title: | Context reduction of the last transform position in JCTVC-D262 for CE11.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
|  | Wei Liu 1730 N. First Street, MD:3NW San Jose, CA 95112 USA |  | 1-408-352-4712 wei.liu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

The JCTVC-D262 is being studied in CE11.1 for the coding of the position of the last transform coefficient. This contribution reduces the number of context for coding the last transform coefficients proposed in JCTVC-D262 from 120 to 82 for YUV 4:2:0 video and with the same BD performance. The result of this contribution is being cross-checked by Panasonic.

# Introduction

In JCTVC-D262 [2], the position of the last significant coefficient is encoded before the position of the other significant coefficients within a block for better parallelization and throughput. The position of the last coefficient is encoded explicitly by signaling its X and Y coordinates with a truncated unary code. The X and Y signaling are independent. The luminance and the chrominance signaling are independent. The contexts for different block sizes are also independent. Consequently, for YUV 4:2:0 video, a total of 120 contexts are used for coding the last position.

This contribution proposes a different assignment of the contexts in JCTVC-D262 to reduce the number of contexts from 120 to 82 and with the same BD performance.

# Context Assignment of the Last Position in JCTVC-D262

In JCTVC-D262, the X and Y context assignment are the same. For simplicity, the assignment of X for luminance is presented here.

As shown in Figure 1, the luminance X coordinate is encoded with the following contexts:

- For block width of 4, it is binarized with a maximum of 3 bits by truncated unary code, and it has 3 contexts. Each bin has its context.

- For block width of 8, it is binarized with a maximum of 7 bit by truncated unary code, and it has 7 contexts. Each bin has its context.

- For block width of 16, it is binarized with a maximum of 15 bits by truncated unary, and it has 12 contexts. The bins with the same context number in Figure 1 share the same context.

- For block width of 32, it is binarized with a maximum of 31 bits by truncated unary code, and it has 16 contexts. The bins with the same context number in Figure 1 share the same context.

The luminance X coordinate has a total of 3+7+12+16=38 contexts. For YUV 4:2:0 video, the largest block width for chrominance is 16, and it has a total of 3+7+12=22 contexts.

Figure 1: Context number of a coordinate of the last position in JCTVC-D262. For each block width, the coordinate is binarized by truncated unary code, and the bins with the same context number shared one context. It has 38 contexts for a luminance coordinate.

Figure 2: Proposed context number of a coordinate of the last position. It has 26 contexts for a luminance coordinate.

# Proposed Context Assignment for Context Reduction

To reduce the number of context for coding a coordiante of the last position, the context of the bins in Figure 1 with orange color is replaced by the other contexts. The new context assignment is shown in Figure 2. It has 3+5+7+11=26 contexts for one coordiante of the luminance last position.

As shown in Figure 2, the context assignment for the width of 4 is the same as in JCTVC-D262. For the width of 8, the context 4 and 6 are removed from JCTVC-D262. For the width of 16, the context 4, 6, 8, 10, 11 are removed from JCTVC-D262. Finally, for the width of 32, the context, 4, 6, 8, 10, 15 are also removed from JCTVC-D262.

# Comparison of BD Performance

As shown in Figure 3, there is virtually no difference in BDR between JTCVC-D262 and the proposed context assignments when compared with HM2.0.

Figure 3: There is virtually no difference in BD-rate between the proposed and JCTVC-D262 when compared with HM2.0.

# Comparison of Bin Counts

The bin statistics required by CE11 [1] were also collected. Figure 4 compares the bin statistics of the proposed with respect to JCTVC-D262. There were virtually no difference between the proposed and JCTVC-D262. Also, the bin statistics of the proposed and the HM2.0 are shown in Figure 5.

|  |  |  | PROPOSED |  |  |  |  |  |  | JCTVC-D262 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  |  |  |  | Intra |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |  | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 32% | 9% | 30% | 84% | 16% |  | 32% | 9% | 30% | 84% | 16% |
| Class B | 0% | 33% | 11% | 24% | 85% | 15% |  | 33% | 11% | 24% | 85% | 15% |
| Class C | 0% | 27% | 13% | 24% | 85% | 15% |  | 27% | 13% | 24% | 85% | 15% |
| Class D | 0% | 26% | 13% | 25% | 84% | 16% |  | 26% | 13% | 25% | 84% | 16% |
| Class E | 0% | 26% | 11% | 24% | 86% | 14% |  | 26% | 11% | 24% | 86% | 14% |
| All | 0% | 29% | 11% | 25% | 85% | 15% |  | 29% | 11% | 25% | 85% | 15% |
|  | Random access |  |  |  |  |  |  | Random access |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |  | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 29% | 8% | 18% | 85% | 14% |  | 29% | 8% | 18% | 85% | 14% |
| Class B | 0% | 28% | 9% | 16% | 87% | 13% |  | 28% | 9% | 16% | 87% | 13% |
| Class C | 0% | 23% | 10% | 15% | 86% | 13% |  | 23% | 10% | 15% | 86% | 13% |
| Class D | 0% | 22% | 10% | 15% | 87% | 13% |  | 22% | 10% | 15% | 87% | 13% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| All | 0% | 26% | 9% | 16% | 86% | 13% |  | 26% | 9% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  |  | Low delay |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |  | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0% | 29% | 10% | 13% | 88% | 11% |  | 29% | 10% | 13% | 88% | 11% |
| Class C | 0% | 25% | 10% | 13% | 88% | 12% |  | 25% | 10% | 13% | 88% | 12% |
| Class D | 0% | 25% | 11% | 12% | 88% | 12% |  | 25% | 11% | 12% | 88% | 12% |
| Class E | 0% | 22% | 8% | 8% | 89% | 8% |  | 22% | 8% | 8% | 89% | 8% |
| All | 0% | 26% | 10% | 11% | 88% | 11% |  | 26% | 10% | 11% | 88% | 11% |

Figure 4: Comparison of bin statistics between the proposed and JCTVC-D262.

|  |  |  | PROPOSED |  |  |  |  |  |  | ANCHOR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  |  |  |  | Intra |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |  | % sig | % last | % coeff | % reg | % bypass |
| Class A | -7% | 32% | 9% | 30% | 84% | 16% |  | 30% | 14% | 27% | 85% | 15% |
| Class B | -4% | 33% | 11% | 24% | 85% | 15% |  | 32% | 14% | 23% | 86% | 14% |
| Class C | -5% | 27% | 13% | 24% | 85% | 15% |  | 28% | 14% | 23% | 85% | 15% |
| Class D | -5% | 26% | 13% | 25% | 84% | 16% |  | 27% | 15% | 24% | 85% | 15% |
| Class E | -5% | 26% | 11% | 24% | 86% | 14% |  | 27% | 13% | 23% | 87% | 13% |
| All | -5% | 29% | 11% | 25% | 85% | 15% |  | 29% | 14% | 24% | 86% | 14% |
|  | Random access |  |  |  |  |  |  | Random access |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |  | % sig | % last | % coeff | % reg | % bypass |
| Class A | -4% | 29% | 8% | 18% | 85% | 14% |  | 28% | 11% | 17% | 86% | 14% |
| Class B | -1% | 28% | 9% | 16% | 87% | 13% |  | 28% | 10% | 16% | 87% | 13% |
| Class C | -2% | 23% | 10% | 15% | 86% | 13% |  | 23% | 10% | 15% | 86% | 13% |
| Class D | -1% | 22% | 10% | 15% | 87% | 13% |  | 23% | 10% | 15% | 86% | 13% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| All | -2% | 26% | 9% | 16% | 86% | 13% |  | 26% | 10% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  |  | Low delay |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |  | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0% | 29% | 10% | 13% | 88% | 11% |  | 29% | 10% | 13% | 88% | 11% |
| Class C | 0% | 25% | 10% | 13% | 88% | 12% |  | 26% | 9% | 13% | 87% | 12% |
| Class D | 1% | 25% | 11% | 12% | 88% | 12% |  | 26% | 9% | 12% | 88% | 12% |
| Class E | 1% | 22% | 8% | 8% | 89% | 8% |  | 23% | 7% | 8% | 89% | 9% |
| All | 0% | 26% | 10% | 11% | 88% | 11% |  | 27% | 9% | 12% | 88% | 11% |

Figure 5: Comparison of bin statistics between the proposed and the HM2.0 anchor.

# Conclusion

JCTVC-D262 provided a set of tools to parallelize context processing and to improve the throughput of the entropy coding of the significance map of the transform coefficients for the high efficiency case. To reduce the complexity of JCTVC-D262, this contribution proposed a context assignment scheme to reduce the number of contexts for coding the last position of the transform coefficients from 120 to 82 for the YUV 4:2:0 video.

This contribution recommends the proposed context reduction technique be adopted in conjunction with JCTVC-D262 for HM 3.0.

# References

- Vivienne Sze, Krit Panusopone, Jianle Chen, Tung Nguyen, Muhammed Coban, “Description of Core Experiment (CE11): Coefficient scanning and coding”, JCTVC-D611, 4th Meeting: Daegu, KR, 20-28 January, 2011.

- J. Sole, R. Joshi, I. S. Chong, M. Coban, M. Karczewicz, “Parallel context processing  for the significance map in high coding efficiency”, JCTVC-D262, 4th Meeting: Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).