| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E297 |
| --- | --- |

| Title: | CE3: Cross-verification of Sony’s Luma Interpolation Filter Contribution (JCTVC-E128 and case 0 of JCTVC-E129) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo, In Suk Chong Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8125 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reoprts our cross-verification results of Sony’s CE3 luma interpolation filter contribution (JCTVC-E128 and case 0 of JCTVC-E129). For both contribution, the coding efficiency have been comfired. Relative encoding and decoding time have some degree of deviation. Considering the difference of operating the environments, results are basically confirmed.

# Introduction

This contribution is a report of verification results of JCTVC-E128 and JCTVC-E129. The test conditions are specified in JCTVC-d603

# Experimental Results

Our encoding environment is using 64bit linux cluster and decoding was performed on a sinlge 64 bit windows PC. The RD data exactly match contribution JCTVC-E128 and case 0 of JCTVC-E129.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.3 | -0.9 | -0.1 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 | -0.6 | -0.3 | -0.4 |
| Class C | -0.1 | -0.3 | -0.3 | -0.4 | -0.3 | -0.3 |
| Class D | 0.1 | -0.4 | -0.3 | -0.3 | -0.6 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.1 | -0.6 | -0.3 | -0.3 |
| Enc Time[%] | 95% |  |  | 96% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.3 | 0.3 | -0.7 | 0.1 | 0.0 |
| Class C | -0.2 | 0.1 | 0.0 | -0.9 | -0.2 | 0.1 |
| Class D | 0.1 | 0.3 | 0.6 | 0.0 | 0.1 | 0.6 |
| Class E | 0.0 | -0.1 | 0.5 | -0.7 | 0.7 | 1.1 |
| All | 0.0 | 0.2 | 0.3 | -0.6 | 0.1 | 0.4 |
| Enc Time[%] | 98% |  |  | 93% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 1 Cross-check results of JCTVC-E128

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.0 | 0.1 | -0.1 | 0.0 |
| Class B | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 95% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.2 |
| Class C | -0.1 | 0.1 | -0.1 | -0.2 | -0.1 | -0.2 |
| Class D | -0.3 | -0.5 | -0.5 | -0.3 | 0.3 | 0.3 |
| Class E | 0.2 | 0.2 | -0.1 | 0.1 | 0.3 | 0.2 |
| All | -0.1 | -0.1 | -0.2 | -0.2 | 0.1 | 0.1 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 2 Cross-check results of JCTVC-E129 case 0

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).