| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-24 March, 2011 | Document: JCTVC-E353_r1 WG11 Doc: m19881 |
| --- | --- |

| Title: | CE10: Fast Core Transforms, Proposal 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Dai Pankaj Topiwala Ilkoo Kim | Email: | daisy@fastvdo.com pankaj@fastvdo.com ilkoo.kim@samsung.com |
| Source: | FastVDO LLC, Samsung |  |  |

_____________________________

# Abstract

The current HM (2.0) utilizes integer transforms of sizes 4-pt to 32-pt, designed using Chen's factorization of the DCT, which provide good decorrelation performance and moderate complexity.  This contribution highlights the introduction of new integer transforms and several variants, which offer substantial computational savings with effectively no performance loss.  The current 4-pt and 8-pt transforms are initially retained, while the more complex 16-pt and 32-pt transforms are systematically simplified, even including multiplication-free designs. The multiplication-free designs afford the lowest complexity as well as exact invertibility in limited bitdepth arithmetic, yet have identical performance.   The transform itself was developed jointly with Samsung, and previously presented in the joint proposal, D365.  A related proposal, also jointly developed, is presented in E277.

# Introduction

Recently, integer transforms for 16×16, and 32×32 sizes based on Chen’s fast algorithm were proposed and implemented in the current HM 0.9 design. While Chen’s factorization has fully recursive and regular butterfly structure which is convenient to hardware design, it is not as efficient as the algorithm based on Loeffler’s [3, 4]. New integer transforms based on Loeffler’s factorization have been developed, jointly with Samsung, which considerably reduce the complexity of current large size transforms. The complexity of the transforms was substantially reduced by introduced lifting-based implementations, and especially by using a multiplication-free design [4].  These designs provide identical performance to the designs with multiplications, but provide dramatically lower complexity, and exact invertibility in low bitdepth arithmetic.  For these reasons, we advocate the multiplication-free designs for adoption in the HM.  The design of an optimized quantization structure suitable for this transform design remains to be developed.  We therefore propose a CE to study and finalize the designs of the fast transforms and appropriate quantization.  We refer to the joint proposal with Samsung, D365, for details.

# New integer transforms with lifting scheme

Fundamental transforms used in image/video coding applications must be invertible; with the recent trend to fixed bitwidth integer transforms, the design of good transforms with invertibility in low bitdepth arithmetic is challenging. One convenient solution to achieve perfect reconstruction in bit-limited arithmetic is to apply the lifting scheme [4, 7].  Many excellent transforms can be constructed this way, including excellent approximations of the DCT (in any dimension).

Essentially, while a typical signal flow representation of a DCT (or any standard unitary transform such as the DFT, or DST) is in terms of butterflies, scalings, and 2D rotations, the rotations can be replaced with a set of 3 lifting steps each, with scaling factors related to the rotation angles [4].  Once rendered in this format, the coefficients (scalings, almost always irrational) can be replaced with more convenient rational numbers for ease of computation.  As the rational numbers form a dense subfield of the real line, one can create an approximation of the original transform to arbitrary accuracy, and therefore decorrelation performance. Furthermore, one can even choose the rational coefficients to be dyadic rational, numbers of the form {m/2^n}, where the denominator is only a power of two.  In fact, the dyadic rational numbers are also a dense subset in the real line (this time only a subring, but not a subfield) [8].  Density of the dyadic rationals on the real line implies the density of matrices with dyadic rational coefficients in the space of all real matrices of dimension n (GL_n(R)).  So again, we can find approximations of arbitrary accuracy of the given transform.

Note that while the original (“ideal”) transform such as the true DCT may have been unitary (in this case, orthogonal), the approximations so obtained will not generally be orthogonal; but they will be biorthogonal, and exactly invertible. It may appear that sacrificing orthogonality could lead to a loss in decorrelation performance, thus in utility in video coding. In fact, it has been observed in several settings (wavelets, lapped transforms) that biorthogonal transforms can actually outperform the orthogonal transforms, see for example [9, 10].  The lifting approach allows one to conveniently design biorthogonal transforms for coding applications.  It also allows one to reduce or even eliminate the number of multiplications, and moreover, some scaling operations can be absorbed in the quantization and dequantization step. More detailed complexity analysis will be given in the next section.  Naturally, the follow-on steps of scaling and quantization need to be tailored to the selected transform, which remains to be developed. We therefore recommend a CE in this topic area to develop and finalize a complete design for recommendation to the JCTVC.

# Complexity analysis

The lifting scheme decomposes a plane rotation into 3 lifting steps, which requires 3 multiplications and 3 additions instead of 4 multiplications and 2 additions for one single rotation.  Even though the total number of operations is the same as 6, this leads to fewer multiplications.  In fact, the beauty of this method is that multiplications can then be eliminated completely. Moreover, some of operations can be integrated into the quantization process, which leads to much less complexity. Number of operations required to compute 2-D inverse transforms is shown in Table 1.  Note that for large transforms, thousands of multiplications can be fully eliminated.  This is especially noteworthy if one considers the opposite extreme – a fully matrix-multiplication approach, as in Table 3 (e.g., 32K mults for 32-pt transform).

Table 1 Number of operations for inverse transforms: Chen vs Loeffler

|  | 16x16 Chen transform (now in HM 0.9) | 16x16 New Integer transform | 16x16 New Integer Transform +lifting | 16x16 New Integer Transform +lifting +Mult-free | 32x32 Chen transform (now in HM 0.9) | 32x32 New integer transform | 32x32 New integer +lifting | 32x32 New integer +lifting +Mult-free |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mults | 1408 | 1344 | 768 | 0 | 7424 | 6656 | 4224 | 0 |
| Adds | 2368 | 2304 | 2496 | 3776 | 12416 | 11904 | 12928 | 22976 |
| Shift | 832 | 768 | 864 | 2784 | 4224 | 3712 | 4480 | 17152 |
| Total | 4608 | 4416 | 4128 | 6560 | 24064 | 22272 | 21632 | 40128 |

Fast transforms with butterfly structure have numerous advantages over matrix multiplication implementations. In fact, matrix multiplication represents the opposite extreme from the multiplication-free designs that we advocate. The number of multiplications and additions required to compute 2-D inverse transforms is shown in Table 2.  Comparing to table 1, we see that the fast, mult-free approach in fact can save some 32K mults for every 32-pt transform, compared to a matrix multiplication approach. In simple terms, a matrix-multiplication implementation incurs 32 multiplies for every sample in the video, whereas the multiplication-free approach incurs no multiplications at all, just adds and shifts (even the total operations count is much smaller).  From an operations complexity analysis, matrix-multiplication appears unfavorable for implementation on generic architectures.

Table 2 Operations count comparing to matrix-multiplication approach

|  | 16x16 matrix mult | 16x16 Chen Transf. | 16x16 Loeffler Transf. | 16-pt Mult-Free Transf. | 32x32 matrix mult. | 32x32 Chen Transf. | 32x32 Loeffler Transf. | 32-pt Mult-free Transf. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mults | 4096 | 1408 | 1344 | 0 | 32768 | 7424 | 6656 | 0 |
| Adds | 3840 | 2368 | 2304 | 3776 | 31774 | 12416 | 11904 | 22976 |
| Shifts | 0 | 832 | 768 | 2784 | 0 | 4224 | 3712 | 17152 |
| Total | 7936 | 4608 | 4416 | 6560 | 64542 | 24064 | 22272 | 40128 |

# Experimental results

While the computational complexity of the mult-free design is significantly lower than other designs, the performance of the mult-free approach is identical to the new integer transform design with multiplications as previously introduced in the joint Samsung / FastVDO proposal D365.  For this cycle, the only change we have made is adding clipping, according to D071, in order to ensure 16-bit bus / intermediate storage within the computational framework.

Results for the Normal QP range per common conditions are reported below.  These results have been cross-checked successfully (matched) by Motorola Mobility, E356. Results on additional ranges will be reported in an update.

# Conclusions

The designs using lifting offer flexibility in operating with multiplications or  multiplication-free, and offer the greatest savings in complexity, while providing excellent performance. An appropriate quantization structure remains to be designed that is well suited to these transforms. We therefore propose a CE to investigate those issues in detail, test with an agreed set of well-formulated (and sw-furnished) complexity metrics, and return with recommendations on the transform and quantization structures.

# References

- K. McCann, W. J. Han, I. K. Kim, J. H. Min, E. Alshina, A. Alshin, T. Lee, J. Chen, V. Seregin, S. Lee, Y. M. Hong, M. S. Cheon, and N. Shlyakhov,  Samsung's response to the call for proposals on video compression technology,  in 1st JCT-VC meeting , no. JCTVC-A124, Apr. 2010.

- W.-H. Chen, C. H. Smith, and S. C. Fralick, A fast computational algorithm for the discrete cosine transform, IEEE Trans. Commun., vol. COMM-25, pp. 1004-1009, Sep. 1977.

- C. Loeffler, A. Lightenberg, and G. Moschytz, Practical fast 1-D DCT algorithms with 11 multiplications,  Proc. IEEE ICASSP, vol. 2, pp. 988–991, Feb. 1989.

- J. Liang and T. D. Tran, Fast multiplierless approximations of the DCT with the lifting scheme, IEEE Trans. Signal Process., no. 12, pp. 3032-3044, Dec. 2001.

- X. Wan, Y. Wang, and W. Chen, Dynamic range analysis for the implementation of fast transform, IEEE Trans. Circuits Syst. Vid. Technol., vol. 5, pp. 178–180, Apr. 1995.

- Yoon Mi Hong, Min-Su Cheon, Il-Koo Kim, Low complexity 16x16 and 32x32 transforms and partial frequency transform, in 3rd JCT-VC meeting, no. JCTVC-C209, Oct. 2010.

- J. Liang, T. Tran, P. Topiwala, “A 16-bit architecture for H.26L, treating DCT transforms and quantization,” ITU VCEG doc M16, Austin, TX, April, 2001.

[8]  Wikepedia page on dyadic rationals, http://en.wikipedia.org/wiki/Dyadic_rational.

[9]  P. Topiwala, Wavelet Image and Video Compression, Kluwer Academic Press, 1998.

[10] H. Malvar, “Biorthogonal and Nonuniform Lapped Transforms for Transform Coding with Reduced Blocking and Ringing Artifacts,” IEEE Trans. Signal Proc., April, 1998.

# Patent rights declaration(s)

FastVDO LLC and Samsung Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).