| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 |  |  |  | Document: JCTVC-E354 WG11 Number: m19882 |  |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Further study on entropy coding |  |  |  |
| Status: |  | Input Document to JCT-VC |  |  |  |
| Purpose: |  | Information |  |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA |  | Tel: Email: Tel: Email: | +1-214-567-4005 sze@ti.com +1-214-480-1861 madhukar@ti.com |  |
| Source: | Texas Instruments Inc. |  |  |  |  |

____________________________

# Abstract

This contribution summarizes the key throughput challenges that exist for CABAC as well as other methods that have been proposed for entropy coding in HEVC.  For CABAC, the serial dependencies due to multiple feedback loops cause a bottleneck in video codec implementations.  Speculative computations are required when using parallelism and pipelining which are common techniques used to increase throughput.  Alternative entropy coding methods for high efficiency entropy coding also contains feedback loops in the context modeling; thus their end-to-end throughput is unknown – accordingly, the throughput benefit compared to the CABAC (particularly at the decoder) needs to be studied.  TE8 and TE12 reported that these approaches have coding losses compared to CABAC (between 0.2 to 0.5%).  In addition to throughput, area cost (e.g. buffers and tables), and implementation complexity (e.g. multiple bitstreams) needs to also be studied.

# Bottlenecks in CABAC

CABAC is a well known throughput bottleneck in the video codec implementations (particularly at the decoder-side).  This is due to the many feedback loops in the CABAC, several of which are in the context selection. Fig. 1 highlights the feedback loops within the CABAC in both the encoder and decoder.  First, the value of the context is updated based on the previously encoded/decoded bins.  Second, the context to be chosen depends on the previously encoded/decoded bins as well as various factors such as macroblock type, binIdx, transform size, location in transform, etc.  At the encoder, this loop is contained in the context selection; at the decoder, this second feedback loop passes through the bin decoder.  Thus parallelism is a particular challenge at the decoder.

These loops make the CABAC difficult to parallelize, which makes it difficult to achieve the high throughput necessary for high resolution and frame-rate videos.  Furthermore, since high throughput can be traded-off for power savings using voltage scaling, the serial nature of CABAC limits the battery life for video codecs that reside on mobile devices.  This limitation is a critical concern, as a significant portion of future video codecs will be battery-operated.

- encoder

- decoder

Fig.1. The CABAC is composed of three key blocks: Binarizer, Context Modeler and Bin Coder (i.e. Arithmetic Coder). Feedback loops are highlighted with dashed lines.

## Speculative Computations for multi-bin decoding

A common technique used in practice is speculative computation which aims to increase the throughput of CABAC.   However, this comes as a cost of increased number of computations per bin.    For a two bin per cycle example: in order to decode bin2 in parallel with bin1 requires 2x computations for bin2 to account for the case that bin1=0 and bin1=1.  In particular, the number of context selection computations increases.  The number of operations increases exponentially as we increase the number of bins that are to be processed in parallel. (e.g. for N bins to be decoded in parallel, we need up to 1+2+4+…+2(N-1) context selection computations)

To reduce the cycle time (i.e critical path), the CABAC needs to be pipelined. For instance, the 4-stage pipeline shown in Fig. 2 is commonly used for AVC CABAC implementations [1].  Due to the feedback loops, each inserted pipeline stage between context selection and bin decoder increases the number of context selection computations by 2x.  For the architecture in Fig. 2, the number of context selections operations grows to by a factor of 4x, which means a 2-bin per cycle pipelined solution requires 12x context selection computations.

Fig. 2 Common pipelined architecture for CABAC. Context modeling blocks in grey.  Feedback loops highlight with dashed lines.

Thus the total increase in computations for context selection for an M-stage pipelined architecture decoding N-bins per cycles is 2(M-1) *(1+2+4+…+2(N-1)). Often optimizations are done in an attempt to reduce the total computations.

# Alternative entropy coding approaches

Alternative entropy coding methods for high efficiency entropy coding also contains feedback loops in the context modeling; thus their end-to-end throughput is unknown – accordingly, the throughput benefit compared to the CABAC (particularly at the decoder) needs to be studied.  TE8 and TE12 reported that these approaches have coding losses compared to CABAC (between 0.2 to 0.5%).  In addition to throughput, area cost (e.g. buffers and tables), and implementation complexity (e.g. multiple bitstreams) needs to also be studied.

# Summary

This contribution highlights the context selection bottleneck in CABAC.  Furthermore, it describes how this alternative entropy coding methods for high efficiency entropy coding does not address this problem while resulting in coding loss and increase in area cost and implementation complexity.

# References

- Y. Yi, I.-C. Park , "High-Speed H.264/AVC CABAC Decoding," IEEE Trans. CSVT April 2007