| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E358 WG11 Number: m19887 |
| --- | --- |

| Title: | CE3: Report on Motorola Mobility's interpolation filter for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo David Baylon Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com +1-858-404-2285 david.baylon@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Motorola Mobility’s interpolation filter for HEVC. The simulations were conducted using HM2.0 software with Motorola Mobility’s modifications. Compared with the current interpolation filter in HM2.0, the proposed method applies a 7-tap filter for quarter-pel positions while keeping the same process for half-pel positions. The proposed method saves 0.1% on LDHE setting and saves 0.3% on LDLC setting. The proposed method loses 0.1% on RAHE setting and loses 0.3% on RALC setting. Cross-check will be provided by Samsung and HHI. The attached spreadsheet contains detailed data of the results.

# Introduction

The target of this document is to verify the performance of Motorola Mobility’s 6-bit interpolation filter for HEVC [1]. The simulations were conducted using HM2.0 software [2] with Motorola Mobility’s modifications. Cross-check will be provided by Samsung and HHI.

# Proposed sub-pel interpolation filters

An 8-tap FIR filter with 6-bit coefficients is used for the half-pel interpolation. The half-pel interpolation filter is the same as the anchor. 3 multiplications and 7 additions, or 9 additions and 4 shifts are needed for each half-pel interpolation. A 7-tap FIR filter with 6-bit coefficients is proposed for the quarter-pel interpolation. 6 multiplications and 6 additions, or 10 additions and 5 shifts are needed for each quarter-pel interpolation.

Fig. 1 illustrates the positions of the resulting half-pel and quarter-pel pixels between full-pel pixels along a pixel line within an image, where L3, L2, L1, L0, R0, R1, R2, and R3 are full-pel pixels, H is a half-pel pixel between L0 and R0, FL is a sub-pel pixel between L0 and H, and FR is a sub-pel pixel between H and R0.

Fig. 1 Sub-Pel Positions

FL = (-1*L3 + 3*L2 - 9*L1 + 57*L0 + 18*R0 - 6*R1 + 2*R2 - 0*R3 + 32) >> 6;

H = (-1*L3 + 4*L2 - 11*L1 + 40*L0 + 40*R0 - 11*R1 + 4*R2 - 1*R3 + 32) >> 6;

FR = (-0*L3 + 2*L2 - 6*L1 + 18*L0 + 57*R0 - 9*R1 + 3*R2 - 1*R3 + 32) >> 6;

A mathematically equivalent representation of the FL calculation using only addition and shift operations is

T1 = -L1 + ((L0 + R0) << 1) + (L0 << 2);

T2 = L2 + L0 - (R1 << 1);

FL = (T1 + T2 – L3 + ((T2 + R2) << 1) + (T1 << 3) + 32) >> 6;

Table 1 summarizes the filter coefficients. In a practical implementation, the operation as “0*R3” or “0*L3” could be skipped. That is, only 7-tap interpolation filters are needed for quarter-pel interpolation.

Table 1 Sub-pel pixel interpolation filter coefficients

| Position | Coefficients |
| --- | --- |
| FL | { -1, 3, -9, 57, 18, -6, 2, 0,} |
| H | { -1, 4, -11, 40, 40, -11, 4, 0,} |
| FR | { 0, 2, -6, 18, 57, -9, 3, -1,} |

# Simulation environment

## Software

The software used to run the simulations is HM2.0. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on PCs with identical settings, i.e., Microsoft Windows XP 64bits, Intel Xeon 3.06GHz with 8G memory. Anchor results from the same PCs are used for fair comparison. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The encoder reconstructed YUV files match the decoder reconstructed YUV files. The experimental results of the rate-distortion and speed comparison are summarized in Table 2. Detailed results are included in the attachment (JCTVC-E358.xls).

Table 2 Rate-distortion and speed performance comparison of Motorola Mobility’s interpolation filters

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.2 | -0.3 | -0.2 | -0.2 |
| Class B | -0.1 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
| Class C | 0.1 | 0.1 | 0.1 | 0.5 | 0.1 | 0.1 |
| Class D | 0.3 | 0.3 | 0.4 | 1.3 | 0.3 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.1 | 0.3 | 0.1 | 0.1 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.1 | -0.1 | -0.9 | -0.1 | 0.2 |
| Class C | 0.1 | 0.3 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class D | 0.1 | 0.4 | 0.2 | 0.8 | 0.5 | 0.6 |
| Class E | -0.4 | -0.7 | 0.6 | -1.3 | -0.5 | 0.0 |
| All | -0.1 | 0.0 | 0.2 | -0.3 | 0.0 | 0.2 |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

# Complexity

Table 3 gives the complexity analysis of the proposed filter.

Table 3 Complexity analysis

| Method |  |  |  | DCTIF 8-tap | Proposal |  |
| --- | --- | --- | --- | --- | --- | --- |
| add/shift or multiple |  |  |  | add/shift | add/shift |  |
| Sheet |  |  |  | Anchor | Proposal | Ratio (%) |
| Single | 4x4 | Operations | Worst case / pixel | 60.00 | 52.50 | 88% |
|  |  |  | Average of operations / pixel | 39.75 | 35.83 | 90% |
|  |  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% |
|  |  |  | Average of bytes / pixel | 5.35 | 4.79 | 89% |
| Single | 8x8 | Operations | Worst case / pixel | 46.00 | 41.25 | 90% |
|  |  |  | Average of operations / pixel | 31.88 | 29.38 | 92% |
|  |  | Bandwith | Worst case / pixel | 3.52 | 3.52 | 100% |
|  |  |  | Average of bytes / pixel | 2.74 | 2.54 | 93% |
| Single | 16x16 | Operations | Worst case / pixel | 39.00 | 35.63 | 91% |
|  |  |  | Average of operations / pixel | 27.94 | 26.16 | 94% |
|  |  | Bandwith | Worst case / pixel | 2.07 | 2.07 | 100% |
|  |  |  | Average of bytes / pixel | 1.76 | 1.68 | 95% |
| Single | 32x32 | Operations | Worst case / pixel | 35.50 | 32.81 | 92% |
|  |  |  | Average of operations / pixel | 25.97 | 24.55 | 95% |
|  |  | Bandwith | Worst case / pixel | 1.49 | 1.49 | 100% |
|  |  |  | Average of bytes / pixel | 1.36 | 1.32 | 97% |
| Single | 64x64 | Operations | Worst case / pixel | 33.75 | 31.41 | 93% |
|  |  |  | Average of operations / pixel | 24.98 | 23.74 | 95% |
|  |  | Bandwith | Worst case / pixel | 1.23 | 1.23 | 100% |
|  |  |  | Average of bytes / pixel | 1.17 | 1.15 | 99% |
| Bi | 4x4 | Operations | Worst case / pixel | 120.00 | 105.00 | 88% |
|  |  |  | Average of operations / pixel | 79.50 | 71.67 | 90% |
|  |  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  |  | Average of bytes / pixel | 10.70 | 9.57 | 89% |
| Bi | 8x8 | Operations | Worst case / pixel | 92.00 | 82.50 | 90% |
|  |  |  | Average of operations / pixel | 63.75 | 58.77 | 92% |
|  |  | Bandwith | Worst case / pixel | 7.03 | 7.03 | 100% |
|  |  |  | Average of bytes / pixel | 5.49 | 5.08 | 93% |
| Bi | 16x16 | Operations | Worst case / pixel | 78.00 | 71.25 | 91% |
|  |  |  | Average of operations / pixel | 55.88 | 52.32 | 94% |
|  |  | Bandwith | Worst case / pixel | 4.13 | 4.13 | 100% |
|  |  |  | Average of bytes / pixel | 3.53 | 3.36 | 95% |
| Bi | 32x32 | Operations | Worst case / pixel | 71.00 | 65.63 | 92% |
|  |  |  | Average of operations / pixel | 51.94 | 49.09 | 95% |
|  |  | Bandwith | Worst case / pixel | 2.97 | 2.97 | 100% |
|  |  |  | Average of bytes / pixel | 2.71 | 2.64 | 97% |
| Bi | 64x64 | Operations | Worst case / pixel | 67.50 | 62.81 | 93% |
|  |  |  | Average of operations / pixel | 49.97 | 47.48 | 95% |
|  |  | Bandwith | Worst case / pixel | 2.46 | 2.46 | 100% |
|  |  |  | Average of bytes / pixel | 2.34 | 2.31 | 99% |
| Single |  | Operations | Worst case / pixel | 60.00 | 52.50 | 88% |
|  |  |  | Average of operations / pixel | 30.10 | 27.93 | 93% |
|  |  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% |
|  |  |  | Average of bytes / pixel | 2.48 | 2.30 | 93% |
| Bi |  | Operations | Worst case / pixel | 120.00 | 105.00 | 88% |
|  |  |  | Average of operations / pixel | 60.21 | 55.86 | 93% |
|  |  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  |  | Average of bytes / pixel | 4.95 | 4.59 | 93% |
| Total |  | Operations | Worst case / pixel | 120.00 | 105.00 | 88% |
|  |  |  | Average of operations / pixel | 45.15 | 41.90 | 93% |
|  |  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  |  | Average of bytes / pixel | 3.71 | 3.44 | 93% |

# References

[1] J. Lou and L. Wang, “New DCT-based interpolation filters,” JCT-VC document, JCTVC-D246, January 2011

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-2.0

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, January 2011

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).