| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E359 WG11 Number: m19888 |
| --- | --- |

| Title: | Motorola Mobility’s Adaptive Interpolation Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo David Baylon Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com +1-858-404-2285 david.baylon@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

JCTVC-E359 proposes an adaptive method for generation of sub-pixel samples for temporal prediction of each Prediction Unit (PU). In this contribution, the choice of filters is implicitly signaled based on the PU size. More specifically, this contribution proposes that for temporal prediction, a PU with a given size uses a certain class of interpolation filters to generate samples at each sub-pixel position. Although with this scheme it is possible to have as many classes of interpolation filters as the number of allowed PU sizes, this contribution shows the results based on two classes of interpolation filters. The proposed method saves 0.1% on LDHE setting and saves 0.5% on LDLC setting. The proposed method achieves on average the same performance as the anchor on RAHE setting and loses 0.1% on RALC setting. Cross-check will be provided by Sharp. The attached spreadsheet contains detailed data of the results.

# Introduction

HEVC allows for motion vectors with fractional pixel accuracy. There can be two purposes for sub-pixel prediction amongst others:

- Motion during two instances of time (the current and the reference frames’ capture times) can correspond to a fractional pixel position in the picture coordinates.

- Generation of different prediction data, corresponding to each sub-pixel position, provides the possibility of conditioning the prediction signal to better match the signal in the current block.

For sub-pixel interpolation, HEVC uses FIR filters with limited precision. For example, HM 2.0 uses separable 8 tap filters in both horizontal and vertical positions with 6 bits precision to obtain sub-pixel values for different half-pel and quarter-pel positions.

If the coefficients of the filters are fixed and applicable to compression of all sequences, then the interpolation filters would be called fixed. However, if the filter choice varies from sequence to sequence, from picture to picture, from reference to reference (in case of multiple references), or within a picture from PU to PU, then the filter is referred to here as an Adaptive Interpolation Filter (AIF).

The choice of adaptive filters can be communicated to the decoder explicitly either by sending the filter coefficients or by sending the index of the preferred filter from a known array of different filter choices. Alternatively, the decoder can derive the choice of adaptive filter implicitly by using information available at decoding time.

Since natural images demonstrate localized characteristics, it is preferred to perform adaptive sub-pixel interpolation for each predicted block (PU). While this is preferred, the cost associated with signaling the choice of the sub-pixel interpolation filter for each PU has to be considered. Explicit signaling can be costly since it can significantly increase the required bit rate in temporally predicted pictures that tend to consume comparable number of bits relative to the bits required to indicate either the filter coefficients or the filter index.

To overcome this problem this contribution proposes to re-use some of the information that is already available to both encoder and decoder at the time of motion compensation to implicitly decide the choice of the sub-pixel filter at both the encoder and decoder.

# Proposed Approach Implicit Selection of Interpolation Filter class

In the proposed method, a specific class of interpolation filters is assigned to each PU size (which is derived based on the CU size and PU partition type). Note that the interpolation filter class refers to a set of filters for a PU size. For example in HM 2.0, there is one specific set of filters for ½ pixel, and another specific filter for ¼-pixel positions. Since the encoder and decoder know about the current PU size, they can find the appropriate filter for sub-pixel interpolation. In this approach, the number of interpolation categories can be as large as the number of PU sizes. The mapping between PU size and interpolation filter class can be signaled in one of the sequence or slice headers, or a default mapping can be used for both encoder and decoder.

In general, the size of a current PU can be an indication for characteristics of the reference block when the quality of the coded pictures is consistent for consecutive frames, where the compression quality from frame to frame doesn’t generally change much. Based on this assumption, the preferred choice of PU size for compression can indicate certain characteristics of the reference signal in terms of noise level or high frequency content. The different characteristics can be exploited with a different filter class.

# Implementation

To report simulation results, an adaptive filtering scheme was devised where two classes of filters were assigned for deriving the quarter-pixel positions. Fig. 1 illustrates the positions of the resulting half-pel and quarter-pel pixels between full-pel pixels along a pixel line within an image, where L3, L2, L1, L0, R0, R1, R2, and R3 are full-pel pixels, H is a half-pel pixel between L0 and R0, FL is a sub-pel pixel between L0 and H, and FR is a sub-pel pixel between H and R0.

Fig. 1. Sub-Pel Positions

The half-pel pixel H is always interpolated using the HM2.0 filter.

H = (-1*L3 + 4*L2 - 11*L1 + 40*L0 + 40*R0 - 11*R1 + 4*R2 - 1*R3 + 32) >> 6;

If the PU width is no larger than 8 pixels or PU height is no larger than 8 pixels, the sub-pel pixels FL and FR are interpolated using the values of spatial neighboring full-pel pixels, L3, L2, L1, L0, R0, R1, R2, and R3, as follows,

FL = (-1*L3 + 4*L2 - 10*L1 + 57*L0 + 19*R0 - 7*R1 + 3*R2 - 1*R3 + 32) >> 6;

FR = (-1*L3 + 3*L2 - 7*L1 + 19*L0 + 57*R0 - 10*R1 + 4*R2 - 1*R3 + 32) >> 6;

Notice that this set of filters is currently implemented in HM2.0.

Otherwise, FL and FR, are interpolated as follows

FL = (-1*L3 + 3*L2 - 9*L1 + 57*L0 + 18*R0 - 6*R1 + 2*R2 - 0*R3 + 32) >> 6;

FR = (-0*L3 + 2*L2 - 6*L1 + 18*L0 + 57*R0 - 9*R1 + 3*R2 - 1*R3 + 32) >> 6;

Notice that this set of filters is also proposed in [1].

Table 1 summarizes the filter coefficients. In a practical implementation, the operation as “0*R3” or “0*L3” could be skipped. That is only 7-tap interpolation filters are needed when PU width is larger than 8 pixels and PU height is larger than 8 pixels.

Table 1 High efficiency sub-pel pixel interpolation filter coefficient

|  | Position | Coefficients |
| --- | --- | --- |
| PUwidth<=8 or PUheight<=8 | FL | { -1, 4, -10, 57, 19, -7, 3, -1,} |
|  | FR | { -1, 3, -7, 19, 57, -10, 4, -1,} |
| PUwidth>8 and PUheight>8 | FL | { -1, 3, -9, 57, 18, -6, 2, 0,} |
|  | FR | { 0, 2, -6, 18, 57, -9, 3, -1,} |

# Simulation environment

## Software

The software used to run the simulations is HM2.0 [2]. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.06GHz with 8G memory. Anchor results from the same PCs are used for fair comparison. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The encoder reconstructed YUV files match the decoder reconstructed YUV files. The experimental results of the rate-distortion and speed comparison are summarized in Table 2. Detailed results are included in the attachment (JCTVC-E359.xls).

Table 2 Rate-distortion and speed performance comparison of Motorola Mobility’s adaptive interpolation

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.1 | -0.3 | -0.1 | -0.1 |
| Class B | -0.1 | 0.1 | 0.0 | -0.2 | -0.1 | 0.0 |
| Class C | 0.1 | 0.2 | 0.2 | 0.4 | 0.2 | 0.1 |
| Class D | 0.2 | 0.4 | 0.3 | 0.7 | 0.2 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.0 | 0.1 | -0.9 | 0.0 | 0.3 |
| Class C | 0.0 | 0.3 | 0.3 | 0.0 | 0.2 | 0.0 |
| Class D | 0.0 | 0.2 | 0.2 | 0.2 | 0.5 | 0.5 |
| Class E | -0.2 | -0.2 | 0.5 | -1.2 | -0.3 | -0.1 |
| All | -0.1 | 0.1 | 0.2 | -0.5 | 0.1 | 0.2 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  |

# Complexity

Table 3 shows the complexity analysis for the proposed approach.

Table 3 Complexity Analysis

| Method |  |  |  | DCTIF 8-tap | Proposal |  |
| --- | --- | --- | --- | --- | --- | --- |
| add/shift or multiple |  |  |  | add/shift | add/shift |  |
| Sheet |  |  |  | Anchor | Proposal | Ratio (%) |
| Single | 4x4 | Operations | Worst case / pixel | 60.00 | 60.00 | 100% |
|  |  |  | Average of operations / pixel | 39.75 | 39.75 | 100% |
|  |  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% |
|  |  |  | Average of bytes / pixel | 5.35 | 5.35 | 100% |
| Single | 8x8 | Operations | Worst case / pixel | 46.00 | 46.00 | 100% |
|  |  |  | Average of operations / pixel | 31.88 | 31.88 | 100% |
|  |  | Bandwith | Worst case / pixel | 3.52 | 3.52 | 100% |
|  |  |  | Average of bytes / pixel | 2.74 | 2.74 | 100% |
| Single | 16x16 | Operations | Worst case / pixel | 39.00 | 35.63 | 91% |
|  |  |  | Average of operations / pixel | 27.94 | 26.16 | 94% |
|  |  | Bandwith | Worst case / pixel | 2.07 | 2.07 | 100% |
|  |  |  | Average of bytes / pixel | 1.76 | 1.68 | 95% |
| Single | 32x32 | Operations | Worst case / pixel | 35.50 | 32.81 | 92% |
|  |  |  | Average of operations / pixel | 25.97 | 24.55 | 95% |
|  |  | Bandwith | Worst case / pixel | 1.49 | 1.49 | 100% |
|  |  |  | Average of bytes / pixel | 1.36 | 1.32 | 97% |
| Single | 64x64 | Operations | Worst case / pixel | 33.75 | 31.41 | 93% |
|  |  |  | Average of operations / pixel | 24.98 | 23.74 | 95% |
|  |  | Bandwith | Worst case / pixel | 1.23 | 1.23 | 100% |
|  |  |  | Average of bytes / pixel | 1.17 | 1.15 | 99% |
| Bi | 4x4 | Operations | Worst case / pixel | 120.00 | 120.00 | 100% |
|  |  |  | Average of operations / pixel | 79.50 | 79.50 | 100% |
|  |  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  |  | Average of bytes / pixel | 10.70 | 10.70 | 100% |
| Bi | 8x8 | Operations | Worst case / pixel | 92.00 | 92.00 | 100% |
|  |  |  | Average of operations / pixel | 63.75 | 63.75 | 100% |
|  |  | Bandwith | Worst case / pixel | 7.03 | 7.03 | 100% |
|  |  |  | Average of bytes / pixel | 5.49 | 5.49 | 100% |
| Bi | 16x16 | Operations | Worst case / pixel | 78.00 | 71.25 | 91% |
|  |  |  | Average of operations / pixel | 55.88 | 52.32 | 94% |
|  |  | Bandwith | Worst case / pixel | 4.13 | 4.13 | 100% |
|  |  |  | Average of bytes / pixel | 3.53 | 3.36 | 95% |
| Bi | 32x32 | Operations | Worst case / pixel | 71.00 | 65.63 | 92% |
|  |  |  | Average of operations / pixel | 51.94 | 49.09 | 95% |
|  |  | Bandwith | Worst case / pixel | 2.97 | 2.97 | 100% |
|  |  |  | Average of bytes / pixel | 2.71 | 2.64 | 97% |
| Bi | 64x64 | Operations | Worst case / pixel | 67.50 | 62.81 | 93% |
|  |  |  | Average of operations / pixel | 49.97 | 47.48 | 95% |
|  |  | Bandwith | Worst case / pixel | 2.46 | 2.46 | 100% |
|  |  |  | Average of bytes / pixel | 2.34 | 2.31 | 99% |
| Single |  | Operations | Worst case / pixel | 60.00 | 60.00 | 100% |
|  |  |  | Average of operations / pixel | 30.10 | 29.21 | 97% |
|  |  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% |
|  |  |  | Average of bytes / pixel | 2.48 | 2.45 | 99% |
| Bi |  | Operations | Worst case / pixel | 120.00 | 120.00 | 100% |
|  |  |  | Average of operations / pixel | 60.21 | 58.43 | 97% |
|  |  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  |  | Average of bytes / pixel | 4.95 | 4.90 | 99% |
| Total |  | Operations | Worst case / pixel | 120.00 | 120.00 | 100% |
|  |  |  | Average of operations / pixel | 45.15 | 43.82 | 97% |
|  |  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  |  | Average of bytes / pixel | 3.71 | 3.67 | 99% |

# Summary

This proposal provides a framework for adaptive interpolation filtering which can be used to condition the signal and noise in the prediction block. It’s worth noting that the results reported in this contribution come from one implementation based on the general idea of using PU size to signal the choice of interpolation filter class.

In terms of complexity the proposal has lower complexity compared to the HM 2.0 anchor due to the use of smaller taps for quarter-pixel filters which are used for larger PUs.

As the minimum and maximum size of CUs (and therefore PUs) can be set based on the application, it seems logical to signal the mapping between the interpolation filter classes and PU sizes in the corresponding header where minimum and maximum CU sizes are set.

The proposed method saves 0.1% on LDHE setting and saves 0.5% on LDLC setting. The proposed method achieves on average the same performance as the anchor on RAHE setting and loses 0.1% on RALC setting.

# References

[1] J. Lou, K. Minoo, D. Baylon, K. Panusopone, and L. Wang, “CE3: Report on Motorola Mobility's interpolation filter for HEVC,” JCT-VC document, JCTVC-E358, March 2011

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-2.0

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, January 2011

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).