| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E360 |
| --- | --- |

| Title: | Cross-Verification of ETRI’s Contribution (JCTVC-E223) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | G. Van der Auwera, X. Wang, and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: geertv@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The cross-verification reports the rate-distortion results, and processing times of ETRI’s contribution JCTVC-E223 on single interpolation for multi-sample prediction (SIMP) for intra coding. The rate-distortion results of this cross-verification match those reported by ETRI in JCTVC-E223. The encoding and decoding times are comparable to HM2.0.

# Introduction

Single interpolation for multi-sample prediction (SIMP) for intra coding modifies angular intra prediction. This contribution reports the objective rate-distortion results and the processing times.

# Objective Verification Results

## Rate-Distortion Results

Table 1 enumerates the rate-distortion results as BD-rate differences. The common test conditions are those specified in JCTVC-D600 for intra coding in the high-efficiency and low-complexity configurations. The conclusion is that the BD-rate differences match those reported by ETRI in JCTVC-E223.

Table 1 Rate-distortion results

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.1 |
| All | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

## Processing Times

Table 1 reports the encoding and decoding times for ETRI’s SIMP method.

The encodings were executed on a Linux computing cluster consisting of machines with various computing performance and memory configurations. The encoder executable was built using GCC version 4.1.2. The differences in encoding times between our measurements and ETRI’s are likely attributable to the variation in execution times on our computing cluster.

The decodings were executed on a PC with the following configuration: Windows 7, 6 GB memory, Intel Xeon 2.79GHz (W3530), Microsoft Visual Studio 2008. The decoding times are similar compared to the decoding times of the anchors.

# Source Code Evaluation

The SIMP method modifies the prediction of angular intra prediction for PU size 32×32 by duplicating a predicted even indexed row onto the next odd indexed row. The prediction of the odd indexed row does not involve interpolation.

# Conclusion

This cross-verification matches the rate-distortion results reported in ETRI’s SIMP contribution (JCTVC-E223). The encoding and decoding times are similar to those from HM2.0.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).