| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5rd Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E361 WG11 Number: m19890 |
| --- | --- |

| Title: | Supporting one list only coding for HM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo David Baylon Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com +1-858-404-2285 david.baylon@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

Many applications have a tight restriction on complexity and memory access bandwidth. One list only coding is hence preferred for these applications. However, the current HM test condition excludes one list only coding and hence this coding mode may become more and more inefficient to the point of uselessness as new tools are added to the HEVC standard. This document requests modifications to HM common test condition to include the one list only coding case.

- Introduction

Video playback applications are growing in popularity for handheld devices such as smartphones and tablets due to improving video quality, availability of a wide selection of content, and increasingly powerful decoding engines. However, video decoding is a very demanding task and computing resources in handheld devices are limited. Generally, some constraints have to be applied to the video bitstream so that the decoding engine can handle it without exceeding its resources capability.

One such constraint is one list only coding. This constraint reduces complexity since the decoder does not have to generate the second temporal prediction block to compute an interpolated block. Moreover, one list only coding reduces memory access bandwidth since the decoder only has to fetch once for the temporal prediction block instead of twice. Encoder complexity is much less with one list only coding since there is no need to perform motion estimation for the second temporal prediction block.

Previous video coding standards such as MPEG-2 and AVC allow one list coding in their P picture/slice and two lists coding in their B picture. In the current common test conditions of HEVC [1], a GPB concept is used where all pictures are B pictures with two list coding [2]. Although it improves coding performance in HEVC, it suffers a major coding loss when a one list coding constraint is applied. This is due to the fact the many tools in HEVC are optimized to support two list coding.

- Proposed changes to support one list only coding

At a high level, one list only coding can be supported by using a P slice type.

At a tool level, it is important to modify some tools for them to be efficient with one list only coding. One problem with the current HM is that it produces a one list only bitstream with poor coding performance. The performance of some tools such as the interpolation filter can be improved significantly if the tools are designed to handle one list coding. The experimental results provided in the proposal will serve as an example.

To ensure that HEVC performs at a satisfactory level, it is necessary to include a one list only coding constraint in at least one of its common test conditions. Low delay low complexity may be a suitable test condition to require one list only coding. The coding conditions used for this set of simulations are the same as the low delay low complexity settings defined in [1] except that GPB is set as 0. We may call it as “one list only prediction low delay low complexity (OLO LDLC)”.

# Simulation environment

## Software

The software used to run the simulations is HM2.0. The binaries were compiled using Microsoft Visual Studio 2008 64bit.

Two sets of simulation results are provided. The anchor results are simulated using HM2.0 software with the related HM2.0 r587 changes [3]. The other set is provided using the software in [4] with the related HM2.0 r587 changes. The related changes in r587 address the problem mentioned in [5] and they are necessary for the OLO LDLC testing.

YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.06GHz with 8G memory. Anchor results from the same PCs are used for fair comparison. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The encoder reconstructed YUV files match the decoder reconstructed YUV files. The experimental results comparing speed and rate-distortion performance between the HM2.0 anchor with LDLC setting and the HM2.0 anchor with OLO LDLC setting are summarized in Table 1. The experimental results comparing speed and rate-distortion performance between the HM2.0 anchor with OLO LDLC setting and the filter from [4] are summarized in Table 2. Detailed results are included in the attachment (JCTVC-E361.xls).

Table 1 Rate-distortion and speed performance comparison between HM2.0 LDLC anchor and HM2.0 OLO LDLC anchor

|  | HM2LDLCAnchor VS HM2OLOLDLCAnchor |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 19.1 | 8.3 | 7.8 |
| Class C | 9.9 | 4.5 | 4.4 |
| Class D | 9.2 | 4.4 | 4.5 |
| Class E | 16.9 | 5.2 | 5.7 |
| All | 13.9 | 5.8 | 5.7 |
| Enc Time[%] | 53% |  |  |
| Dec Time[%] | 85% |  |  |

Table 2 Rate-distortion and speed performance comparison between HM2.0 OLO LDLC anchor and the interpolation filter from [4]

|  | OLO LDLC |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -3.8 | -0.2 | -0.1 |
| Class C | -1.9 | 0.0 | -0.1 |
| Class D | -1.7 | 0.3 | 0.5 |
| Class E | -6.3 | -0.2 | -1.3 |
| All | -3.3 | 0.0 | -0.2 |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 97% |  |  |

# Summary

This contribution document requests modifications to the HM common test condition to include the one list only coding case. Examples are provided to show that the current HM2.0 suffers a major coding loss when a one list coding constraint is applied.

# References

[1] Frank Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-D600, Jan. 2011.

[2] “HEVC Specification Working Draft 2,” JCT-VC document, JCTVC-D503, Jan. 2011.

[3] http://hevc.kw.bbc.co.uk/trac/changeset/587

[4] J. Lou, K. Minoo, D. Baylon, K. Panusopone, and L. Wang, “CE3: Report on Motorola Mobility's interpolation filter for HEVC,” JCT-VC document, JCTVC-E358, Mar. 2011.

[5] http://hevc.kw.bbc.co.uk/trac/ticket/130

Motorola Mobility Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).