| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E362 WG11 Number: m19891 |
| --- | --- |

| Title: | On context selection for significant_coeff_flag coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

The scheme used in the current HM2.0 for CABAC significance map coding utilizes the nearest 5 neighbors for context modeling. This contribution document studies using of 4, 3, and 2 neighbors for context modeling for significance map coding in CABAC. With the reduced number of neighbors, fewer coded neighbors are used for deriving the contexts. Especially for the 4 and 2 neighbor cases, only 1 scanning line of previous coded neighbors needs to be buffered as compared to 2 lines in the current design. The experimental results show that there is 0.2% to 0.3% bitrate increase for the 4 neighbor design, 0.0% to 0.2% bitrate increase for the 3 neighbor design and 0.3% to 0.4% bitrate increase for the 2 neighbor design. The cross-check results are provided by I2R.

# Introduction

The scheme used in the current HM2.0 [1] for CABAC significance map coding utilizes the nearest 5 neighbors [1] for context modeling in order to achieve better estimation of the probability distribution. An example is given in Fig.1. The context of the element at position (y, x) is determined by 5 coded neighbors (y-2, x), (y-1, x-1), (y, x-2), (y-1, x), and (y, x-1). If the position is on the block boundary, some of the neighboring positions are not available and fewer neighbors are used.

This contribution document studies using of 4, 3, and 2 neighbors for context modeling for significance map coding in CABAC.

Fig. 1 Context selection utilizing 5 coded neighbor information in significance map coding

# Utilizing 4, 3, and 2 coded neighbors for significance map coding

## Utilizing 4 coded neighbors

Fig.2 gives the example of utilizing 4 coded neighbors for significance map coding. The context of the element at position (y, x) is determined by 4 coded neighbors (y-2, x+1), (y-1, x), (y, x+1), and (y+1, x-2). Only 1 scanning line of previous coded neighbors needs to be buffered as compared to 2 lines in the current design. The same number of contexts as the HM2.0 is used.

Fig. 2 Context selection utilizing 4 coded neighbor information in significance map coding

## Utilizing 3 coded neighbors

Fig.3 gives the example of utilizing 3 coded neighbors for significance map coding. The context of the element at position (y, x) is determined by 3 coded neighbors (y-1, x-1), (y-1, x), and (y, x-1). The number of contexts is reduced by 1 compared to HM2.0.

Fig. 3 Context selection utilizing 3 coded neighbor information in significance map coding

## Utilizing 2 coded neighbors

Fig.4 gives the example of utilizing 2 coded neighbors for significance map coding. The context of the element at position (y, x) is determined by 2 coded neighbors (y-1, x) and (y, x-1). Only 1 scanning line of previous coded neighbors needs to be buffered compared to 2 lines in the current design. The number of contexts is reduced by 2 compared to HM2.0.

Fig. 4 Context selection utilizing 2 coded neighbor information in significance map coding

# Simulation environment

## Software

The software used to run the simulations is HM2.0. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows XP 64bits, Intel Xeon 2.80GHz with 8G memory. Anchor results from the same PCs are used for fair comparison. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The encoder reconstructed YUV files match the decoder reconstructed YUV files. The experimental results of the rate-distortion and speed comparison for utilizing 4 coded neighbors in context modeling are summarized in Table 1. Detailed results are included in the attachment (JCTVC-E362_4Neighbors.xls). The experimental results of the rate-distortion and speed comparison for utilizing 3 coded neighbors in context modeling are summarized in Table 2. Detailed results are included in the attachment (JCTVC-E362_3Neighbors.xls). The experimental results of the rate-distortion and speed comparison for utilizing 2 coded neighbors in context modeling are summarized in Table 3. The Bin count statistics is provided in Table 4, Table 5, and Table6. Detailed results are included in the attachment (JCTVC-E362_2Neighbors.xls).

Table 1 Rate-distortion and speed performance comparison of utilizing 4 coded neighbors

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.4 | 0.5 |
| Class B | 0.3 | 0.2 | 0.4 |
| Class C | 0.3 | 0.1 | 0.2 |
| Class D | 0.2 | 0.1 | 0.1 |
| Class E | 0.4 | 0.2 | 0.3 |
| All | 0.3 | 0.2 | 0.3 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.5 | 0.6 |
| Class B | 0.3 | 0.3 | 0.4 |
| Class C | 0.3 | 0.0 | 0.1 |
| Class D | 0.2 | 0.0 | 0.0 |
| Class E |  |  |  |
| All | 0.2 | 0.2 | 0.3 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.4 | 0.3 | -0.2 |
| Class C | 0.5 | 0.1 | 0.2 |
| Class D | 0.1 | 0.0 | 0.0 |
| Class E | 0.3 | -0.6 | 1.8 |
| All | 0.3 | 0.0 | 0.4 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2 Rate-distortion and speed performance comparison of utilizing 3 coded neighbors

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.1 | 0.2 |
| Class B | 0.2 | 0.1 | 0.1 |
| Class C | 0.1 | 0.1 | 0.1 |
| Class D | 0.1 | 0.1 | 0.1 |
| Class E | 0.2 | 0.0 | 0.0 |
| All | 0.2 | 0.1 | 0.1 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.1 | -0.3 |
| Class B | 0.1 | 0.1 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 |
| Class D | 0.1 | -0.3 | -0.1 |
| Class E |  |  |  |
| All | 0.1 | 0.0 | -0.1 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.1 | -0.1 | 0.3 |
| Class C | 0.1 | 0.1 | 0.2 |
| Class D | 0.0 | 0.1 | 0.1 |
| Class E | -0.1 | -1.0 | 0.3 |
| All | 0.0 | -0.2 | 0.2 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3 Rate-distortion and speed performance comparison of utilizing 2 coded neighbors

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 0.4 | 0.5 |
| Class B | 0.4 | 0.3 | 0.4 |
| Class C | 0.3 | 0.2 | 0.2 |
| Class D | 0.2 | 0.2 | 0.1 |
| Class E | 0.5 | 0.2 | 0.2 |
| All | 0.4 | 0.3 | 0.3 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.4 | 0.2 |
| Class B | 0.3 | 0.2 | 0.2 |
| Class C | 0.3 | 0.1 | -0.1 |
| Class D | 0.1 | 0.0 | 0.0 |
| Class E |  |  |  |
| All | 0.3 | 0.2 | 0.1 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.4 | 0.3 | 0.1 |
| Class C | 0.5 | 0.3 | 0.3 |
| Class D | 0.1 | -0.1 | -0.2 |
| Class E | 0.1 | -0.8 | 0.8 |
| All | 0.3 | 0.0 | 0.2 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4 Bin count statistics for 4 coded neighbor scheme

|  | Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | -1% | 30% | 14% | 27% | 85% | 15% |
| Class B | -1% | 31% | 14% | 23% | 86% | 14% |
| Class C | -1% | 27% | 14% | 23% | 85% | 15% |
| Class D | 0% | 27% | 15% | 24% | 85% | 15% |
| Class E | -1% | 26% | 13% | 23% | 87% | 13% |
| All | -1% | 28% | 14% | 24% | 86% | 14% |
|  | Random access |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 28% | 11% | 17% | 86% | 14% |
| Class B | -1% | 27% | 10% | 16% | 87% | 13% |
| Class C | -1% | 23% | 10% | 15% | 86% | 13% |
| Class D | 0% | 23% | 10% | 15% | 86% | 13% |
| Class E |  |  |  |  |  |  |
| All | -1% | 25% | 10% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |
| Class B | -1% | 29% | 10% | 13% | 88% | 12% |
| Class C | -1% | 25% | 9% | 13% | 87% | 12% |
| Class D | 0% | 26% | 9% | 12% | 88% | 12% |
| Class E | -1% | 23% | 7% | 8% | 89% | 9% |
| All | -1% | 26% | 9% | 12% | 88% | 11% |

Table 5 Bin count statistics for 3 coded neighbor scheme

|  | Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 30% | 14% | 27% | 85% | 15% |
| Class B | 0% | 32% | 14% | 23% | 86% | 14% |
| Class C | 0% | 28% | 14% | 23% | 85% | 15% |
| Class D | 0% | 27% | 15% | 24% | 85% | 15% |
| Class E | 0% | 27% | 13% | 23% | 87% | 13% |
| All | 0% | 29% | 14% | 24% | 86% | 14% |
|  | Random access |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 28% | 11% | 17% | 86% | 14% |
| Class B | 0% | 28% | 10% | 16% | 87% | 13% |
| Class C | 0% | 23% | 10% | 15% | 86% | 13% |
| Class D | 0% | 23% | 10% | 15% | 86% | 13% |
| Class E |  |  |  |  |  |  |
| All | 0% | 26% | 10% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |
| Class B | 0% | 29% | 10% | 13% | 88% | 11% |
| Class C | 0% | 26% | 9% | 13% | 87% | 12% |
| Class D | 0% | 26% | 9% | 12% | 88% | 12% |
| Class E | 0% | 23% | 7% | 8% | 89% | 9% |
| All | 0% | 26% | 9% | 12% | 88% | 11% |

Table 6 Bin count statistics for 2 coded neighbor scheme

|  | Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | -1% | 30% | 14% | 27% | 85% | 15% |
| Class B | -1% | 31% | 14% | 23% | 86% | 14% |
| Class C | -1% | 27% | 14% | 23% | 85% | 15% |
| Class D | 0% | 27% | 15% | 24% | 85% | 15% |
| Class E | -1% | 26% | 13% | 23% | 87% | 13% |
| All | -1% | 28% | 14% | 24% | 86% | 14% |
|  | Random access |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 28% | 11% | 17% | 86% | 14% |
| Class B | -1% | 27% | 10% | 16% | 87% | 13% |
| Class C | -1% | 23% | 10% | 15% | 86% | 13% |
| Class D | 0% | 23% | 10% | 15% | 86% | 13% |
| Class E |  |  |  |  |  |  |
| All | -1% | 25% | 10% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |
| Class B | -1% | 29% | 10% | 13% | 88% | 12% |
| Class C | -1% | 25% | 9% | 13% | 87% | 12% |
| Class D | 0% | 26% | 9% | 12% | 88% | 12% |
| Class E | -1% | 23% | 7% | 8% | 89% | 9% |
| All | -1% | 26% | 9% | 12% | 88% | 11% |

# Summary

In this contribution document, utilizing 4, 3, and 2 coded neighbors for context modeling for significance map coding in CABAC is studied. With the reduced number of neighbors, fewer coded neighbors are used for deriving the contexts. Especially for the 4 and 2 neighbor cases, only 1 scanning line of previous coded neighbors needs to be buffered compared to 2 lines in the current design. The experimental results show that there is 0.2% to 0.3% bitrate increase for the 4 neighbor design, 0.0% to 0.2% bitrate increase for the 3 neighbor design and 0.3% to 0.4% bitrate increase for the 2 neighbor design.

# References

[1] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-2.0

[2] C. Auyeung and W. Liu, “Parallel processing friendly simplified context selection of significance map,” JCT-VC document, JCTVC-D260, March 2011

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, January 2011

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).