| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E365 WG11 Number: m19894 |
| --- | --- |

| Title: | Evaluation of RQT in HM and related TU representation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | 1)Krit Panusopone, Xue Fang, Limin Wang 2)Keiichi Chono 3)Yih Han Tan, Chuohao Yeo 4)Minhua Zhou | Tel: Email: | +1-858-404-3618 krit@motorola.com |
| Source: | Motorola Mobility1), NEC2), Institute for Infocomm Technology3), Texas Instruments4) |  |  |

_____________________________

# Abstract

This contribution reports outcome of an evaluation of RQT in HM and related TU representations. Several different configurations of RQT were tested to evaluate trade-off between coding performance and complexity. For RQT depth equal to 1, an average coding performance loss of 0.7% across all common test conditions is reported. For RQT depth equal to 2, an average coding performance loss of 0.1% across all common test conditions is reported. For RQT depth equal to 3, an average coding performance gain of 0.1% across all common test conditions is reported. For RQT depth equal to 4, an average coding performance gain of 0.1% across all common test conditions is reported. For implicit TU signaling method, an average coding performance loss of 1.6% across all common test conditions is reported.

# Introduction

In Daegu, there were discussions on trade-off between coding performance and complexity of RQT, and the group agreed to have further study of this topic in Coding block structures AhG. The following test plan was suggested as a starting point of this study.

- Relationship between TU and CU

  - TU relates to CU; i.e., RQT

  - Independent tree regardless of CU size

- Tree constraint

  - Flexible; i.e., RQT

  - Flat tree; i.e., all four nodes at the same tree level have the same split decision

- Tree depth

  - 1 level

  - 2 levels

  - 3 levels

  - 4 levels

- Implicit tree signaling; TU size determined based on CU size and PU type

Due to the limitation in the current HM [1], time and resource constraints, only items 3 and 4 evaluation were conducted in this study.

# Experiments

Sub-test 3 – Tree depth. This sub-test was performed by NEC and cross checked by I2R. HM-2.0 software is used in this sub-test. Four sets of experiment were conducted for four different RQT depths. The same RQT depth is used for both intra mode and inter mode. Other configurations follow common test condition [2]. Tables 1-4 show simulation results of this sub-test with common test condition results serve as anchor.

Sub-test 4 – Implicit tree signaling. Encoder does not use RQT to represent TU partition but determines TU size based on CU size and PU partition type. RQT depth and TU split flag are not used in this sub-test. Other configurations follow common test condition. Motorola Mobility provided modified HM-2.0 software that allows implicit TU feature for this test based on [3]. This sub-test was performed by Motorola Mobility and cross checked by TI. Table 5 shows simulation results of this sub-test with common test condition results serve as anchor.

Table 1-1 Simulation results of RQT depth equal to 1 in intra only configurations

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.5 | 0.4 | 0.4 | 0.0 | 0.0 | 0.0 |
| Class C | 0.5 | 0.4 | 0.4 | 0.0 | 0.0 | 0.0 |
| Class D | 0.4 | 0.3 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E | 0.5 | 0.2 | 0.4 | 0.0 | 0.0 | 0.0 |
| All | 0.4 | 0.3 | 0.3 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 92% |  |  | NA |  |  |
| Dec Time[%] | 104% |  |  | NA |  |  |
| Enc Time[%] | 91% |  |  | NA |  |  |
| Dec Time[%] | 100% |  |  | NA |  |  |

Table 1-2 Simulation results of RQT depth equal to 1 in random access configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.1 | 0.5 | -0.1 | 1.2 | 0.7 | 0.7 |
| Class B | 1.5 | 0.6 | 0.4 | 1.5 | -0.7 | -0.9 |
| Class C | 1.8 | 0.4 | 0.4 | 2.0 | -0.7 | -0.8 |
| Class D | 2.7 | 0.6 | 0.7 | 3.2 | -1.1 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | 1.7 | 0.5 | 0.4 | 1.9 | -0.5 | -0.5 |
| Enc Time[%] | 85% |  |  | 98% |  |  |
| Dec Time[%] | 106% |  |  | 103% |  |  |
| Enc Time[%] | 83% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 1-3 Simulation results of RQT depth equal to 1 in low delay configurations

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.8 | 1.4 | 1.2 | 2.3 | -1.5 | -1.4 |
| Class C | 2.6 | 0.9 | 1.0 | 3.3 | -1.0 | -1.6 |
| Class D | 3.7 | 1.7 | 0.9 | 4.9 | -1.7 | -1.7 |
| Class E | 0.8 | -1.1 | 1.3 | 1.4 | -2.3 | -1.6 |
| All | 2.3 | 0.9 | 1.1 | 3.0 | -1.6 | -1.6 |
| Enc Time[%] | 87% |  |  | 99% |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |
| Enc Time[%] | 85% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2-1 Simulation results of RQT depth equal to 2 in intra only configurations

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.1 | 0.1 | 1.0 | 1.0 |
| Class B | 0.2 | 0.2 | 0.2 | -0.2 | 0.7 | 0.8 |
| Class C | 0.1 | 0.1 | 0.1 | -0.2 | 0.3 | 0.4 |
| Class D | 0.1 | 0.1 | 0.1 | -0.1 | 0.3 | 0.2 |
| Class E | 0.2 | 0.1 | 0.1 | -0.2 | 2.0 | 1.7 |
| All | 0.1 | 0.1 | 0.1 | -0.1 | 0.8 | 0.8 |
| Enc Time[%] | 95% |  |  | 108% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
| Enc Time[%] | 95% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

Table 2-2 Simulation results of RQT depth equal to 2 in random access configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.2 | -0.2 | 0.0 | 1.3 | 1.1 |
| Class B | 0.4 | 0.4 | 0.4 | -0.1 | 0.8 | 0.7 |
| Class C | 0.3 | 0.2 | 0.2 | -0.1 | 0.3 | 0.5 |
| Class D | 0.3 | -0.1 | 0.1 | 0.0 | 0.2 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.2 | 0.1 | -0.1 | 0.7 | 0.6 |
| Enc Time[%] | 92% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |
| Enc Time[%] | 92% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2-3 Simulation results of RQT depth equal to 2 in low delay configurations

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 1.0 | 0.9 | -0.1 | 0.4 | 0.7 |
| Class C | 0.3 | 0.8 | 0.7 | -0.1 | 0.3 | 0.0 |
| Class D | 0.3 | 0.9 | 0.3 | 0.0 | 0.0 | 0.5 |
| Class E | 0.1 | -0.9 | 0.9 | -0.1 | 1.6 | 0.7 |
| All | 0.3 | 0.6 | 0.7 | -0.1 | 0.5 | 0.5 |
| Enc Time[%] | 93% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |
| Enc Time[%] | 93% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3-1 Simulation results of RQT depth equal to 3 in intra only configurations

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 1.2 | 1.1 |
| Class B | 0.0 | 0.0 | 0.0 | -0.5 | 0.6 | 0.6 |
| Class C | 0.0 | 0.0 | 0.0 | -0.5 | 0.1 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | -0.3 | 0.1 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | -0.5 | 2.0 | 1.5 |
| All | 0.0 | 0.0 | 0.0 | -0.4 | 0.7 | 0.6 |
| Enc Time[%] | NA |  |  | 115% |  |  |
| Dec Time[%] | NA |  |  | 100% |  |  |
| Enc Time[%] | NA |  |  | 117% |  |  |
| Dec Time[%] | NA |  |  | 102% |  |  |

Table 3-2 Simulation results of RQT depth equal to 3 in random access configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 2.0 | 1.6 |
| Class B | 0.0 | 0.0 | 0.0 | -0.3 | 1.1 | 1.0 |
| Class C | 0.0 | 0.0 | 0.0 | -0.3 | 0.3 | 0.6 |
| Class D | 0.0 | 0.0 | 0.0 | -0.3 | 0.5 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | -0.2 | 1.0 | 0.9 |
| Enc Time[%] | NA |  |  | 105% |  |  |
| Dec Time[%] | NA |  |  | 100% |  |  |
| Enc Time[%] | NA |  |  | 106% |  |  |
| Dec Time[%] | NA |  |  | 100% |  |  |

Table 3-3 Simulation results of RQT depth equal to 3 in low delay configurations

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | -0.2 | 1.0 | 1.1 |
| Class C | 0.0 | 0.0 | 0.0 | -0.4 | 0.4 | 0.3 |
| Class D | 0.0 | 0.0 | 0.0 | -0.3 | 0.0 | 0.6 |
| Class E | 0.0 | 0.0 | 0.0 | 0.3 | 2.3 | 2.5 |
| All | 0.0 | 0.0 | 0.0 | -0.2 | 0.8 | 1.0 |
| Enc Time[%] | NA |  |  | 104% |  |  |
| Dec Time[%] | NA |  |  | 99% |  |  |
| Enc Time[%] | NA |  |  | 106% |  |  |
| Dec Time[%] | NA |  |  | 101% |  |  |

Table 4-1 Simulation results of RQT depth equal to 4 in intra only configurations

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 1.2 | 1.1 |
| Class B | -0.1 | 0.0 | 0.0 | -0.5 | 0.6 | 0.5 |
| Class C | 0.0 | 0.0 | 0.0 | -0.5 | 0.0 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | -0.3 | 0.1 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | -0.4 | 1.9 | 1.4 |
| All | 0.0 | 0.0 | 0.0 | -0.3 | 0.7 | 0.6 |
| Enc Time[%] | 104% |  |  | 122% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Enc Time[%] | 102% |  |  | 122% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 4-2 Simulation results of RQT depth equal to 4 in random access configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 | 0.2 | 2.1 | 1.8 |
| Class B | -0.1 | -0.1 | -0.1 | -0.3 | 1.2 | 1.0 |
| Class C | -0.1 | -0.1 | 0.0 | -0.4 | 0.4 | 0.6 |
| Class D | 0.0 | -0.1 | 0.0 | -0.4 | 0.4 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.0 | 0.0 | -0.2 | 1.0 | 1.0 |
| Enc Time[%] | 107% |  |  | 111% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Enc Time[%] | 106% |  |  | 111% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 4-3 Simulation results of RQT depth equal to 4 in low delay configurations

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.0 | -0.3 | 0.0 | 1.1 | 1.3 |
| Class C | -0.1 | -0.1 | 0.0 | -0.4 | 0.6 | 0.3 |
| Class D | -0.1 | 0.0 | -0.5 | -0.3 | 0.2 | 0.4 |
| Class E | -0.2 | 0.1 | 0.7 | 0.6 | 3.5 | 2.4 |
| All | -0.1 | 0.0 | -0.1 | 0.0 | 1.2 | 1.0 |
| Enc Time[%] | 106% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
| Enc Time[%] | 105% |  |  | 109% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 5-1 Simulation results of implicit TU signaling in intra only configurations

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.5 | 0.4 | 0.4 | 0.0 | 0.0 | 0.0 |
| Class C | 0.5 | 0.4 | 0.4 | 0.0 | 0.0 | 0.0 |
| Class D | 0.4 | 0.3 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E | 0.5 | 0.4 | 0.4 | 0.0 | 0.0 | 0.0 |
| All | 0.4 | 0.3 | 0.4 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 89% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
| Enc Time[%] | 89% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 5-2 Simulation results of implicit TU signaling in random access configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 0.1 | -0.2 | 0.7 | 0.4 | 0.5 |
| Class B | 1.2 | 0.5 | 0.4 | 0.8 | 0.2 | 0.1 |
| Class C | 1.0 | 0.3 | 0.4 | 0.7 | 0.1 | 0.1 |
| Class D | 0.9 | 0.1 | 0.1 | 0.8 | 0.0 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 1.0 | 0.3 | 0.2 | 0.8 | 0.2 | 0.2 |
| Enc Time[%] | 85% |  |  | 96% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
| Enc Time[%] | 84% |  |  | 93% |  |  |
| Dec Time[%] | 104% |  |  | 91% |  |  |

Table 5-2 Simulation results of implicit TU signaling in low delay configurations

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.3 | 1.7 | 1.6 | 1.2 | 0.4 | 0.4 |
| Class C | 1.2 | 1.0 | 0.9 | 1.1 | 0.7 | 0.4 |
| Class D | 1.1 | 0.9 | 0.0 | 1.3 | 0.7 | 0.9 |
| Class E | 0.5 | -0.8 | 0.9 | 0.6 | 0.4 | -0.1 |
| All | 1.1 | 0.9 | 0.9 | 1.1 | 0.5 | 0.5 |
| Enc Time[%] | 88% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Enc Time[%] | 85% |  |  | 94% |  |  |
| Dec Time[%] | 103% |  |  | 94% |  |  |

# Discussions

Simulation results show that RQT provides flexibility with multiple coding performance and complexity trade-offs.  One way to demonstrate impact on coding performance among testing methods is to compute normalized gain achieved by each method over the worst performing approach. Table 6 shows coding performance of all testing methods for luma component and Table 7 shows normalized gain by each method over RQT depth equal to 1.

Table 6 Coding performance of all testing methods for luma component

| Column1 | Implicit | D=1 | D=2 | D=3 | D=4 |
| --- | --- | --- | --- | --- | --- |
| AIHE | 0.4 | 0.4 | 0.1 | 0.0 | 0.0 |
| AILC | 0.0 | 0.0 | -0.1 | -0.4 | 0.3 |
| RAHE | 1.0 | 1.7 | 0.3 | 0.0 | -0.1 |
| RALC | 0.8 | 1.9 | -0.1 | -0.2 | -0.2 |
| LDHE | 1.1 | 2.3 | 0.3 | 0.0 | -0.1 |
| LDLC | 1.1 | 3.0 | -0.1 | -0.2 | 0.0 |
| Average | 0.7 | 1.6 | 0.1 | -0.1 | -0.1 |

Table 7 Normalized gain by each method over RQT depth equal to 1

| Column1 | Implicit | D=2 | D=3 | D=4 |
| --- | --- | --- | --- | --- |
| AIHE | 0% | 61% | 91% | 100% |
| AILC | 0% | 27% | 100% | 98% |
| RAHE | 41% | 80% | 97% | 100% |
| RALC | 52% | 92% | 100% | 99% |
| LDHE | 49% | 83% | 95% | 100% |
| LDLC | 60% | 96% | 100% | 96% |

It should be noted that this study didn’t include impact of several contributions related to RQT were submitted at this meeting [4, 5, 6, 7]. Moreover, implicit TU signaling can be modified to improve further. These proposals may improve trade-off between coding performance and complexity but effects of these contributions on TU representation were not evaluated in this study due to resources constraints and HM limitation.

# References

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, Oct. 2010

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, Jan. 2011

- K. Panusopone, X. Fang, L. Wang, “Efficient Transform Unit Representation”, JCTVC-D250, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, Jan. 2011

- S. Liu, Z. Zhou, S. Lei, “Evaluations and Suggestions about TU Representation”,JCTVC-E104, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Mar. 2011

- Y. H. Tan, C. Yeo, H. L. Tan, Z. Li, “RQT depth selection”, JCTVC-E104, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Mar. 2011

- K. Panusopone, X. Fang, L. Wang, “Proposal on RQT root location”, JCTVC-E364, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Mar. 2011

- L. Guo, X. Wang, P. Chen, M. Karczewicz, “Limiting Chroma Transform Depth in Residual Quad Tree (RQT)”, JCTVC-E377, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Mar. 2011

# Patent rights declaration(s)

Motorola Mobility, NEC, Institute for Infocomm Technology, Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).