| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E366-r1 |
| --- | --- |

| Title: | CE10: Crosscheck of Cisco/TI's contribution (JCTVC-E243) on core transforms - normal QP range |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | R. Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-658-4511 Email: rajanj@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Cisco/TI’s proposal on core transforms (JCTVC-E243). The results of our crosscheck matched those reported by Cisco/TI for normal QP range. The encoding and decoding times also matched closely.

# Introduction

Cisco/TI’s contribution proposes the use of matrix multiplication to implement 4, 8, 16 and 32-point transforms. Right-shifting at the input of inverse transform stage and after the 1st transform stage for forward as well as inverse transform results in the 16-bit precision at the input to transform stages as well as the transpose buffer.  The transform coefficient matrices have 8-bit precision. In this crosscheck, we confirmed the results reported by Cisco/TI for the normal QP range.

- Results

Results for all 6 configurations described in the common test conditions were crosschecked. Table 1 shows a summary of the results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding was done on a LINUX cluster and decoding was done on a PC using Microsoft Visual C++ (32-bit).

Table 1: BD-rate results for Cisco/TI’s proposal for normal QP range

# Conclusions

We have verified results reported in Cisco/TI’s proposal on core transforms for normal QP range. The encoding and decoding times also matched closely.